3GPP TSG SA WG 1 Meeting #109 	S1-250560

Athens, Greece, 17-21 February 2025	(revision of S1-250222 was244752)

Source:		OPPO, Futurewei

Title:	Network operation: Network managed Extended PIN for local traffic transmission among 3GPP UEs

Agenda item:	8.1.1

Document for:	agreement

Contact:	Yang Xu (xuyang@oppo.com)

Abstract: This contribution provides a use case in the area of network operation. The Macro-network managed Extended PIN for local traffic transmission among 3GPP UEs.

********** Start of Change*********

5.x	Network managed extended 6G PIN for local traffic transmission among 3GPP UEs

5.x.1	Description

PIN feature defined in the 5G can be further extended to become extened 6G PIN which can support new capabilities in 6G. One of new capability of Extended 6G PIN is to establish one or more local PIN(s) which are managed by one Network operator (can be PLMN or NPN), while the 3GPP UEs as PIN element in the same Extended PIN can communicate with each other via a 6G access network, with authentication and authorization by core network.

Support PIN element communication using 6G radio interface helps offloading Local services to PIN to reduce load for centrally located CN nodes (Higher Experienced Rate and Connection Density), and local traffic management provides better performance of latency and mobility (Stricter latency and mobility requirement).

Comparison to the existing technology is shown in table 5.x.1-1 below.

The Extended PIN may have a wider coverage which can be used for communication of 3GPP devices belonging to a user, a family, or used for a Customer Premises Network (CPN) for a residence, office or shop).

5.x.2	Pre-conditions

Not applicable

5.x.3	Service Flows

Not applicable

5.x.4	Post-conditions

Not applicable

5.x.5	Existing features partly or fully covering the use case functionality

In 3GPP TS 22.261 clause 3.1 and 6.38, it already has the service requirement of PIN. But 5G PIN is a much smaller scope and it’s tighted to individual cusomters and exclusive for PLMN, such as “CPNs and PINs have in common that in general they are owned, installed and/or (at least partially) configured by a customer of a public network operator”. PIN in 6G can be extended to bigger scale as indicated in the use case.

The Extended PIN assumes the two 3GPP UEs communicate to each other via Uu supported by 6G network

5G PIN support local inter-PIN element communication using direct network connection only if the PIN elements are in the range to use PIN direct connection, as indicated in TS22.261. clause 6.38.1 “Via a PIN Element with Gateway Capability, PIN Elements have access to the 5G network services and can communicate with PIN Elements that are not within range to use PIN Direct Connection.”

5.x.6	Potential New Requirements needed to support the use case

Functionality Requirements:

[P.R.5.x.6-1] Subject to operator policy and user consent, the 6G system shall support a user data path trasversing via 6G network for intra-PIN communications no regard if PIN elements are within the range to use PIN direct connection or not.

[P.R. 5.x.6-2] Subject to operator policy and user consent, the 3GPP system shall be able to identify the PIN elements in the Extended 6G PIN and support the association between the PIN elements and a User ID.

NOTE: the User ID can be an identification of a human user, a famility or an enterprise.

[P.R. 5.x.6-3] Subject to operator policy and user consent, the 3GPP system shall be able to support charging for the communication service of an Extended PIN.

Editor’s Note: it is FFS for KPI supported by the extended PIN.

*******************End of Change*****************