3GPP TSG-SA WG1 Meeting #109 	S1-250704

Athens, Greece, 17-21 February 2025	(revision of S1-250045)

Title:	Use case on AI agent for network performance assurance

Agenda Item:	8.1.2

Source:	ZTE, China Telecom

Contact:	Lin Chen (chen.lin23@zte.com.cn)

Xu Xia (xiaxu@chinatelecom.cn)

Menghan Yu (yumh1@chinatelecom.cn)

Abstract: <provide a short description of the content>

This contribution proposes a new use case about AI agent for network performance assurance, which uses AI agents in 6G network to integrate the communication knowledge, structured data and network atomic capabilities to achieve the desired network performance for big events.

1. Introduction

This contribution proposes a use case about AI agent for network performance assurance, which uses AI agents in 6G network to integrate the communication knowledge, structured data and network atomic capabilities to achieve the desired network performance for big events.

- Reason for Change

Based on the comments received online, the following changes are made for S1-250704:

-	Change the title to “AI agent for network performance assurance”

-	Remove the second change for “3.1 Definitions”

-	Change the PR-1 to PR-4 to address the online comments

3. Proposal

It is proposed to agree the following new use cases to 3GPP TR 22.870 v0.1.1.

************************ First change ***********************************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[5.1X]	QIAN LI et all, “6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies” in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[6.1x]	Russell, S.J. Norvig, P., Artificial Intelligence: A Modern Approach, Pearson Education, Inc., 2010.

[6.1y]	Zhiheng Xi, Wenxiang C., etc., The rise and potential of Large Language Model based agents: a survey, 2023.

************************ Second change ***********************************

6.x	Use case on AI agent for network performance assurance

6.x.1	 Description

According to [6.1x], an agent in AI refers to an artificial entity capable of perceiving its environment using sensors, making decisions, and then taking actions in response using actuators. The evolution of AI agents has undergone several stages, ranging from symbolic agents, reactive agents, reinforcement learning-based agents, transfer learning and meta learning based agents, to large language model(LLM) based agents [6.1y].

Due to the LLM's powerful capability in knowledge acquisition, multi-modal comprehension and generation, planning and reasoning, more and more research focuses on the usage of LLM to build AI agent. Generally speaking, an LLM based AI agent may be capable of performing certain tasks via perceiving environment, understanding intents, reasoning, planning, making decisions, and taking actions with tools. AI agent may decompose the complex task into smaller, manageable sub-tasks that can be executed either sequentially or in parallel by one or more AI agents.  In other word, a sub-task is a smaller, self-contained, and goal-oriented unit of work derived from a larger, complex task. By defining sub-tasks with clear objectives, multiple AI agents can collaborate to tackle more complex real-world problems. For example, each AI agent can be equipped with specialized skills and domain knowledge and engage in specific tasks. Through the collaboration of multiple AI agents, the task efficiency and decision accuracy can be improved.

LLM based AI agent can be used in 6G network to solve complex tasks such as big event assurance. As we know, the big events, such as Olympic games, concerts, marathon, etc., usually pose stringent requirements for network performance for a given area. Large number of concurrent users, big volume of data transmission and frequent real-time operations are assumed. With Asian games in 2022 as an example, there are 48 matches and 1.5 million users over 16 days. Moreover, the big event such as technology expo, trade fair, or corporate gathering, may be held within an industrial park. In these cases, there are a lot of participants of the event, including exhibitors, visitors, and staffs. The network performance requirement is higher than usual. During these events, full network coverage, good user experience, and zero complaints are expected.

In order to provide the desired network performance, AI agent can be used to integrate the communication knowledge, structured data and network atomic capabilities to achieve the big event assurances. For example, the third party may raise the network performance assurance task and requirements, for example, the service types, bandwidth requirement, the number of users that needs to be supported, etc. Through the multi-modal human-computer interaction, user agent can understand the intent, decompose the intent into several sub-tasks and calling service agents. The service agent makes action plan for efficient network configuration and resource allocation, network coverage, communication service performance monitoring and quality assurance in runtime, risk prediction and avoidance, alarm and root cause analysis, etc. Finally, action agents may execute the specific action by using various tools, including built-in tools, API interfaces, models, etc., to complete each sub-tasks step by step.  Here the  user agent may be LLM based while the service agent and action agent may use light-weight AI models.

Based on the assistance of multi-agent collaboration in 6G network, it is expected that the network performance requirement can be fulfilled and the labor workload can be reduced.

6.x.2	 Pre-conditions

National game will be held with millions of athletes and visitors. Operator A is responsible for the communication service provision with 6G network around the national game area. Operator A supports the AI agent for the efficient 6G network performance assurance. Various AI agents are created when the 6G network is deployed.

6.x.3	 Service Flows

- Before the national game, the operation team inputs the requirements that national game needs to be prepared, such as the service types, bandwidth requirement, and the number of users, etc.

- Upon receiving such input, the user intent is analyzed by AI agent and divided into several sub-tasks. The sub-tasks may cover efficient network configuration, personalized guarantee of user experience, real-time performance KPI monitoring, risk prediction and avoidance, etc.

- The AI agents for sub-tasks will make detailed action plan for the sub-tasks. The action plan will be evaluated and tested until it works smoothly before the national game.

- During the game, AI agents involved in the big event assurance take actions to provide the desired network performance autonomously. Moreover, the monitoring dashboard is created so that the status of user experience and network performance can be continuously monitored.

- If pre-warning is received during the game, the corresponding AI agent can make optional solution based on pre-set intents. If this solution is evaluated as feasible, the network re-configuration will be carried out by AI agents automatically.

6.x.4	 Post-conditions

With the intelligent AI agents collaborating together, the network performance can be assured and workload on complex routine network configuration and risk prediction/avoidance can be reduced.

6.x.5	 Existing features partly or fully covering the use case functionality

In TS 22.261, clause 6.51.2 on monitoring of network elements interactions in 5G include the following requirements:

NOTE 1: The monitoring system is outside of the 5G network. Both the monitoring system and the monitored network elements in the requirements below are fully under the control of the MNO.

The monitored network elements in the 5G network shall support the transmission of a secured copy of the outgoing and incoming signalling traffic to a monitoring system.

The 5G network shall enable the MNO to configure network monitoring, e.g., switching on/off per network element, selecting what type of elements and what type of signalling from these elements is the target for monitoring.

The 5G network shall allow the monitoring (i.e., transmit secured copies of outgoing and incoming signalling traffic) of a transmitting network element and, separately, the monitoring of the receiving network element while facilitating correlation of the information received from both network elements by the external system.

NOTE 2: These requirements do not imply/assume any design of the network elements. How the copies are created within the element, e.g., physical, virtual or container based, is expected to be implementation specific.

The signalling traffic shall be securely transmitted from the monitored network elements of the 5G network to the monitoring system while minimizing the degradation of network performance.

NOTE 3: The monitoring system is not integrated with the key management scheme of the 5G core.

The transmission of signalling traffic from the monitored network elements of the 5G network to the monitoring system shall be compliant with privacy legislation, data protection regulations and protection of confidential system internal data.

The transmission of signalling traffic from the monitored network elements of the 5G network to the monitoring system shall be limited regarding the number of file formats (e.g., JSON, PCAP, etc.) to assist with the ingestion of traffic feeds.

6.x.6  Potential New Requirements needed to support the use case

[PR.6.x.6-1]	Subject to operator’s policy, the 6G network shall be able to support the use of AI agents to provide network performance assurance(e.g. capacity, user density, latency, etc.) for a given area.

[PR.6.x.6-2]	Subject to operator’s policy, the 6G network shall be able to provide means to receive the request for network performance assurance via intent from third party.

[PR.6.x.6-3]	Subject to operator’s policy, the 6G network shall be able to support the collaboration of multiple AI agents to perform the tasks (e.g. network configuration, resource allocation, risk prediction and avoidance, etc.) automatically.

[PR.6.x.6-4]	Subject to operator’s policy, the 6G network shall be able to support the AI agent to monitor the network performance and assure the QoS in runtime.