3GPP TSG SA WG 1 Meeting #109 	S1-250851

Athens, Greece, 17-21 February 2025	(revision of S1-250017)

Source:	vivo

pCR Title:	Considerations for 6G LPWA and eMBB

Draft Spec:	3GPP TR 22.870v0.1.1

Agenda item:	8.1.1

Document for:	Approval

Contact:	Yanchao Kang, kangyanchao@vivo.com

Abstract: This contribution analyses the common aspects for eMBB and LPWA, and propose potential requirements for 6G system to support eMBB service and LPWA service.

* * * First Change * * * *

5.X	Use case on considerations for 6G LPWA and eMBB

5.X.1	Description

In the era of 6G, the mobile system will provide its services to a myriad of devices, encompassing not only eMBB-type devices such as smartphones but also variety of IoT devices including LPWA terminals. Despite their diverse form factor and distinct service scenarios, these terminals share certain commonalities in accessing mobile communication services.

Low Power Wide Area (LPWA) plays a crucial role in Internet of Things (IoT) applications, especially for scenarios that require broad geographic coverage but have low data transmission needs. Here are some typical application scenarios LPWA：

- Smart Cities: This includes smart street lighting, parking management, waste management, etc. These applications require devices to operate in a low-power mode for long periods while covering entire cities.

- Agricultural IoT: Used for soil monitoring, weather stations, water resource management, etc. These scenarios require devices to operate in remote areas with deep coverage to ensure reliable data transmission.

- Asset Tracking: Such as cargo tracking systems in the logistics industry, these devices require long battery life and extensive coverage to meet the demands of long-distance transport.

- Smart Metering: Remote reading of water, electricity, and gas meters; these devices are typically installed in homes or enterprises, requiring deep coverage and low power consumption.

All the aforementioned scenarios collectively embody a set of service attributes:

- Low Power Consumption: IoT devices need to have very low power consumption to extend battery life, often requiring devices to operate for several years without a battery change.

- Deep Coverage: Reliable data transmission is needed even in basements or remote areas. Support deep coverage is also beneficial to reduce the operator’s cost to deploy and maintain the radio access network.

- Low Mobility: Many LPWA scenarios involve fixed or low-speed moving terminal devices, therefore it would be possible and beneficial to reduce the complexity and power consumption of handling high mobility.

- Small Data Volumes: LPWA devices typically transmit or receive small data packets, allowing for simplified network protocol designs to ensure low power consumption and efficient transmission.

- Low Cost: Both device and network deployment costs need to be low to support large-scale deployment, ensuring cost-effectiveness in the manufacture and maintenance of terminal nodes.

Although smartphones are primarily engineered to deliver lightning-fast data speeds and immersive multimedia experiences, power consumption/battery life and deep coverage are also very important for smartphones users, especially when smartphones are in reduced data usage and/or low mobility scenarios:

- Users consistently prioritize extended standby durations for their smartphones, and energy efficiency remains a critical requirement for eMBB services to ensure optimal user experience.

- In challenging coverage environments such as subways, elevators, or remote areas, smartphones significantly benefit from improved network coverage to ensure a stable and reliable connection.

NOTE 1：the operator’s deployment and operating cost can also be reduced if deep coverage is available for smartphones.

The following table 5.x.1-1 summarizes the commonalities between IoT devices and eMBB devices in terms of accessing mobile communication services. It should be noted that these characteristics are consistent requirements for IoT devices and eMBB devices with some specific requirements in special scenarios, as shown in figure 5.x.1-1.

Figure 5.x.1-1: Illustration of eMBB features and LPWA features

NOTE 2：Low operating BW is different from low BW capability. Both LPWA devices and eMBB devices can operate in a low bandwidth, but the maximum BW supported by LPWA devices is typically much smaller than eMBB devices.

Table 5.x.1-1: Commonalities between requirements for LPWA devices and eMBB devices

|  | LPWA devices | eMBB devices |
| --- | --- | --- |
| Low UE power consumption | Very low power consumption when there is DL or UL traffic; Very low power consumption during device standby; Comparable battery life as NB-IoT/MTC, e.g., 10~ 15 years | Very low power consumption during usage of small data volume Very low power consumption during device standby |
| Deep coverage | Comparable coverage as practical use case for NB-IoT/MTC, e.g., deep coverage with 5 ~ [15] dB improvement than normal case. | Continuous service even in extreme coverage, e.g., in subway, elevator or remote coverage scenarios, requiring coverage native design for both initial access and RRC connected mode. |
| Device operating bandwidth | Small bandwidth to reduce device cost & complexity, e.g., at least 3-5 MHz for FDD bands, and 10~20MHz for TDD bands in FR1 | Device operating in larger BW during high data rate scenarios. Device operating in a smaller BW for power saving during low data volume activity, e.g., heart beat or instant message applications. |
| Data rate | Higher data rate than NB-IoT/ LTE Cat-M1 to serve practical NB-IoT/ LTE Cat-M1 use case as well as new use case for massive IoT, e.g., 1Mbps ~ 10Mbps | Various use cases with Mbps to Gbps data rate. |

Energy efficiency and coverage are critical factors for operators when deploying networks. Single network deployment supporting both eMBB and LPWA devices can substantially decrease deployment and operational cost while improving network energy efficiency. This network deployment strategy can address the requirements for low power consumption, extensive coverage, and high-speed, high-capacity data transmission.

- Deployment scenarios: as shown in left half of figure 5.x.1-2, a network deployment includes coverage layer and capacity layer. Where the coverage layer includes a sparser BS deployment providing large coverage (with coverage extension features enabled), however, the data rate and capacity provided by coverage layer could be limited due to sparse deployment. The capacity layer includes denser BS deployment providing higher data rate and capacity. It is noted that BS for coverage layer may use lower or same frequency carrier than BS for capacity layer. In such deployment scenario, it is up to operator’s choice to

  - Serve LPWA devices on coverage layer only, and eMBB devices on capacity layer only

  - Serve LPWA and eMBB devices with resource sharing: shared control channel resource (e.g. cell common control channels) for both eMBB and LPWA, while dedicated resource for data transmission of for eMBB devices with high data volume.

- Network energy saving: in the above deployment scenario, network adaptation can be used to achieve energy saving, e.g.  during night time when eMBB data volume is low, operator can turn off the capacity layer BS and keep the coverage layer BS active providing basic services for both LPWA and eMBB devices, as shown in right half of figure 5.x.1-2. Furthermore, operator can reduce the operating BW of coverage layer BS (e.g. to 10MHz) for further energy saving.

Figure 5.x.1-2: network deployment including coverage layer and capacity layer

5.X.2	Potential New Requirements needed to support the use case

[PR-5.X.2-1]	 The 6G system shall allow different device types, from low-end(e.g. massive IoT devices) to high-end(e.g. smartphones), to be served by the same radio access network.

[PR-5.X.2-2]	 The 6G system shall provide mechanisms to support common features applicable for different device types in specific service area.

[PR-5.X.2-3]	 The 6G system shall provide mechanisms to resource sharing for different device types.