3GPP TSG SA WG 1 Meeting #110 	S1-252046

19-23 May 2025, Fukuoka, Japan	(revision of S1-25xxxx)

Source:	China Mobile

pCR Title:	Pseudo-CR on adding KPI table to 7.4

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	Xiaonan Shi, shixiaonan@chinamobile.com

Abstract: Propose to add KPI table to 7.4.

1. Introduction

While introducing this use case to the TR, a new paremeter sensing capacity was introduced while the KPI table has not been introduced yet.

2. Reason for Change

Sensing capacity can be estimated using the number of sensing targets (N) per cell fulfilling QoS divided by the cell area (A). Assuming sensing capacity of UAV intrusion detection is larger than sensing capacity of UAV trajectory tracking, as tracking is more difficult than detection.

If the inter-site distance (ISD) is 200m, then A is 11547 m2. Assuming N = 2 or 4 target/cell for UAV intrusion detection, then sensing capacity is 173 or 346 per km2, we propose 200 per km2. Assuming N = 1 or 2 target/cell for UAV trajectory tracking, then sensing capacity is 87 or 173 per km2, we propose 100 per km2.

If the inter-site distance (ISD) is 500m, then A is 72169 m2. Assuming N = 10 or 20 target/cell for UAV intrusion detection, then sensing capacity is 139 or 277 per km2, we propose 200 per km2. Assuming N = 5 or 10 target/cell for UAV trajectory tracking, then sensing capacity is 69 or 139 per km2, we propose 100 per km2.

# Table for better illustration

| Case | ISD | cell area A = ISD2 * sqrt(3)/6 | N = number of sensing targets per cell fulfilling QoS | Sensing capacity calculation: = N / A | Proposed KPI |
| --- | --- | --- | --- | --- | --- |
| UAV intrusion detection | 200m | 11547 m2 | 2 | 173 per km2 | 200 per km2 |
|  |  |  | 4 | 346 per km2 |  |
|  | 500m | 72169 m2 | 10 | 139 per km2 |  |
|  |  |  | 20 | 277 per km2 |  |
| UAV trajectory tracking | 200m | 11547 m2 | 1 | 87 per km2 | 100 per km2 |
|  |  |  | 2 | 173 per km2 |  |
|  | 500m | 72169 m2 | 5 | 69 per km2 |  |
|  |  |  | 10 | 139 per km2 |  |

3. Conclusions

Sensing capacity is a meaningful service level KPI and needs to be introduced to KPI table.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

## 7.4	Use case on low-altitude UAV supervision

### 7.4.1	Description

With the development of Uncrewed Aerial Vehicles (UAV) technologies, light and small civilian UAVs have played a great role in aerial photography, agriculture, mapping and other fields. Various commercial UAV applications are now becoming a reality. These UAVs typically operate at low altitudes and may produce a series of safety control problems, e.g. UAV illegal intrusion and UAV collision. Thus, how to realize the low-altitude UAV supervision is important and challenging in 6G.

In some scenarios, these UAVs may follow carefully planned routes that ensure efficient, regulated, and safe operation in designated airspace. To perform tasks like package delivery, surveillance, or environmental monitoring, commercial UAVs operate based on pre-determined flight paths that dictate their altitude, speed, and direction. For instance, a UAV delivering goods will follow a direct route from the dispatch location to the recipient, while a UAV assigned to environmental monitoring will travel from its station to a specific target area for data collection. Route design and optimization are crucial for safe and efficient UAV operations. These flight routes which are approved by UAV operators, prioritize the shortest flight path, avoid restricted airspace, and ensure safe distances from obstacles such as buildings, trees, or other UAVs. Following a strict route minimizes the risk of accidents and enhances the reliability of UAV services.

Although commercial UAVs are equipped with sensors to assist with real-time navigation, these sensors can be affected by environmental conditions like lighting, weather, or geographical obstructions. Such limitations can impair a UAV's ability to accurately determine its position, altitude, or velocity, which may lead to deviations from the approved flight path. Furthermore, for services such as good delivery, where a number of UAVs are involved in a given area, UAV collision might happen due to sensor limitations and lead to safety issues.

The existing UAV tracking technologies, such as ground-based radar systems and dedicated surveillance equipment, provide route monitoring. However, the widespread deployment of these systems faces challenges due to high installation and maintenance costs and limited availability of suitable installation sites.

As illustrated in Figure 7.4.1-1 [9], UEs connected to 6G Radio Access Network (RAN) entities can be configured to support sensing operations. This configuration enhances sensing coverage, provides additional positioning reference points for sensing measurements, and improves the accuracy and reliability of sensing results. These improvements are due to the higher density of UEs compared to base stations, which increases the likelihood that some UEs are positioned closer to the UAV than the 6G RAN entities (for example, with a UAV located between two 6G RAN entities and a UE located directly beneath the UAV). Additionally, certain UEs may be placed in reflection directions that provide a larger radar cross section (RCS) for the UAV, taking into account the UAV's RCS variations in different incident/reflection angles.

NOTE:	The term “6G Radio Access Network” does not imply any architectural assumption, e.g. whether 6G core network is a new or evolved core network (compared to 5G).

The 6G sensing processing unit can gather sensing data from one or multiple network infrastructures. Upon request, the 6G network operator can provide UAV flight trajectory tracking services to trusted third-party applications, such as UAV service operators, regulatory agencies, Uncrewed Aerial System Traffic Management (UTM) systems, UAV itself, etc.

Figure 7.4.1-1: Low-altitude UAV trajectory tracing by 6G system

Thus, how to realize the low-altitude UAV supervision (e.g. UAV intrusion detection, UAV trajectory tracking) is important and challenging in 6G.

Sensing is an efficient technology for object detection by means of 6G radio signals, e.g. monitoring UAV illegal flying in a specific area. 6G network could provide sensing service by collecting sensing data, transmitting sensing data, processing sensing data, storing sensing data and support sensing result exposure to the third application platform. These sensing data has the following characteristics:

-	These data may not necessarily belong to a specific UE while these data may be produced by the relationship between the network and the physical environment.

-	These data may be produced by a UE or a base station in order to complete a specific task which needs multi-dimensional cooperation.

-	These data may have a relationship with time and space which needs efficient on-demand transmission, storage and collection.

-	These data may be collected from non-3GPP sensing sources which needs unified management in the 6G network.

To address these new data characteristics, the 6G network capability of data processing needs to be extended based on the 5G network including multi-source heterogeneous data collection, efficient and guaranteed large-scale data transmission, efficient data processing within the network, and unified data storage to support multi-node cooperative sensing and multi-node information convergence.

In the low-altitude UAV supervision scenario, the 6G network could be used for sensing the UAV intrusion such as a UAV illegally flying in a restricted area including government and company regions. In this scenario, the network security and data security need to be guaranteed due to the privacy issue. Furthermore, the historical data may be used to identify an illegal UAV. In addition to the communication property of the detected UAV itself, the data management in this low-altitude sensing scenario must be operated within the network.

In a word, the 6G network will break through the "pipeline" capability and go towards to a new type of information service network by realizing diversified data collection, transmission, processing, storage and exposure within the core network.

### 7.4.2	Pre-conditions

In this use case, a UAV Operator/UTM provides package delivery services within an area covered by a 6G network.

Network Operator NN provides 6G sensing service for UAV flight assistance service, including illegal UAV intrusion detection, UAV flight trajectory tracing, UAV collision prediction and etc. NN can make use of wireless base station to sense the airspace within their coverage area and report the sensing information (including tracked UAV and the environment around the UAV) to the USS (Uncrewed Aerial System Service Supplier)/UTM.

The Company MM uses the USS/UMT to supervise the low-altitude UAVs and manage potential illegal intrusion into the restricted areas. MM has proved its restricted area information to the USS/UMT.

The USS/UMT uses 6G sensing service provided by the 6G network operator NN to detect potential UAV illegal intrusion and UAV collision prediction.

The UAV Operator/UTM provides specific details to the 6G network operator NN, including the characteristics of the UAV that will be tracked, along with details about the time and location for flight tracing. This information includes regulated flight paths as well as potential areas where the UAV might temporarily deviate from its route.

The 6G Network Operator NN can realize 6G data collection, 6G data transmission, 6G data processing, 6G data storage within the network, and provide sensing results to the USS/UMT/UAV.

### 7.4.3	Service Flows

For illegal UAV intrusion:

1.	The Company MM requests 6G sensing service for illegal UAV intrusion detection in the restricted area from the USS/UMT.

2.	The USS/UMT transmits the request to the 6G Network operator NN.

3.	The 6G Network Operator NN selects the base stations located in the restricted area to collect and process initial sensing data by collaborative sensing. The selected 6G base station constantly collect sensing data of the location of UAVs near the restricted area and sends the sensing data to the 6G core network with a defined frequency to obtain the sensing result (i.e., the distance between the UAV and the border or motion trail).

NOTE:	The term “6G core network” does not imply any architectural assumption, e.g. whether 6G core network is a new or evolved core network (compared to 5G).

4.	The 6G Network aggregates, forwards and transmits the data generated by the base stations, and processes the data to obtain the sensing results.

5.	The 6G Network exposes the sensing result to the USS/UMIT. The USS/UMIT could trigger to send warning messages to the UAV or intercept the illegal UAV directly based on the sensing results.

For UAV flight trajectory tracing:

1.	When the scheduled time for tracking begins, the 6G network operator activates the UAV trajectory tracing service within the designated area until the tracking session ends. The UAV operator then launches UAV#1, which takes off from the delivery source and heads toward the destination, following a pre-set flight path.

2.	Using radio sensing, a network of 6G base stations and connected devices (UEs) detect UAV#1 and continuously gather data on its position and movement, such as distance, velocity and angle. These metrics, also known as 3GPP sensing data, are sent to a 6G processing unit for real-time analysis.

3.	During the flight, if UAV#1 leaves the coverage range of one base station and enters a new coverage zone, the 6G system could let the old base station stop radio sensing, and switch to a new base station for sensing UAV#1 until it is out of coverage. This transition is based on the UAV's estimated position and velocity, which the 6G processing unit calculates. The network can automatically adjust the sensing operations at base stations depending on this data or based on a pre-defined time frame. In certain cases, sensing handover may be triggered to maintain continuous coverage. For instance, if the current base station's connection weakens or if another nearby base station can offer better coverage for UAV#1, the system proactively shifts the sensing function to this new station to ensure uninterrupted tracking.

4.	The 6G processing unit can aggregate sensing data from multiple sources, including RANs and UEs, to estimate UAV#1's location and velocity. Similar approach can also be applied to UAV#2 - UAV#N, in the case there are multiple UAVs providing service in the area. This real-time information is then transmitted to the UAV operator and/or UTM, who monitors the UAV's trajectory.

5.	If UAV#1 - UAV#N deviate from prescribed routes, the UAV operator and/or UTM receives alerts, allowing them to take corrective action and redirect the UAVs as necessary.

### 7.4.4	Post-conditions

For illegal UAV intrusion: The illegal UAVs are moved away from the restricted area. Potential privacy risks are avoided. Thanks to the wide-area and constant sensing capability of the 6G base stations, and the efficient data transmission, processing and storage by the 6G core network, the safety supervision of the low-altitude space of Company 'MM' is improved.

NOTE:	The term “6G core network” does not imply any architectural assumption, e.g. whether 6G core network is a new or evolved core network (compared to 5G).

For UAV flight trajectory tracing: UAV#1 follows the tracked flight route to deliver the package to its destination; any off-route movements are detected.

For UAV collision prediction: UAV#1 flies efficiently to its destination.

### 7.4.5	Existing features partly or fully covering the use case functionality

In TS 22.137 [6], there are requirements specified for 5G system on integration of sensing and communication.

### 7.4.6	Potential New Requirements needed to support the use case

[PR 7.4.6-1] Subject to operator policy, the 6G network shall enable the base station to send sensing measurement data to the core network, and enable the core network to aggregate, collect, process, and store sensing measurement data from base stations.

[PR 7.4.6-2] The 6G system should support energy-efficient sensing operations.

[PR 7.4.6-3] The 6G system should provide mechanisms to ensure sensing service is able to be provided with a given sensing system capacity.

NOTE:	The term 'sensing system capacity' is the maximum number of targets that can be detected per unit area given sensing QoS requirements per target, which include localization accuracy and sensing service latency [11].

[PR 7.4.6-4] The 6G system shall be able to provide sensing with following KPIs [9][11]:

Table 7.4.6-1	Performance requirements of sensing results for low-altitude UAV supervision

| Scenario |  | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Sensing capacity (per km2) NOTE 1 | Max sensing service latency [ms] | Refreshing rate (Hz) | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s] |  |  |  |  |  |
| Low-altitude UAV supervision | UAV intrusion detection | Outdoor | ≥95 | ≤10 | ≤10 | N/A | N/A | [≥10] | [≥5] | 200 | [≤1000] | [≤1] | [≤5] | [≤5] |
|  | UAV trajectory tracking | Outdoor | ≥90 | 1-2 | 1-2 | 3-5 | 3-5 | N/A | N/A | 100 | 100~1000 | ≤1 | ≤5 | ≤5 |
| NOTE 1：Sensing capacity can be estimated using the number of sensing targets one cell can sense while fulfilling required sensing QoS and the area one cell can cover. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |