3GPP TSG SA WG 1 Meeting #110 	S1-252073

19-23 May 2025, Fukuoka, Japan	(revision of S1-25xxxx)

Source:	Deutsche Telekom

pCR Title:	Pseudo-CR on alignment of 6G terms

Draft Spec:	3GPP TR 22.870 V0.2.1

Agenda item:	8.1.1

Document for:	Approval

Contact:	kurt.bischinger@magenta.at

Abstract:

1. Introduction

This contributions proposes to align 2 notes to the text they support.

2. Reason for Change

At SA1 #109 it was agreed to add a note to every mentioning of 6G core network or 6G radio access network. There are two notes (in clauses 3.1 and 7.4.1) where the note is not fully aligned with the text, one talking of CN/RAN whereby the text is only about CN, the other talking of CN while the text is about the RAN.

3. Conclusions

-

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.2.1.

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Al Agent: an automated intelligent entity capable of e.g interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, executing tasks (autonomously or in collaboration with other Al Agents) to achieve a specific goal.

Editor’s Note: This definition could be updated as the study goes on to align with the potential new understand of AI agent.

AI service: an 6G service (e.g., AI model inference) to support AI-related activities, which is provided by 6G core network to the 6G user, considering the required quality of the service (e.g., inference accuracy, latency).

NOTE 1:	The above term does not imply any architectural assumption, e.g. whether 6G CN is a new or evolved CN/ (compared to 5G).

Editor’s Note:	The definition of AI service is FFS.

Digital Twin: A real-time representation of physical assets in a digital world.

NOTE 2: This definition was taken from ITU-T Recommendation Y.3090 [113].

Intelligent Communication Assistant: The virtual intelligent communication assistant locates in operator network and interacts with the users through voice, video, text, gestures or other modalities. The assistant can be customized for each particular user by accessing user data stored in the network, with user’s consent. It can provide various communication services and support individual users based on user’s intention and requirement. One subscriber can have one or more Intelligent Communication Assistants.

Network Digital Twin: virtual replica of (part of) a mobile network to emulate (or simulate) the behaviour of the actual network.

Editor’s Note: it is FFS to update this definition.

Personal Data: any information relating to a user or subscriber that can be used to, either directly or indirectly, identify that user or subscriber, or to distinguish that user or subscriber from others.

Sensing target density: total number of objects to be sensed per geographic area. It is a measure of how many objects the 3GPP system can detect, identify and/or track within a target sensing area.

* * * Next Change * * * *

### 7.4.1	Description

With the development of Uncrewed Aerial Vehicles (UAV) technologies, light and small civilian UAVs have played a great role in aerial photography, agriculture, mapping and other fields. Various commercial UAV applications are now becoming a reality. These UAVs typically operate at low altitudes and may produce a series of safety control problems, e.g. UAV illegal intrusion and UAV collision. Thus, how to realize the low-altitude UAV supervision is important and challenging in 6G.

In some scenarios, these UAVs may follow carefully planned routes that ensure efficient, regulated, and safe operation in designated airspace. To perform tasks like package delivery, surveillance, or environmental monitoring, commercial UAVs operate based on pre-determined flight paths that dictate their altitude, speed, and direction. For instance, a UAV delivering goods will follow a direct route from the dispatch location to the recipient, while a UAV assigned to environmental monitoring will travel from its station to a specific target area for data collection. Route design and optimization are crucial for safe and efficient UAV operations. These flight routes which are approved by UAV operators, prioritize the shortest flight path, avoid restricted airspace, and ensure safe distances from obstacles such as buildings, trees, or other UAVs. Following a strict route minimizes the risk of accidents and enhances the reliability of UAV services.

Although commercial UAVs are equipped with sensors to assist with real-time navigation, these sensors can be affected by environmental conditions like lighting, weather, or geographical obstructions. Such limitations can impair a UAV's ability to accurately determine its position, altitude, or velocity, which may lead to deviations from the approved flight path. Furthermore, for services such as good delivery, where a number of UAVs are involved in a given area, UAV collision might happen due to sensor limitations and lead to safety issues.

The existing UAV tracking technologies, such as ground-based radar systems and dedicated surveillance equipment, provide route monitoring. However, the widespread deployment of these systems faces challenges due to high installation and maintenance costs and limited availability of suitable installation sites.

As illustrated in Figure 7.4.1-1 [9], UEs connected to 6G Radio Access Network (RAN) entities can be configured to support sensing operations. This configuration enhances sensing coverage, provides additional positioning reference points for sensing measurements, and improves the accuracy and reliability of sensing results. These improvements are due to the higher density of UEs compared to base stations, which increases the likelihood that some UEs are positioned closer to the UAV than the 6G RAN entities (for example, with a UAV located between two 6G RAN entities and a UE located directly beneath the UAV). Additionally, certain UEs may be placed in reflection directions that provide a larger radar cross section (RCS) for the UAV, taking into account the UAV's RCS variations in different incident/reflection angles.

NOTE:	The term “6G Radio Access Network (RAN)” does not imply any architectural assumption, e.g. whether 6G RAN is a new or evolved RAN (compared to 5G).

The 6G sensing processing unit can gather sensing data from one or multiple network infrastructures. Upon request, the 6G network operator can provide UAV flight trajectory tracking services to trusted third-party applications, such as UAV service operators, regulatory agencies, Uncrewed Aerial System Traffic Management (UTM) systems, UAV itself, etc.

Figure 7.4.1-1: Low-altitude UAV trajectory tracing by 6G system

Thus, how to realize the low-altitude UAV supervision (e.g. UAV intrusion detection, UAV trajectory tracking) is important and challenging in 6G.

Sensing is an efficient technology for object detection by means of 6G radio signals, e.g. monitoring UAV illegal flying in a specific area. 6G network could provide sensing service by collecting sensing data, transmitting sensing data, processing sensing data, storing sensing data and support sensing result exposure to the third application platform. These sensing data has the following characteristics:

-	These data may not necessarily belong to a specific UE while these data may be produced by the relationship between the network and the physical environment.

-	These data may be produced by a UE or a base station in order to complete a specific task which needs multi-dimensional cooperation.

-	These data may have a relationship with time and space which needs efficient on-demand transmission, storage and collection.

-	These data may be collected from non-3GPP sensing sources which needs unified management in the 6G network.

To address these new data characteristics, the 6G network capability of data processing needs to be extended based on the 5G network including multi-source heterogeneous data collection, efficient and guaranteed large-scale data transmission, efficient data processing within the network, and unified data storage to support multi-node cooperative sensing and multi-node information convergence.

In the low-altitude UAV supervision scenario, the 6G network could be used for sensing the UAV intrusion such as a UAV illegally flying in a restricted area including government and company regions. In this scenario, the network security and data security need to be guaranteed due to the privacy issue. Furthermore, the historical data may be used to identify an illegal UAV. In addition to the communication property of the detected UAV itself, the data management in this low-altitude sensing scenario must be operated within the network.

In a word, the 6G network will break through the "pipeline" capability and go towards to a new type of information service network by realizing diversified data collection, transmission, processing, storage and exposure within the core network.