3GPP TSG-SA WG1 Meeting #110 	S1-252099

Fukuoka, Japan, 19-23 May 2025	(revision of S1-25xxxx)

Title:	Missing MMI codes for Supplementary Services

Source:	Apple

Agenda item:	6.3 (Rel-18 and earlier CRs)

Document for:	Discussion

Contact:	Mona Mustapha at apple dot com

Abstract: This contribution raises the issue of missing MMI codes for several supplementary services since 3G and proposes a way forward to provide a correction.

1. Introduction

RAN5 define test specifications for supplementary services using MMI codes defined in SA1 specification TS 22.030 "Man-Machine Interface (MMI) of the User Equipment (UE)". Annex B defines the normative MMI codes for Supplementary Services, as shown below:

Table B.1: Input information for handling of defined Supplementary Services

Supplementary	Service
		Service	Code	SIA	SIB	SIC

22.067
		eMLPP	75 and 75n		where n=0-4

22.072
		CD	66

22.081	
		CLIP	30	-	-	-
		CLIR	31	-	-	-
		COLP	76	-	-	-
		COLR	77	-	-	-

	In temporary mode, to suppress CLIR for a single call, enter:	
		" * 31 # <called number> SEND "	

	In temporary mode, to invoke CLIR for a single call enter:	
		" # 31 # <called number> SEND "

22.082
		CFU	21	DN	BS	-
		CF Busy	67	DN	BS	-
		CF No Reply	61	DN	BS	T
		CF Not Reachable	62	DN	BS	-

	all CF		002	DN	BS	T
	all conditional CF	004	DN	BS	T

22.083
		WAIT	43	BS	-	-
			see section 6.5.5	
		HOLD	see section 6.5.5

22.084
		MPTY	see section 6.5.5

22.087
		UUS Service 1	361 	R	-	-
		UUS Service 2	362	R	-	-
		UUS Service 3	363	R	-	-
		all UUS Services	360	R	-	-

		If UUS shall be activated when originating a call, enter:	
		" * 36X * R # <called number> SEND"	
		(X is indicating the requested UUS service)

The table above is missing the codes for the following supplementary services:

- Communication Forwarding on Not logged-in (CFNL)

- Barring of All Incoming Calls from anonymous users, assumed to be equivalent to "Anonymous Communication Rejection" (ACR)

This means that the RAN5 test cases defined in the following specifications cannot be executed on devices which have MMI code dependency:

- TS34.229-1, 15.10 Communication Forwarding on Not logged-in

- TS34.229-5, 8.9 Communication Forwarding on Not Logged-in /Configuration / 5GS

- TS34.229-5, 8.19 Barring of All Incoming Calls from anonymous users /Configuration / 5GS

This issue is also being raised in RAN5 in document R5-252248.

Note that from an SA1 perspective:

CFNL functionality is defined in TS 22.173 clause 8.2.7 Communication DIVersion (CDIV)

ACR functionality is defined in TS 22.173 clause 8.2.6 Anonymous Communication Rejection (ACR)

2. Additional observations

The SA1 specifications that define the supplementary services such as those listed below refer to either GSM / UMTS / LTE or GSM / UMTS:

TS 22.030: Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); LTE; Man-Machine Interface (MMI) of the User Equipment (UE)

TS 22.088: Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); Call Barring (CB) supplementary services; Stage 1

TS 22.082: Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); Call Forwarding (CF) Supplementary Services; Stage 1

SA1 should discuss and decide if those specifications are also applicable to LTE and 5G (i.e. IMS), and update the cover pages to include those, or whether TS 22.173 supersedes the series of (CS) supplementary services in specifications such as TS 22.081, 22.082, all the way up to 22.097.

2. Proposal

It is proposed to add the missing codes to the table in TS 22.030, a draft CR has been prepared in S1-252120 for discussion and to check if this approach is suitable.

For the release in which to apply the changes, it is proposed to discuss how far back the changes should go, whether this can be done from e.g. Rel-15 onwards.

SA1 decision is also needed on whether the changes would be applicable to LTE and 5G (i.e. IMS), and whether the related cover pages for TS 22.030, and other specifications on supplementary services (22.081, 22.082, 22.083, etc) should be updated to reflect the changes, or whether a different approach is needed.