3GPP TSG-SA1 Meeting #110	S1-252153

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.278 | CR | 0289 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Satellite regulatory requirement support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For regions which do not allow satellite phone usage, the UE is prohibited from transmitting to a satellite. As such it is important for the UE to discover if a particular cell it is trying to camp on and the associated PLMN provides terrestrial coverage or satellite coverage. When the UE (at switch-on or recovery from out of coverage scenarios) is in a region that prohibits satellite access, but receiving only 'spillover' satellite coverage from a neighbouring region where satellite access is allowed, the UE could inadvertently consider itself to be under terrestrial coverage and try and register to the network, thus violating the local regulations. To help the UE comply with local regulations, an indication to the UE of whether an LTE cell is terrestrial or satellite will need to be provided, at least in locations where a violation of regulatory requirement is possible due to ambiguity at the UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new requirement is introduced to enable an indication of whether a cell is terrestrial or satellite to be provided, subject to regulations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential non-compliance to local regulatory requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.3	Satellite Access

### 7.3.1	Introduction

EPS supports E-UTRA over satellite networks for UEs, with specific capabilities related to NB-IoT / eMTC, via satellites in Geosynchronous orbits (GSO) as well as Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO). To ensure correct operations, UEs not supporting such NB-IoT / eMTC capabilities are not allowed to access these networks via satellite access.

### 7.3.2	Support for regulatory requirements

Subject to local regulatory requirements, an EPS with satellite access shall provide UEs with an indication that the access network is via satellite.

NOTE:	This indication enables a UE to comply with regulatory requirements during network selection where e.g. the satellite access is visible to the UE in a region that prohibits UE to access satellite services.