3GPP TSG-SA WG1 Meeting #110 	S1-252163

19-23 May 2025, Fukuoka, Japan	(revision of S1-25xxxx)

Source:	SoftBank Corp., NTT DOCOMO INC., KDDI Corporation, Deutsche Telekom AG, Gilat

New use case title:	New use case on Flexible and Resilient Backhaul using HAPS Platforms

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.5

Document for:	Approval

Contact:	Shiro Fukumoto (shiro.fukumoto01@g.softbank.co.jp)

Abstract: This use case details the utilization of HAPS as a versatile and resilient backhaul platform for connecting terrestrial network sites (e.g., gNBs). It covers scenarios such as backhauling remote sites, providing temporary connectivity during outages or events, and offering redundant backhaul paths, leveraging various HAPS backhaul routes (HTG, HTS, HTH) and potentially adapting IAB concepts.

---------- Use Case template ----------

8.x	Use case on Flexible and Resilient Backhaul using HAPS Platforms

8.x.1	Description

This use case focuses on utilizing HAPS as a flexible and rapidly deployable backhaul solution for terrestrial mobile network sites (gNBs/eNBs) or other communication nodes. This is particularly relevant for:

- Connecting remote or difficult-to-reach terrestrial sites where fiber/microwave backhaul is unavailable or prohibitively expensive.

- Providing temporary backhaul during planned maintenance or unexpected outages of primary backhaul links.

- Offering resilient/redundant backhaul paths for critical infrastructure sites.

- Connecting temporary cell sites for events or emergencies. HAPS can offer different backhaul routes (HAPS-to-Ground (HTG), HAPS-to-Satellite (HTS), HAPS-to-HAPS (HTH)) and potentially use different frequency bands (e.g., microwave, mmWave) or optical links (for HTH or HTS), providing flexibility and resilience options. This leverages concepts like Integrated Access and Backhaul (IAB) but with the donor node located on the HAPS platform.

8.x.2	Pre-conditions

- HAPS platform is deployed within line-of-sight of the terrestrial site(s) requiring backhaul.

- The terrestrial site is equipped with a suitable HAPS-facing antenna/transceiver (e.g., IAB-MT functionality).

- HAPS platform has established its own backhaul connection (HTG, HTS and/or HTH) to the core network.

- Spectrum is allocated for the backhaul link between the terrestrial site and HAPS.

- Network configuration supports routing traffic via the HAPS backhaul link.

8.x.3	Service Flows

- Terrestrial site (acting as IAB-node or similar) establishes a backhaul link to the HAPS (acting as IAB-donor or providing a transport link).

- Control and user plane traffic from the terrestrial site's users is encapsulated and transmitted over the HAPS backhaul link.

- HAPS relays this traffic via its own primary backhaul (HTG, HTS and/or HTH) to the core network.

- Return traffic from the core network follows the reverse path.

- The system may dynamically switch to HAPS backhaul if a primary terrestrial backhaul link fails (resilience scenario).

- Multiple terrestrial sites may be backhauled by a single HAPS platform.

- HAPS-to-HAPS backhaul links (HTH) can be used to extend range or create resilient mesh backhaul networks in the stratosphere before connecting to a ground gateway or satellite.

8.x.4	Post-conditions

- The terrestrial site has active backhaul connectivity via the HAPS platform.

- End-users served by the terrestrial site have access to core network services.

- Network resilience is enhanced (if used as a secondary path).

8.x.5	Existing features partly or fully covering the use case functionality

- IAB (Integrated Access and Backhaul) architecture and procedures (Rel-16 onwards).

- NTN architecture elements.

- Transport network protocols.

- Multi-connectivity/redundancy features.

8.x.6	Potential New Requirements needed to support the use case

[PR 8.x.6-1] The 6G system shall support protocols and procedures, including IAB enhancements, optimized for HAPS-to-terrestrial backhaul links and providing time-frequency synchronization under extended path length, atmospheric effects and HAPS platform movement.

[PR 8.x.6-2] The 6G system shall meet service-dependent latency, throughput and reliability performance targets for HAPS-based backhaul links.

[PR 8.x.6-3] The 6G system shall enable dynamic selection, aggregation and switching among HAPS backhaul routes (HTG, HTS, HTH) based on real-time link performance and operator policies.

[PR 8.x.6-4] The 6G system shall provide mechanisms for efficient spectrum coordination and sharing for HAPS backhaul links, including operation in shared or unlicensed bands and coexistence with terrestrial and satellite systems.

[PR 8.x.6-5] The 6G system shall support HAPS-to-HAPS backhaul connectivity, including routing, resource management and self-organization of a stratospheric mesh network.