3GPP TSG-SA1 Meeting #110	S1-252202

19-23 May 2025, Fukuoka, Japan	(revision of xx-yyxxxx)

Source:	China Telecom

pCR Title:	Use case on seamless auto-recovery of network elements

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.2

Document for:	Approval

Contact:	Jianzhao Li, lijz@chinatelecom.cn

Yinglin Chen, chenyl37@chinatelecom.cn

Abstract: This use case proposes to consider a 6G capability of seamless auto-recovery of network elements to improve network resilience.

1. Introduction

Please read the use case below.

2. Reason for Change

This is a use case.

3. Conclusions

None.

4. Proposal

It is proposed to study the use case in System and Operation Aspects.

* * *Start of Change (All new text)* * * *

5.4.x	Seamless auto-recovery of network elements

5.4.x.1	Description

In 5G, when a network element fails, explicit restoration signalling is triggered to its backup element to maintain UE registration status or service sessions. A data-center-level malfunction can generate signalling traffic peaks tens of times higher than normal, potentially causing network congestion or even cascading failures.

For example, a malfunction in a commercial 5GC network resulted in the failure of all network elements in a data center. RAN and other functional network elements in the backup data center detected this failure and simultaneously attempted to restore all affected UEs and sessions. This resulted in a 20-fold surge in registration requests and a 69-fold increase in PDU session establishment requests to the backup AMF—far exceeding normal operational levels, as shown in Figure 5.x.1-1 below.

To avoid such signalling storms, the failure of a network element and its backup’s takeover should be transparent to UE, RAN, and other functional network elements, such that no restoration signalling is needed.

Figure 5.4.x.1-1: The number of registration requests and PDU session establishment requests the backup AMF received during a data-center level malfunction.

5.4.x.2	Pre-conditions

Network element A and network element B back up each other in case of failure.

5.4.x.3	Service Flows

1. Network element A fails.

2. Its backup network element B seamlessly takes over network element A. Messages sent to the service IP address of network element A are automatically routed to network element B, and network element B processes these messages as network element A does. From the perspective of UE, RAN, and other functional network elements, network element A remains functional.

5.4.x.4	Post-conditions

Messages sent to service IP address of network element A will be switched back to network element A after network element A and its missing context are recovered.

5.4.x.5	Existing features partly or fully covering the use case functionality

5G supports restoration procedures, which handle network element failures via its backup, as specified in TS 23.527. However, these procedures require network element re-selection and session re-establishment (e.g., alternative NF re-selection, PFCP sessions re-establishment.)

Context transfer procedures specified in clause 5.21.4 of 23.501 and clause 4.26 of 23.502 allow a backup network element to take over the service from the failed network element.

5.4.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 6G system shall provide mechanisms to support seamless auto-recovery of failed network elements which is invisible to other elements of the 6G system.

* * *End of Change * * * *