3GPP TSG SA WG 1 Meeting #110 	S1-252266

19-23 May 2025, Fukuoka, Japan	(revision of S1-25xxxx)

Title:	Use Case on search and rescue at remote sea via satellite network

Agenda item:	8.1.5

Source:	CSCN

Contact:	Li Qiuting (liqt@chinasatnet.com.cn),

Shang Jiece (shangjc@chinasatnet.com.cn)

Abstract: This contribution proposes search and rescue at remote sea via satellite network.

1. Introduction

“Ubiquitous Connectivity” has been identified as one of key areas for 6G study. This contribution proposes a new use case on search and rescue at remote sea via satellite network.

2. Reason for Change

This pCR introduce requirements for support the search and resucre in remote area via 6G system with satellite.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

8.x	Use case on search and rescue at remote sea via satellite network

8.x.1	Description

A maritime distress incident occurred in a remote sea area beyond terrestrial cellular network coverage. Utilizing low Earth orbit (LEO) satellite internet services, the international aid agencies is coordinating nearby vessels to implement emergency rescue operations. A temporary rescue network is established to enable real-time reliable communication among rescue coordination centers, temporary rescue vessels, shipborne UAVs, distressed vessels and personnel. Satellite network covering the remote sea ensures service stability, allowing temporary rescue coordination vessels to seamlessly communicate with neighboring national rescue centers for comprehensive ocean emergency response.

8.x.2	Pre-conditions

The distressed area is beyond terrestrial cellular network coverage.

8.x.3	Service Flows

Figure 8.x.3-1: Application scenario of search and rescue at remote sea for 6G system

- Distressed vessel transmits SOS signal.

- LEO satellites rapidly connect emergency terminals, identify distress signals, locate positions, and relay information to nearest Rescue Coordination Centre(RCC).

- Coordination center establishes cloud-edge-end network(Cloud: RCC,Edge: Rescue vessel group,end: UAVs).

- LEO Satellite creates virtual constellation-based rescue network around distress location and form temporary rescue groups with prioritized QoS. The temporary rescue group include a Rescue Command Vessel acting as the primary coordinator, responsible for on-sence rescue coordination and communication with the RCC.

- The satellite network enable the following communication links requiring guaranteed QoS:

- End-to-end communication between temporary rescue groups and distressed users

- End-to-end communication between vessel and UAV

- Intra-group communication within temporary rescue groups

- End-to-end communication between the Rescue Command Vessel and the RCC

The entire process shall support both on-demand (unicast) and multicast communication modes.

- Due to the limited coverage time of a single low-orbit satellite, multiple satellites can enter/leave this temporary rescue network, and it is necessary to ensure the continuity of rescue operations for realizing seamless switching and real-time communication between terminals in the rescue sea area and satellites.

- UAV-vessel for area search transfer the real-time data e.g 1080P images/video to the Rescue Command Vessel, and the Rescue Command Vessel may further transfer these data to the members with the group.

- The group members execute rescue strategy and operation upon locating distressed targets.

- After rescue, the temporary group is deleted, and normal service levels are restored for the users within the group.

8.x.4	Post-conditions

All terminals resume normal service levels, with LEO satellite networks returning to standard operation modes.

8.x.5	Existing features partly or fully covering the use case functionality

Requierements for positioning for 5G system with satellite access are defined in TS22.261 [14]:

-	Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support 3GPP positioning methods for UEs using only satellite access.

-	A 5G system with satellite access shall be able to provide positioning service to a UE using only satellite access and the information on positioning services (e.g. supported positioning performance). UE can be with or without GNSS capabilities

-	A 5G system with satellite access shall be able to support negotiation of positioning methods, between UE and network, according e.g. to 3GPP RAT and UE positioning capability, the availability of non-3GPP positioning technologies (e.g. GNSS).

8.x.6	Potential New Requirements needs to support the use case

[PR 8.x.6-1]  Based on application request, the 6G system is able to create the temporary group in the specified area where the satellite network is available, and provides the services for the users within the group with applicable parameters e.g. QoS, priority..

[PR 8.x.6-2] Based on application request, the 6G system is able to update and delete the temporary group.