3GPP TSG-SA WG1 Meeting #110 	S1-252286

Fukuoka, Japan, 19-23 May 2025	(revision of S1-25xxxx)

Source:	InterDigital

Use case title:	Update to 6.1 UC on 6G Infrastructure Optimization

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.2

Document for:	Approval

Contact:	Catalina Mladin, Catalina.Mladin@Interdigital.com

## Abstract:

This contribution proposes enhancements to the 6G Infrastructure utilization usecase with support of trustworthiness.

Changes include:

Step 1:

- A new step 1 is added, as the user starting to play the game had not been fully captured in the past. The description also includes some of the descriptions linking to the preconditions. We believe this addition to be necessary for the usecase, independent of the introduction of any additional functionality, as it clarifies how the following step is triggered.

- Adds that the AI Agent on the UE can access network and third-party information as knowledge sources additional to the user prompt, as described in steps 1-2 of clause 6.12 usecase.

- Adds that the AI Agent can output information for overall resource optimization, as described in clause 6.3 usecase.

Step 2 (previously step 1):

- Clarifies link to step 1 and indicates the use of additional parameters.

Step 3 (previously step 2):

- Adds trustworthiness assessment as part of the request processing, in order to proceed to 6G infrastructure optimization.

Various editorial changes are also proposed, including changing the gaming company name from X to GameCoX.

New potential requirements are added in clause 6.1.6 to reflect the usecase enhancements.

## ---------------- Change 1: -----------------------------------------------

## 6.1	Use case on Optimizing 6G Infrastructure Utilization via Resource Exposure in 6G

6.1.1 	Description

With the advent of advanced technologies such as Generative Artificial Intelligence (Gen AI), Large Language Models (LLMs), and AI-accelerated xPUs, more applications based on AI can utilize wireless network of operators which are now equipped with substantial computational and storage infrastructures that often remain underutilized in the deployed edge data centres. These computation resources may be hosted at edge sites in a distributed manner. It is in the strategic interest of these operators to provide and expose availability of these dormant resources to a third-party in a trustworthy manner. This exposure allows third-parties to leverage the computational power and AI capabilities, as value-add resources, when required. In exchange, operators can monetize these assets by charging nominal fees for offered computing resources (e.g., computing cycles) from the edge data centres at the 6G infrastructure. For instance, applications for enabling AI services may demand extensive computing resources for processing the training data as well as for inferencing (for instance for task offloading, while performing LLM training or inference) and storage utilization by third parties. In addition, operators can monetize their network’s capability to assess the trustworthiness of the value-add resources by exposing selected and abstracted information to the third parties.

This approach not only generates additional revenue streams for network operators but also ensures the optimal utilization of existing infrastructure and capabilities. Consequently, it justifies the need for new requirements in 6G networks to expose and manage these resources and capabilities effectively. By incorporating standardized interfaces, robust security protocols, and advanced resource management frameworks, 6G can facilitate efficient and secure access to these resources. This transformation would position 6G networks as comprehensive platforms for hosting in operator’s edge data centres distributed computing and AI services for third-parties, aligning with the evolving demands of the technology landscape and provide resource as a service. This unique network-aware resource exposure capability is enabled by the 6G system's centralized policy framework.

The 6G system will be built on the cloud, optimized for ubiquitous computing, and natively provide computing services (e.g. in forms of infrastructure services, platform services, and software services) [2].

Figure 6.1.1-1: Illustration of edge data centre resource usage and exposure to 3rd party

NOTE:	The figure above is just an illustration to depict where computation resources could be deployed e.g. at an edge site.

### 6.1.2	Pre-conditions

Network Operator A is equipped with significant compute, storage, and AI processing resources in deployed edge data centres providing services to UEs hosting applications . Some of these resources are used by NFs, OAM, hosted services, AI processing of network AI agents or supporting UE AI agents etc.

However, resource utilization varies based on the actual utilization by the hosted applications, network load, and during periods of low demand-often at certain times of day or night-up to 40 % of these resources remain idle. These idle resources represent untapped potential, lying unused with no opportunity for additional utilization.

Meanwhile, Operator A's customer, a gaming company called GameCoX, experiences high demand for computational resources during nighttime hours when many gamers are online on both computers and mobile devices. As games become increasingly complex and distributed, companies like GameCoX are integrating LLMs into their platforms to enhance gameplay and to derive user context (e.g. user intent). Offloading some of this processing to external infrastructure would be beneficial, particularly if GameCoX could show that, enabled by UE AI Agents, it has means to determine the idle resources it requests at a competitive rate from Operator A in a precise and efficient manner.

As an existing customer of Operator A, company GameCoX  is authorized to access Operator A’s APIs for requesting access to specific/ additional  resources at favourable rates, based on Operator A’s policies.

When playing on their UEs, users of gaming platform GameCoX consent to hosting AI agents on their UEs that leverage user context (e.g. as user intent) for the purpose of network and compute resource optimization.

### 6.1.3	Service Flows

- A user starts playing game GameCoX on their UE which include a variety of AI user prompts. The user activities result in large computation needs and includes access to LLM as part of the invoked application or Agent Y. Upon the user prompt and based on the AI model, the AI Agent on the UE accesses network (Operator A) and third-party GameCoX information as knowledge sources additional to the user prompt. Based on such knowledge sources and the user’s requirements, the AI Agent determines user-specific information (e.g., communication QoS, offloading compute, compute requirements) for overall resource optimization. The AI Agent Y provides this information to the third-party gaming platform.

2.	The third-party requests access to specific xPU (GPU, CPU etc.) or AI resources via an API provided by Operator A, based in part on the AI Agent output in step 1. The request may include trustworthiness parameters for the requester and/or trustworthiness criteria for the resources requested.

3.	The 6G network verifies the request against defined policies and authorizes access if conditions are met. As part of this step, a trustworthiness assessment of the request may be performed based on policies. The trustworthiness assessment may be based on: user, UE, third-party, location, resources requested, service requirements, reliability requirements, security requirements etc.

NOTE: Whether and how trustworthiness parameters and criteria apply (e.g. in relationship to request vs. requestor, resources, etc.) is for study in downstream 3GPP WGs.

4.	The 6G network checks with the policy framework, which in turn may confirm with orchestration system if sufficient resources are available for the expected duration based on the third-party request, by potentially leveraging the available information on the history of network usage and the trustworthiness evaluation. Upon approval, the 6G system allocates the requested resources and establishes secure access for the third-party. This unique network-aware resource exposure capability is enabled by the 6G system's centralized policy framework.

5.	The operator A could also add its own rules and policy to manage the resource allocation with the help of policy framework.

6.	The system continuously monitors resource usage and records data for charging and reporting purposes.

7.	The 3rd party is also allowed to analyse the usage in real time.

### 6.1.4	Post-conditions

1.	Third-party entity completes its tasks using the allocated resources.

2.	The 6G network releases the resources and updates the availability status in real-time.

3.	Usage data is logged, and relevant charges are generated and sent for billing.

4.	Policy function is in control of entire operation.

### 6.1.5	Existing features partly or fully covering the use case functionality

None.

### 6.1.6	Potential new requirements needed to support the use case

[PR 6.1.6-1] Based on operator policy, the 6G network shall provide suitable APIs to allow authorized third parties to retrieve availability information about computational resources (e.g. storage, AI processing units, xPUs information etc.) inside the operator managed data network (e.g. similar to Service Hosting Environment, Edge Compute domain) and to utilize these computational resources for running workloads on demand.

[PR 6.1.6-2] The 6G network shall support charging for the usage of network resources by third parties.

[PR 6.1.6-3] The 6G network shall support detailed monitoring and reporting of computational resource usage in the operator managed data network (e.g. similar to Service Hosting Environment, Edge Compute domain).

[PR 6.1.6-4] Based on operator policy, the 6G system shall provide mechanisms for efficient and secure exposure of the network capability to support Trustworthiness assessment for the purpose of increasing system Trust.

[PR 6.1.6-5] Based on operator policy, the 6G system shall provide suitable APIs for exposure of authorized Trustworthiness assessment enabled by the operator’s network for the purpose of increasing system Trust.