3GPP TSG-SA WG1 Meeting #110 	S1-252349

Fukuoka, Japan, 19-23 May 2025

Source:	Nokia

pCR Title:	Pseudo-CR on updates of use case 8.6

Draft Spec:	3GPP TR 22.870-021

Agenda item:	8.1.5

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 8.6 in FS_6G-REQ TR.

1. Introduction

This contribution proposes to clarify the service flow and refer to SA1 normative specs in the gap analysis.

2. Reason for Change

The service flow is a bit unclear and the gap analysis currently refers to RAN specs and SA1 TRs.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870-021.

* * * First Change * * * *

## 8.6	Use case on "low-energy positioning in satellite networks"

### 8.6.1	Description

This use case corresponds to a capability for 6G with satellite access to provide low-energy positioning for low-power and low-complexity devices (e.g. IoT and low-power devices) when served by a satellite network. Integrating GNSS into IoT devices operating over satellite networks significantly impacts battery consumption, especially in battery-powered UEs requiring frequent positioning updates. The extent of this impact varies based on GNSS start modes (hot, warm, or cold), power consumption of GNSS modules, reporting intervals, and device mobility. According to the analysis carried out in [26], GNSS position fixes can reduce battery life from approximately 5% up to 45% in cases of frequent position fixes. Mitigating the power consumption due to GNSS would therefore be beneficial. To minimize energy consumption, NTN signals can be used for position estimation, reducing dependency on GNSS for position information updates.

### 8.6.2	Pre-conditions

A battery-powered IoT or low-power device has been deployed in an environment where it obtains its initial position and synchronization from non-3GPP methods (e.g. GNSS) and / or terrestrial networks. A combination of non-3GPP technologies (e.g. GNSS) and 3GPP technologies, terrestrial and non-terrestrial, can be configured if compatible with energy consumption requirements.

### 8.6.3	Service Flows

1. At some point, the IoT or low-power device may lose its original positioning sources due to one of the following reasons:

-	GNSS-originated position: GNSS signal may be lost due to interference, or obstruction, or intentionally disabled to augment its autonomy.

-	Terrestrial network-originated position: the IoT or low-power device may have moved beyond terrestrial network coverage, such as in the case of asset tracking sensor installed on a container to be transported by sea, rail, or road.

- In this context, it requires the IoT or low-power device to refresh a more accurate position. The IoT or low-power device, served by the satellite access, is expected to know an approximate position from its original positioning. This requires the IoT or low-power device to refresh a more accurate position.

- Therefore, the IoT or low-power UE initiates the 3GPP positioning method over satellite to estimate its current location.

- The 6G system broadcasts to the UE via its satellite access at least the following necessary information, including but not limited to:

-	Network assistance data: Satellite ephemeris and additional assistance data to improve accuracy (e.g. ionospheric models to correct the atmospheric delay errors, etc.)

-	Reference signal for time of arrival measurements.

### 8.6.4	Post-conditions

The IoT or low-power device successfully estimates/updates its position based on information provided by the 6G system via the satellite access.

### 8.6.5	Existing features partly or fully covering the use case functionality

Positioning services are defined in TS 22.261 [14] clause 6.27, but only for terrestrial networks. They need to be adapted to satellite access context.

In TS 22.261 [14] clause 6.46, some positioning related service requirements for satellite access were not developed at stage 2 and 3 levels.

The performance for asset tracking use cases in TS 22.261 [14] annex G.2 does not include positioning requirements.

Table 8.6.5-1:  Battery life expectancy and message size to support example use cases for asset tracking

|  | Scenario | Battery Life Expectancy (NOTE 1) | Typical Message size | Maximum Message size | Typical Frequency (number of messages per day) | Typical Battery Capacity | Device Density |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Containers (note 2) | 12 years | 200 bytes | 2500 bytes | 24 | 21,6 Wh | 1,4 devices / m3 |
| 2 | Wagons | 20 years | 200 bytes | 2500 bytes | 24 | 36 Wh | 0,3 devices / m2 |
| 3 | Pallets | 7 years | 300 bytes | 300 bytes | 24 | 12 Wh | 4 devices/ m2 |
| NOTE 1: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency NOTE 2: A large containership with a mix of 20 ft and 40 ft containers is assumed. NOTE 3: All the values in this table are targeted values and not strict requirements |  |  |  |  |  |  |  |

### 8.6.6	Potential New Requirements needed to support the use case

[PR 8.6.6-1]: The 6G system shall be able to provide a mechanism for UEs with satellite access to determine its position using 3GPP positioning technology only via satellite access.

[PR 8.6.6-2]: The 6G system with satellite access shall be able to provide positioning service with the following KPIs:

Table 8.6.6-1: Performance requirements for satellite-based positioning services for energy constrained devices

| Scenario | Battery Life Expectancy (note 1) | Horizontal Accuracy (95 % confidence level) | Positioning service availability | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- |
| Containers | 12 years | [100] m | [95 %] | Outdoor | Up to [37] km/h | Container mounted |
| Wagons | 20 years | [100] m | [95 %] | Outdoor | Up to [350] km/h | Train mounted |
| Pallets | 7 years | [100] m | [95 %] | Outdoor | Up to [100] km/h | Vehicle mounted |
| NOTE 1: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency, e.g. the number of messages per day by a typical user |  |  |  |  |  |  |