3GPP TSG SA WG 1 Meeting #110 	S1-252375

19-23 May 2025, Fukuoka, Japan	(revision of S1-252072)

Source:	6G Rapporteurs

pCR Title:	Proposed merger contribution  to Clauses 5.1 and 5.2

Draft Spec:	3GPP TR 22.870v0.2.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: A new proposed revision to Clauses 5.1 & 5.2 (address Roaming, Interworking, and Service Continuity)

1. Introduction

This pCR is replaces  the following documents base on discussions on the 30 April SA1 call:

- S1-252015 (Service Continuity),

- S1-252016 (Roaming Interconnect), and

- S1-252017 (Interworking)

Combining proposed changes into one pCR provides SA1 with the overall view of the clause.

Additional revisions based on reflector discussions.  Qualcomm added as a co-source to S1-252072.

This contribution is a proposed merging of the following contributions into S1-252072:

- S1-252280 (NTT DOCOMO)

- S1-252357 (Telefónica)

- S1-252113 (Charter Communications, Cable Labs, Qualcomm, Comcast, Lenovo)

- S1-252094 (LG Electronics)

- S1-252174 (China Mobile)

Rapporteur’s proposal for S1-252095 and S1-252096 (OTD_US) be considered as proposals for enhancements to Roaming for potential inclusion into clause 5.5.

2. Reason for Change

Proposed changes and explanations.

- Clause 5.1 (General) changes

  - The editor’s note is deleted as text is proposed

  - Provides the high-level information regarding 6G requirements and the assumed “inheritence” of 5G services and requirements into 6G

  - Identifies the potential for enhancements to legacy services and capabilities

  - Provides a high level expectation of 6G service continuity

- Clause 5.2 (Migration to 6G) is deleted.  No text has been proposed and it added an additional/unnecessary Clause “level”

- Clause 5.2.1 (Roaming & Interconnect) is deleted.  Roaming requirements from 5G were already agreed to be supported in the sub-clause “Support of legacy services”.  If new 6G roaming requirements are needed, they can be added into the clause “6G enhancements of legacy/existing services”

- Clause 5.2.2 (Interworking) changes

  - This clause is “elevated to Clause 5.2 and renamed “Interworking with Legacy Systems)

  - The editor’s note is deleted as additional text is provided

  - Adds in mobility between 6G CN and the 5G CN and an editor’s note that other inter-system requirements are exlusions are FFS

  - Provides information on how requirements from TS 22.261, Clause 5.1.2.2 not supported by the 5GS apply to the 6G system with a modification

  - A note is added to provide guidance on 5G-to-6G terminlogy changes

- Clause 5.2.3 (Support for legacy service requirements) changes

  - The first editor’s note is removed as there is text in this clause,

  - The clause is “elevated” to 5.3 (and all other sub-clauses “elevated”

  - The first line is deleted as the text is captured now in clause 5.1 (General)

  - Addiitonal text is added to clarify requirements include non-3GPP and satellite access

  - Additional text is added to clarify that inter-system interworking is now in clause 5.2

  - Editorial corrections (moved a “space” in two TSs (TS 22.011 and TS 22.185) in (new) clause 5.3.3

  - The exception text concerning voice service continuity is deleted as it is now in clause 5.2

  - A note is added to clause 5.3.3 to provide guidance on 5G-to-6G terminlogy changes

  - The editor’s note concerning “exceptions” has been edited based on proposed text in this pCR

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870v0.2.1.

* * * First Change * * * *

# 5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

Editor’s Note:  The proposed sub-clauses are provided as guidance from the rapporteurs based on the previously agreed EN (above).   If text is not agreed for any clause or sub-clause, these clauses/sub-clauses will be removed from the document.

## 5.1	General

The 6G system shall support new 6G services and capabilities and it is assumed that the 6G system shall support existing 5G services and system requirements, unless explicitly identified.

It is expected that the 6G system may enhance existing legacy services and capabilities, maximizing backwards compatibility.

Improving user experience is an on-going concern for network operators, and service continuity during changes in network access or connectivity is especially important for services that are sensitive to disruptions.  The 6G system is expected to support service continuity unless explicitly identified.

Options to migrate to 6G shall be restricted within 3GPP standards, to prevent fragmentation, security and compatibility issues. The following aspect shall be considered for migration:

- introducing an infrastructure built specifically for 6G services that will enable the operators to provide the 6G features with the promised high performance and intended benefits for the users and will avoid dependency between the networks during migration.

### 5.2	Interworking with Legacy Systems

Subject to operator’s policy, the 6G system shall support mobility between the core network of the 6G system and a 5G core network with minimum impact to the user experience (e.g. QoS, QoE).

Subject to operator’s policy, the 6G system shall support mobility between the core network of the 6G system and an EPC with minimum impact to the user experience (e.g. QoS, QoE).

The 3GPP system shall support a mechanism enables the UE to access both 6G and 5G core networks.

The 3GPP system shall support shared subscription information between 5G and 6G core networks so that the UE can be served by both networks by using the same subscriber identity to access 5G and 6G core network.,

The 3GPP system shall provide a mechanism that enables the UE to decide whether to use 6G or/and 5G network for a certain service/application if both 5G and 6G networks are available.

6G System shall support session continuity for all the access technologies provided by 4G, 5G or 6G, including IoT RAT-Types that support session continuity.

Editor’s note: other inter-system requirements (e.g. between 6G and 4G), or exclusions, are FFS.

Editor’s note:   the above requirements are subject to further discussion.

Requirements in [14] clause 5.1.2.2, related to inter-RAT capabilities not to be supported by 5GS, apply similarly to 6GS with the following modification:

-	voice service continuity from the radio access network of the 6G system to UTRAN CS shall not be supported.

NOTE:  Terms referring to 5G (e.g., “the 5G system”, “NG RAN”) should be implicitly replaced by the corresponding terms for 6G (e.g., “6G system”, “radio access network of the 6G system”) in those requirements.

### 5.2.1	Interworking with non-3GPP accesses

Interoperability among the various access technologies supported by the 6G system will be imperative. For optimization and resource efficiency, the 6G system will select the most appropriate 3GPP or non-3GPP access technology for a service, potentially allowing multiple access technologies to be used simultaneously for one or more services active on a UE.

The 6G system shall be able to support a user to access network services via 3GPP and/or non-3GPP access (e.g., WLAN or Wireline).

The 6G system shall be able to support 5G system requirements for non-3GPP access as defined in [14].

Subject to operator’s policy, the 6G system shall support mobility between the 6G 3GPP access and non-3GPP access, with minimum impact to the user experience (e.g. QoS, QoE).

### 5.3	Support for legacy service requirements

#### 5.3.1 	Introduction

The following sub-clauses cover a list of services to be supported (or not supported) and other 5GS requirements implicitly supported (or explicitly excluded).

The requirements in these sub-clauses apply also to non-3GPP access and satellite access, which are assumed to be supported by the 6G system.

6G requirements about inter-system interworking are defined in clause 5.2.

#### 5.3.2	Support of legacy services

The 6G system shall be able to support the following services:

-	Mission Critical Services, i.e., MCPTT, MCData, MCVideo, ref TS 22.179 [53], TS 22.280 [54], TS 22.281 [55]. and TS 22.282 [56],

-	Message service, ref TS 22.262 [57],

-	SMS, ref TS 22.101 [58],

-	Multimedia communication services, ref TS 22.101 [5f], TS 22.261 [14], TS 22.173 [59],

-	IMS Multimedia Telephony Service, ref TS 22.261 [14], TS 22.173 [59],

-	Roaming services, ref TS 22.011 [60], TS 22.101 [58], TS 22.261 [14],

-	Location and positioning services, ref TS 22.261 [14], TS 22.071 [61],

-	Broadcast and Multicast Services, ref TS 22.261 [14],

-	Emergency Services, ref TS 22.101 [58],

-	PWS, ref TS 22.268 [62],

-	MPS, ref TS 22.153 [63],

-	Lawful Intercept, ref TS.22.261 [14], and

-	Other regulatory services, based on regional/ national regulatory requirements.

Editor’s note: other services can be added, FFS.

The following services do not need to be supported by the 6G system:

-	CS related telephony services, and the seldom used IMS supplementary services, e.g., CCBS (Completion of Communications to Busy Subscriber), CCNR (Completion of Communication on No Reply) and CCNL (Completion of Communications on Not Logged-in).

Editor’s note: other items can be added, FFS.

#### 5.3.3	Support of other legacy requirements

The 6G system shall be able to support 5G system requirements (functional and performance requirements) definedin TS 22.261 [14], TS 22.104 [64], TS 22.011 [60], TS 22.173 [59], TS 22.071 [61], TS 22.262 [57], TS 22.185 [65], TS 22.186 [66], TS 22.125 [35], TS 22.263 [67], TS 22.115 [68], TS 22.101 [58], TS 22.153 [63], TS 22.289 [69], TS 22.468 [70], TS 22.368 [71], TS 22.156 [28], TS 22.137 [6].

NOTE:  Terms referring to 5G (e.g., “the 5G system”, “NG RAN”) should be implicitly replaced by the corresponding terms for 6G (e.g., “6G system”, “radio access network of the 6G system”) in those requirements.

Requirements in this clause do not apply to requirements involving the 5G system and other systems/RATs (e.g., E-UTRAN, UTRAN, GERAN).  Specific 6G system requirements on inter-system interworking are defined in clause 5.2.

Editor’s note: exceptions are FFS, e.g. about 5GS legacy requirements (inherited from 4G, or under ongoing discussion whether to be supported or simplified in 6G).

Editor’s note: the list of specs is FFS.

* * * Second Change * * * *

Renumber the remaining clauses in Clause 5 to align.

- Security for 6G (clause 5.3) becomes clause 5.4

- Resislience (clause 5.4) becomes clause 5.5

- 6G enhancements of legacy/existing services (clause 5.5) becomes clause 5.6

- Sustainability and Energy Efficiency (clause 5.6) becomes clause 5.7

- Network Aspects (clause 5.7) becomes clause 5.8

- Device Support (clause 5.8) becomes clause 5.9

* * * End of Changes * * * *