3GPP TSG SA WG 1 Meeting #110 	S1-252399

19-23 May 2025, Fukuoka, Japan	(revision of S1-252331)

Source:	Huawei

pCR Title:	Use case on UE energy efficiency for XR rendering/AI tasks

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Eddy Hall

edward.hall@huawei.com

Abstract:  it is proposed a new use case on UE energy efficiency for XR rendering/AI tasks.

* * * First Change (new text) * * * *

# 5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

## 5.X	Use case on UE energy efficiency for XR rendering/AI tasks

### 5.x.1	Description

User requirements for data rates and quality of service (QoS) are growing exponentially. Running processing-intensive applications (e.g. XR rendering application, AI inference application) at the UE is constrained by limited battery capacity and energy consumption of the UE. High battery consumption creates serious obstacles that limit users to fully enjoy such applications on their devices. For example, the higher the battery consumption is, the shorter the battery life is. On the other hand, even if the high battery consumption is only within a short time duration (e.g., several minutes), it leads to the thermal issue, which significantly impact the user experience. Therefore, to extend the battery life, as well as to avoid the thermal issue, the UE energy consumption should be reduced.

In order to reduce UE’s energy consumption for processing capabilities, offloading the XR rendering/AI tasks from UE to the core network or Service Hosting Environment is a promising trend since the XR rendering/AI tasks (e.g., rendering, AI inference) does not have to be executed locally. However, UE still spends certain amount of energy for communication in order to 1) transmit uplink data for XR rendering/AI tasks to the core network or Service Hosting Environment, and 2) receive results of the XR rendering/AI tasks from the core network or Service Hosting Environment.

Normally, the offloading decision is based on data transmission status and processing resources availability in the cloud in order to maintain deterministic execution latency. Different XR rendering/AI tasks offloading decision will bring different energy consumption of UE for both data transmission and processing.

Therefore how to minimize the combined energy consumption of UE needs to be considered in 6G.

Figure 5.x.1: Combined energy consumption of UE for XR rendering/AI tasks

### 5.x.2	Pre-condition

1. Processing resources are deployed in the core network or Service Hosting Environment.

2. The UE#1 and UE#2 support to map the XR rendering/AI tasks  from the application to the local and/or the remote (in the core network or Service Hosting Environment).

### 5.x.3	Service flow

Figure 5.x.3: UE energy efficiency for XR rendering/AI tasks

- Both UE#1 and UE#2 are successfully registered with the 6G network. Both UE#1 and UE#2 are visiting the same applications (e.g., XR rendering application, AI inference application).

- At T#1,  the UE#1 performs the XR rendering/AI tasks locally since the energy consumption of local execution (Ecomputing) is significantly lower than the energy consumption of data transmission/reception required by the task offloading (Ecommuncation).

At the same time, the UE#2 offloads data to the MEC server as the energy consumption of data transmission/reception required by the task offloading (Edata transmission) is significantly lower than energy consumption of local execution (Eprocessing).

- At T#2, the data transmission link changes. the UE#1 offloads tasks to the MEC server as the energy consumption of data transmission/reception required by the task offloading (Edata transmission) is significantly lower than energy consumption of local execution (Eprocessing).

At the same time, the UE#2 decides to perform the XR rendering/AI tasks locally since the energy consumption of local execution (Ecomputing) is significantly lower than the energy consumption of data transmission/reception required by the task offloading (Eprocessing).

In other words, the combined energy consumption for both data transmission and processing are minimized for each UE.

### 5.x.4	Post condition

The XR rendering/AI tasks offloading decision minimizes the energy consumption at the UE while satisfying the execution delay of the XR rendering/AI tasks.

### 5.x.5	Existing features partly or fully covering the use cases functionality

< Highlight existing features in the existing set of normative specifications that partly or fully cover this use case.>

| Specifications and clauses | Examples of Existing Requirements | Gap Analysis |
| --- | --- | --- |
| TS 22.261 6.15 Energy efficiency | The 5G access network shall support an energy saving mode with the following characteristics: - the energy saving mode can be activated/deactivated either manually or automatically; - service can be restricted to a group of users (e.g., public safety user, emergency callers). NOTE: When in energy saving mode the UE's and Access transmit power may be reduced or turned off (deep sleep mode), end-to-end latency and jitter may be increased with no impact on set of users or applications still allowed. The 5G system shall support mechanisms to improve battery life for a UE over what is possible in EPS. The 5G system shall optimize the battery consumption of a relay UE via which a UE is in indirect network connection mode. The 5G system shall support UEs using small rechargeable and single coin cell batteries (e.g. considering impact on maximum pulse and continuous current). | Normally, the offloading decision is based on data transmission status and processing resource availability in the cloud in order to maintain deterministic execution latency. Different XR rendering/AI tasks offloading decision will bring different energy consumption of UE for both data transmission and processing. During task offloading decision, the requirements to optimize the battery consumption of UE are not considered in 5G. |

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-001] 	Subject to user consent, the 6G system shall support mechanisms to optimize the energy consumption of a UE via enabling task offloading (e.g. XR rendering/AI inference) from the UE to the core network/Service Hosting Environment, considering battery life and thermal issue of UE.

* * * End of Change * * * *