3GPP TSG-SA WG1 Meeting #110	S1-252442

Fukuoka, Japan, 19-23 May 2025	revision of S1-252294

Title:	Use case on ground stations access satellite network

Agenda Item:	8.1.5

Source:	CSCN，CATT，ZTE

Contact:	Qiuting Li, <liqt@chinasatnet.com.cn>

Qiyao Xiang, <xiangqy@chinasatnet.com.cn>

Yuling Cui, <cuiyl@chinasatnet.com.cn>

Abstract: This document discusses the potential demand for satellite networks to provide access capability for feeder link establishment with ground stations in 6G scenarios.

1. Introduction

For 6G systems, it is expected that a ground station can flexibly access the satellite network to establish feeder links to enable rapid deployment of local core and data networks.

This flexibility can facilitate satellite infrastructure provision from a satellite operator to global customers (e.g., enterprises and terrestrial network operators who plan to support satellite access). Besides, with service link and feeder link using unified air interface technology, satellite operators can perform resource trade-off between service link and feeder link based on its global business conditions, which can improve satellite resource utilization efficiency and flexibility in resource allocation.

2. Reason for Change

This paper introduces a use case where a ground station can flexibly access to the satellite network as needed, establish a satellite-ground feeder link and conduct data transmission.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change (All new)* * * *

8.x	Use case on ground stations access satellite network

8.x.1	Description

Currently, in 5G-based satellite networks, the feeder link is relatively left to the network implementation not having to support the technologies defined for service link. In such case, there are some drawbacks when deploying legacy feeder link and ground stations.

- Onboard resources are usually statically allocated between ground stations and user terminals, where feeder links and user links operate as isolated, dedicated resources that cannot be dynamically scheduled based on service demands.

- A satellite may only be able to establish one feeder link with one traditional ground station located in a certain country, even when the satellite footprint can cover different countries, it may not be used to serve other countries due to data localization regulatory policies and communication security requirements of other countries. This may cause more satellites (and corresponding ground stations) infrastructure to be deployed for these other countries in the end.

- In disaster case or remote regions like oceans and deserts, emergency devices and local network face deployment limitations due to reliance on fixed ground stations, hindering rapid response in critical scenarios or establishing a local communication network

Considering the above issues, 6G networks supporting satellite access should support dynamic sharing and flexible scheduling resources for feeder links and user links, while integrating rapid deployment and flexible management capabilities for localized satellite data landing by supporting pluggable ground stations. Ground stations can access satellite networks on-demand, acting as localized data landing and backhaul nodes capable of providing users with on-demand end-to-end real-time services whenever required.

8.x.2	Pre-conditions

Operator A is a satellite operator that provides flexible and rapid network deployment to enable a variety of communication services. Operator A can provide satellites access and network service globally, e.g., directly to customers, or just provide satellite access capability for networks of other terrestrial operators.

Operator B and C are terrestrial network operators from two neighboring countries. The Operator A can be from another country.

Alice and Bob are polar scientists work in a scientific research institution, and have subscribed to the satellite access and local network service of Operator A.

Tom and Jerry are travel enthusiasts, Tom has subscribed to the satellite access service of Operator B, while Jerry has subscribed to the satellite access service of Operator C.

Ground station D, E and F are ground stations provided by Operator A, which is used to offer flexible feeder link establishment capability for the scientific research institution and operator B and C. It can be deployed anywhere to provide the backhaul  according to business needs and regulatory requirements (e.g. data landing).

8.x.3	Service Flows

Service Flow A:

- Alice and Bob plan to conduct a scientific research expedition in the Arctic.

- To provide communication support to the polar scientists, the scientific research institution where Alice and Bob work has ordered the polar communication services from Operator A.

- Operator A deploys localized core network and ground station D at the Arctic scientific research center, providing scientists engaged in polar research activities with satellite-enabled communication capabilities to access polar-specific business system.

- The ground station D accesses to the Operator A satellite network, also acts as a backhaul node connecting the satellite and the localized core network.

- After departing from the Arctic scientific research center to conduct their scientific expeditions, Alice and Bob’s mobile phone successfully connect to the localized core network via satellite. Through ground station D and the localized core network, they access services deployed in the Arctic scientific research center.

Service Flow B:

- To provide communication services in the area without the terrestrial network coverage or with poor terrestrial network coverage, Operator B and C sign the agreement respectively with Operator A to provide the satellite access for their own users.

- In order to meet the regulatory requirement (e.g. data landing), Operator B and C also order data transmission services via satellite from Operator A.

- Operator A deploys ground station E and F for Operator B and C respectively as data landing nodes to achieve data transmission.

- Ground station E and F can access to the same satellite to establish the feeder links.

- Tom and Jerry plan to go hiking in a mountainous area located along the national border of two countries, where without the terrestrial network coverage.

- Tom wants to share the mountain view with friends, while Jerry wants to make a phone call with his mother. Since they were within the coverage area of the same satellite, their mobile phones connected to that satellite and successfully registered to their own network.

- Tom posts the photos in his social media account through Operator B’s core network, while Jerry talks with his mother through Operator C’s core network.

8.x.4	Post-conditions

Thank to ground station can be flexibly deployed, data transmitted via satellite access can be landed in specific area, subject to regulatory requirements of different countries and operators’ policy.

8.x.5	Existing features partly or fully covering the use case functionality

None.

8.x.6	Potential New Requirements needed to support the use case

[PR 8.x.6-1]: Subject to regulatory requirements and the operator's policy, the 6G system with satellite access shall able to support ground station flexible access to satellite with 3GPP access technology.

End of CHANGES