3GPP TSG-SA WG1 Meeting #110 	S1-252485

19-23 May 2025, Fukuoka, Japan	(revision of S1-252313)

Source:	Ericsson

New use case title:	New use case on Network assisted 3D mobility

Draft TS/TR:	3GPP TR 22.870v0.2.1

Agenda item:	8.1.4 Integrated Sensing and Communication

Document for:	Agreement

Contact:	Daniel Lönnblad (Daniel.lonnblad at ericsson.com)

Abstract: Use cases to introduce Network Assisted 3D Mobility in TR 22.870.

## 7.x	Use cases on Network assisted 3D Mobility

### 7.x.1	Description

Network Assisted 3D Mobility refers to use cases where the mobility of persons or connected vehicles such as cars or drones is assisted by the network. The assistance can be in the form of help of e.g., sensing, navigating and positioning of humans or vehicles. Examples can be found in Autonomous Drone Transport, Smart Intersections, and Assisted Vehicles.

Figure 1: Network-assisted 3D Mobility example scenario

Autonomous Drone Transport

In Autonomous Drone Transport, flying drones are carrying goods in urban areas. The drones are equipped with sensors (camera, GPS, etc.) and a processor. Through the onboard 6G device they get a reliable communication link and that allows them to be positioned by the network. Over the communication channel the drone can receive timely information about nearby flight paths and activities and can receive recommended actions for fast and high energy efficient paths. The drone can also receive its corrected position as well as map data from the network. It can also, depending on need, offload tasks such as image processing from the camera feed and share the resulting data on the surrounding environment with the network. The users and deliverers of the drone transport service can count on a reliable, efficient, just-in-time delivery that can be tracked at any time, and people in the streets can trust that no collisions will occur by the drones passing over their heads.

Smart Intersections

Mobile communication infrastructure featuring sensing capabilities would enable a wide-coverage and well-connected radar mesh. Such network can be seen as a set of wide area sensors, that will add value to its users at nearly no extra cost. Safety and efficiency of urban traffic can be improved by means of real-time spatial and temporal sensing of critical roads and intersections to support the vehicle when its onboard sensors do not have line of sight. For example, even before a vehicle gets to a junction, it will be made aware of the conditions around the junction (such as vulnerable pedestrians or other vehicles coming around the corner) so that the vehicle can take the right precautions to advert collision or accidents. Different from monitoring with video cameras, such sensing approach would allow perception even in complete darkness, or in adverse weather conditions like rain or fog (with certain performance limitations). Also, privacy concerns for citizens would be reduced as no actual video or picture would be used. Finally, sharing the sensed real-time data with traffic control centres can help to improve traffic flow management in smart cities.

Assisted Vehicles

Vehicles today have many onboard sensors to support applications such as cruise control, pedestrian detection etc. In the 6G era, the enabling of advanced automotive features such as autonomous driving and autonomous coordinated manoeuvring is envisioned through leveraging the wide area sensors from the network, in addition to on-board vehicle sensors. Autonomous driving would allow vehicles to be navigated through challenging traffic conditions and terrains without the need for human interaction.

In addition, the autonomous coordinated manoeuvring feature would allow multiple vehicles to autonomously navigate through roads and highways in a coordinated fashion to ease traffic congestion and improve the traffic flow. These advanced automotive features would require a comprehensive detailed knowledge of the environment surrounding the vehicles as well as high precision localization and positioning. This would be enabled by the fusion of the wide-area sensor information provided by the network, vehicle on-board sensors and/or even sensors embedded in the transport infrastructure.

Situational awareness for mission critical applications

Just like for advanced mobility, situational awareness is vital in mission critical operations such as those seen in public safety. This includes firefighting, law enforcement, and emergency medical services. It involves understanding the environment, ongoing events, and the projection of these into the future, to be used as input to decision making and to manage risks.

For supporting improved situational awareness for mission critical use, the 6G system can be used to transfer, generate and process relevant information. Sensor information coming from devices worn by public safety personnel can be used to monitor health status, track position and mobility, and to provide instant audio-visual information for scenario understanding. The 6G system can also provide location services, either for specific users or to monitor the flow and density of people in an area. Sensing is an additional service that can provide information for scenario understanding through object detection and tracking, or through collection of mobility information related to vehicles and people in an area.

Additionally, mission critical use of the 6G system in resource constrained scenarios leads to a need to differentiate or prioritize services, e.g., between public safety users and non-public-safety users, or even among public safety users. In some scenarios, a service to a user may need to be terminated to free up resources for a specific public safety user. Such prioritization is not only valid within a specific service domain, but also between services that are relying on the same radio or system resources, e.g., positioning, sensing and communication.

Key Value impact analysis

Energy resources: Less energy/fuel would be consumed due to driving via optimized routes, however the additional sensing equipment will require energy therefore energy consumption needs to be considered while developing the required solutions.

Material resources: Need for waste treatment is expected to reduce due to decreased number of accidents. In addition, if the application of sensing technology leads to producing less or smaller vehicles, less material will be needed.

Emissions: Improving the traffic flow and assisted driving should lead to reduction of emissions.

Health: Reduced transport-related accidents should lead to enhanced safety and well-being,

Trustworthiness: Preserved privacy is enabled through network-assisted mobility compared to video-based solutions, however there are still potential risks for privacy intrusion associated to localization data, which can be mitigated with the help of privacy preserving technologies, and an increased risk of cyber-attacks or unintended errors which may lead to accidents.

Work & income: With self-driving vehicles the job opportunities for drivers are reduced. Potential improved efficiency from freeing resources e.g. by providing the ability of working during transport/driving.

Infrastructure: Safer road transport systems are a result of the use-case and the access to reliable transports increases. However, building network infrastructure to meet the requirements for high reliability of services can be costly.

### 7.x.2	Pre-conditions

-	Vehicles and drones using the service are equipped with UEs.

-	The service is offered in a well-defined service area/service volume.

These use cases are fulfilled within the service volume as shown in table 7.x.2-1, a defined volume where the service is offered, that the specific use case would require (e.g. an airspace volume for drones, ground level for cars).

Table 7.x.2-1:	Service Volume

| Scenario | Service volume |
| --- | --- |
| Autonomous Drone Transport | Area 10 km*10 km Height: 150 m |
| Smart Intersections | Area 200 m*200 m Height: 50 m |
| Assisted Vehicles | Area: Circle with 110 m radius with vehicle in centrum. Height: 50 m |

### 7.x.3	Service Flows

In this use case, illustrated in Figure 1, vehicles (cars, AGVs, drones, etc.) are relying on the network assisted 3D mobility service that includes network nodes and devices for localization of connected and unconnected objects and for determination of their properties such as size and velocity, contextual info, trajectories etc. Vehicles have a reliable connection to the network, with services available in a well-defined service area. Networks measure the physical environment in traffic scenarios and analyse detected objects, and aggregate data of device positions and from this large data set extract information to relay to vehicles. Position data shared with vehicles can include position of vulnerable road users (VRUs), such as pedestrians and cyclists.

This can be done on several levels; networks can provide raw or processed environmental and position data, or navigational support ranging from collecting and sharing of data, over navigation assistance, to full operation, leveraging on beyond-communication capabilities. The network can thereby with the network assisted 3D mobility service, support vehicles with different levels of autonomy and different modes of operation and enable smart transport in urban areas.

Furthermore, there is a possibility to use the understanding of the physical environment around nodes and devices to improve communication to vehicles by tailoring beams, avoiding blockers, etc.

Some examples of the services provided by networks are: network assistance info (warnings, trajectories, object locations), network assistance map (digital 3D map data), network navigation (path/route recommendations) and context-aware communication (path selection, scheduling).

### 7.x.4	Post-conditions

Connected vehicles using the service get up to date traffic, position and map information, thereby increasing traffic safety.

### 7.x.5	Existing features partly or fully covering the use case functionality

Based on operator policy, 5G system shall be able to provide means to predict and expose predicted network condition changes (i.e. bitrate, latency, reliability) per UE, to an authorized third party.

-	Network positioning available in 5G standard as in TS 22.261 and TS.22.125

Table 7.3-1: Positioning performance requirements

| Scenario | Accuracy (95 % confidence level) |  | Availability | Heading | Latency for position estimation of UE | UE Speed | Corresponding Positioning Service Level in TS 22.261 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal accuracy | Vertical accuracy |  |  |  |  |  |
| 8K video live broadcast | [0.5 m] | [1 m] | 99% |  | 1s | [<120 km/h] | 5 |
| Laser mapping/ HD patrol | [0.5 m] | [1 m] | 99% |  | 1s | [<120 km/h] | 5 |
| 4*4K AI surveillance | [0.1 m] |  |  |  |  | [<60 km/h] |  |
| Remote UAV controller through HD video | [0.5 m] | [1 m] | 99% |  | 1s | [<120 km/h] | 5 |
| Periodic still photos | [0.1 m] | [1 m] |  |  |  | [<60 km/h] |  |
| NOTE: The positioning accuracy in this table is not related to navigation or safety. |  |  |  |  |  |  |  |

-	Sensing requirements for 5G as specified in TS 22.137. Dependent on what object to give assistance to the requirement on the sensing service will vary. For the drone/vehicle use case described above sensing service category 2 or 3 specified in Table 6.2-1 performance requirements of 5G Wireless sensing TS 22.137 would be required.

Table 6.2-1:	Performance requirements for 5G Wireless sensing

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 2 | 95 | 2 | 5 | 1 | N/A | 1 | 1 | 1000 | 0.2 | 0.1 to 5 | 5 | Outdoor (e.g., detection of human, UAV) |
|  | 3 | 95 | 1 | 1 | 1 [3], [4] | 1 | 1 [3], [4] | 1 x 1 [3] | 100 [2], or 1000 (NOTE 3); 5000 for detection in highway | 0.05 to 1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV) |
|  | NOTE 1: For sensing service categories to which UAV, human or vehicle is a sensing target, the typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of human is 0.5m x 0.5m x 1.75m, and the typical size of vehicle is 7.5m x 2.5m x 3.5 m. NOTE 2: The safe distance between pedestrian/vehicle and power transmission station/line is 0.7m/0.95m. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: This value is derived from the water level where people feel difficulty in walking. |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.x.6	Potential New Requirements needed to support the use case

The following requirements for Network Assisted 3D Mobility, which is a service offered to vehicles, drones etc, are derived from the use cases above and are valid within the service volume defined in Table 7.x.2-1.

[PR 7.x.6-1] The 6G system shall be able to predict if communication connection will be bad or lost to allow to mitigate for the loss of Network assisted 3D Mobility service.

[PR 7.x.6-2] The 6G system shall support confidentiality, integrity protection, and privacy of the spatial data related to Network Assisted 3D Mobility that will be collected and processed.

[PR 7.x.6-3] The 6G system shall be able to offer this Network assisted 3D mobility service by ensuring the KPIs for communication, location and sensing are fulfilled simultaneously.

[PR 7.x.6-4] The 6G system shall be able to prioritize the components used in Network Assisted 3D Mobility, i.e., communication and the non-communications service sensing and positioning.

[PR 7.x.6-5] The 6G system shall support following KPI’s for Network Assisted 3D Mobility:

- For the communication part:

  - User experienced data rate in the range of [1-10Mb/s], depending on use case. Lower for sending warnings and assistance and higher for digital maps.

  - Round trip latency in the range of [40ms].

  - Service availability for communication 99.99% within service volume.

  - Connection density in the range of 104 devices/km2 (all served vehicles on ground and in the air).

- For the spatial data part:

  - Location accuracy in 3D, [1m] with 90% probability and within 99% of the service volume

  - Sensing of objects in line with category 2 or 3 in Table 6.2-1 in TS 22.137, within 99% of the service volume.