3GPP TSG SA WG 1 Meeting #110 	S1-252492

19-23 May 2025, Fukuoka, Japan	(revision of S1-252188)

Source:	CATT

pCR Title:	Update of 9.7 Use Case on Holographic Communication

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.6

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes to update use case on holographic communication to solve the editor’s note and address more potential requirements to realize the use case.

1. Introduction

In SA1 #109 meeting, the use case on holographic communication is agreed and captured in TR22.870 clause 9.7. However, the KPI values and PR2 are marked with editor’s note for future study. Also, the necessary requirements to support this holographic communication use case are not fully identified and captured. This contribution proposes to address the above missing work.

2. Reason for Change

1. Reword PR2 to remove EN.

High-quality holographic communication may require ultra-high bandwidth, ultra-low latency and lower packet loss. Due to the impact of device capabilities and the network status, sometimes the demand on bandwidth needs to be reduced by powerful compression algorithms. As a result, the processing latency will increase and consume the delay budget reserved for the transmission, while the reliability may be also impacted. Thus, the extreme demands may not always be satisfied simultaneously. The QoS and policy mechanisms shall be enhanced to adapt to the varying balance of quality to ensure immersive Quality of Experience (QoE). The above consideration will be reflected in the PR2 wording.

2. KPI value will be updated with the same consideration.

3. Introduce PR3 for synchronization.

The generation of hologram requires the convergence of multiple data streams with different media characteristics, so the synchronization becomes critical for the user to have realistic, seamless, and interactive 3D visual experiences. This will be addressed by PR3.

4. The description of the use cases is updated to align with the requirements.

Change to 2492

Delete PR3 and PR4, and reverse the deletion of 3rd ENs (for Nokia and Qualcomm comments).

modify KPIs table and add reference for the value.as well as reverse the deletion of 1st EN (for Nokia, Ericsson, Qualcomm, Huawei comments)

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.870 v0.2.1

* * * First Change * * * *

## 9.7	Use case on holographic telepresence in Healthcare

### 9.7.1	Description

Holographic telepresence will enable users located in multiple remote locations to interact with each other in real-time as if they were together in a single shared space – for example, one projected as holographic presence appearing in the office to present the document in front of other colleagues while actually being in the car. The 6G system is expected to facilitate such seamless, real-time immersive interaction, allowing participants to engage in activities ranging from virtual meetings to collaborative work and remote medical consultations. Utilising immersive media such as hologram can enhance communication efficiency and user experience, bridging geographical distances and creating a sense of co-presence among users.

Although audio and video calls can realise interaction between the participants in different locations, those interactions are still very different from those which can be achieved when all participants are in the same physical location. The reasons of this difference can be the limitation of capturing devices such as cameras and microphones to capture the atmosphere of one location as well as the limitation of displays to render that atmosphere in another location. With the real-time telepresence based on holography and avatar technology, the participants in multiple locations are possible to enjoy truly immersive experience as if all of them would interact in the same physical location. However, to realize holographic communication, there are multiple challenges that 6G network need to address.

Bandwidth, latency and Computing of Holographic communication

AR/VR has been identified as one of the important technologies to realise immersive communications in 5G. The immersive experience can be further enhanced with more immersive media including hologram, avatar and multi-sensorial media. Hologram displays require a large volume of data about the user’s motion and sensory inputs captured by cameras, sensors and actuators as well as volumetric data from multiple viewpoints to accommodate 6DOF movements of the observer [104]. Thus, the synchronized multi-modal data streams for holograms may require extremely high bandwidth, potentially in the range of100 Gbps or beyond [105], [106]. To reduce the challenge to the transmission bandwidth, various compression and decompression technologies are developed such as point-cloud techniques, and even powered by AI. Also holographic avatars or computer-generated holograms to replace photo-realistic hologram are considered, and may reduce the demand on transmission rate to 1Gbps or lower. However, the additional processing latency may consume the delay budget reserved for transmission while the increased computation workload must be optimized or offloaded to support portable display devices that the users can access ubiquitously.

Security of holographic communication

Even with such ultra-high data rate communications, confidentiality must be guaranteed as it is today. Widely used encryption algorithms can only support data rates up to 38.70 Gbps on mobile environments [107], [108]. Existing algorithms cannot cope with the required data rates for communications utilizing immersive media so that shall be enhanced in 6G systems. The 6G system shall support encryption and integrity protection algorithms that can handle data rates in the 100 Gbps range. As the computational performance of the chipset on UE improves over the years, and the exploration of distributed computing technologies, the possibilities of the existing algorithms achieving the required performance also improves. Considering the amount of data which needs to be encrypted and integrity protected, having a faster algorithm will improve overall efficiency.

On the other hand, the captured and transmitted data for the holograms contains a significant amount of personal biometric information such as facial features, voiceprints, which need proper privacy protection to ensure the compliance to regulatory requirements.

Key Value Impact Analysis

Economic Growth:  Holographic communication cannot be achieved without technological advancements in many areas including capturing devices, displays and communications. Once holographic communication is achieved, we can expect a new type of collaborations in many industries, which can lead to further economic growth.

Emissions: Reduction of emissions will be expected by reducing physical travels.

Inclusion: Holographic communication can realise closer interaction among parties more frequently than ever without traveling. It also brings an opportunity for those who are unable to travel to explorer a remote location as if they are actually in that location.

### 9.7.2	Pre-conditions

A patient Alice is in the house, while the practitioner Bob is at the clinic A and the practitioner Tom is at the hospital B.

The house, clinic and hospital are equipped with dedicated sensory system (including multiple cameras, microphones and other sensors), holographic display device and a mobile communication device, while the practitioners additionally have XR devices to capture the pose and gesture, and haptic information.

The operator managed edge/cloud-based application servers can execute avatar generation, hologram computation and rendering.

The mobile network supports 6G as well as the communication devices do.

### 9.7.3	Service Flows

Scenario#1 One-to-One diagnostic

- Alice and Bob start the holographic communication application on the mobile communication devices.

- They enter a session ID to establish a connection between two locations.

- They connect their displays to mobile devices to start receiving/sending the holographic data.

- The sensory system in Alice’s house captures her image and status and then send them to the cloud-based application server via mobile device for processing and rendering.

- The application server generates Alice’s hologram data based on the received multiple data streams with individual modal media such as audio, video, haptic information, etc. and sends it to the mobile device in the clinic.

- Upon receiving the hologram data, the display can project Alice’s 3D image in the clinic.

- Bob with XR device can examine Alice’s status as if Alice is in the clinic.

- Similarly, Bob’s hologram data including associated gesture and pose is sent to the communication device in Alice’s home via the application server and displayed in naked eyes.

- Alice can receive feedback from Bob as if Bob is in the house.

- All data transmitted over wireless links is encrypted as well as integrity protected to ensure the security of the communication.

Scenario#2 Joint diagnostic (avatar-based)

- Alice, Bob and Tom setup a holographic conference via the mobile communication device. For the purpose of privacy, Tom chooses to use holographic avatar which is generated in the network.

- When Tom makes an examination and talks, the multi-modal data streams captured in the hospital and his holographic avatar data are transmitted to the selected edge servers of Alice and Bob for rendering, which may include the modal data of Tom’s pose, gesture, motion, or facial expression, the conversational voice, and the interactive information. As the examination goes on, the ty of modal data may change accordingly.

- Alice’s and Bob’s mobile devices send captured hologram related data to the edge servers chosen for other participants respectively.

- Tom’s animated holographic avatar is simultaneously generated in Alice’s and Bob’s edge server, and then is converged with hologram of Bob or Alice individually to render the whole scenario.

- The rendered virtual conference is transmitted to Alice’s and Bob’s mobile device for display.

### 9.7.4	Post-conditions

Alice and Bob can engage in real-time, bidirectional holographic communication as if they are in the same room, despite the geographical distance. This will be beneficial especially when they need close interaction during the consultation, for example rehabilitation.

Bob and Tom can make the joint diagnostics for Alice’s health as if they are in the same room. And Tom’s avatar can reflect his gesture, motion and facial expression correctly and synchronized.

### 9.7.5	Existing features partly or fully covering the use case functionality

The service requirements for avatar-based real-time communication have been specified in clause 5.2.2 of 3GPP TS 22.156 [28]. The high data rate requirements for 5G use cases are specified in clause 7.1 of TS 22.261 [14]. However, these are not requirements for 6G systems and are insufficient for realising 6G immersive media communication that utilises realistic 3D image.

### 9.7.6	Potential new requirements needed to support the use case

[PR 9.7.6-1] The 6G system shall support holographic communication with the following KPIs.

Table 9.7.6-1: Performance Requirements for Holographic Telepresence

| Use Case | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area |
| Holographic telepresence in Healthcare | Presence: [<100ms] Movement: [<20ms] (Note1） | 4K video resolution: [~ 400Mbps] (30fps, 8 parallax, 15 bit/pixel, H.265/H.266 with compression ratio 100) 8K video resolution: [~ 900Mbps] (30fps,8 parallax, RGB, compression ratio 200) Hologram/Point cloud [500Mbps-1Gbps] (30fps,8 parallax, RGB, AI-based) (Note2) | [99.9%-99.999%] | 1 | Stationary, Pedestrian | [5 m x 5 m] (Note 4) |
| NOTE 1: For real-time presence of hologram, refer to [9.7-x1]; for more interaction, the motion-to-photon delay will consider the effect of cyber sickness[9.7-x2] NOTE 2: For volumetric-based holography, the bandwidth is impacted by the effective pixel count, which is related to the resolution, colour quality and bit-depth [9.7-x4] the data rate is calculated based on [105] for uncompressed raw data, and optimized by different compression algorithms with different compression rate stated in [9.7-x3]. With the help of AI technologies such as Neural Holographic Video Compression (NHVC), the bandwidth for transmitting hologram will be optimized a lot [9.7-x5] NOTE 3: The size of a small room/office. |  |  |  |  |  |  |

Editor’s Note: The KPI value for holographic communication is FFS.

[PR 9.7.6-2]Subject to operator’s policy, the 6G system shall be able to dynamically determine QoS parameters for the media traffic in a conversational holographic communication to ensure deterministic QoE under varying network conditions.

Editor’s Note: The consideration of other requirements necessary for realizing the above use case is FFS.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[9.7-x1]	Xuemin S., Jie Gao,”Toward Immersive Communications in 6G”, Frontiers of Computer Science, Volume 4(2022), 11 January 2023

[9.7-x2]	Kougioumtzidis, G., Poulkov, V.,” QoE Assessment Aspects for Virtual Reality and Holographic Telepresence Applications”. In: Future Access Enablers for Ubiquitous and Intelligent Infrastructures (pp.171-180)

[9.7-x3]	A. Clemm, M. T. Vega, “Toward truly immersive holographic-type communication: Challenges and solutions,” IEEE Communications Magazine, vol. 58, no. 1, pp93-99, 2020.

[9.7-x4]	Xu, Xuewu & Pan, Yuechao & Lwin, Phyu & Liang, Xinan. (2011). 3D holographic display and its data transmission requirement. 10.1109/IPOC.2011.6122872.

[9.7-x5]	H. Ban, S. Choi, J. Y. Cha, Y. Kim and H. Y. Kim, "NHVC: Neural Holographic Video Compression with Scalable Architecture," 2024 IEEE Conference Virtual Reality and 3D User Interfaces (VR), Orlando, FL, USA, 2024, pp. 969-978, doi: 10.1109/VR58804.2024.00116

* * * End of Changes * * * *