3GPP TSG-SA WG1 Meeting #110 	S1-252517

Fukuoka, Japan, 19-23 May, 2025	(revision of S1-252258, S1-252580)

Title:	TR 22.870 pCR Use Case on enabling Non-3GPP Sensors

Agenda Item:	8.1.4 (FS_6G_REQ - Integrated Sensing and Communication)

Source:	Samsung

Contact:	Erik Guttman <erik.guttman@samsung.com>

Abstract: This contribution seeks to resume study activity on non-3GPP sensing use cases whose potential requirements were not accepted as normative as part of the sensing study in Release 19.

Discussion

This contribution seeks to resume study activity on non-3GPP sensing use cases whose potential requirements were not accepted as normative as part of the sensing study in Release 19.

Changes in S1-252517

- Reduce the scope of the use case to be entirely for ‘operator offered and controlled sensors’ [Ericsson, Huawei]

Changes in S1-252480

- Remove ‘control’ aspects of the use case to narrow the focus [Qualcomm, Huawei]

- Reduce the scenario to sensors offered by the operator, to help focus the use case clarity [Qualcomm]

- Simplify the text in description and requirements regarding authorization (requirement -3) [Xiaomi]

Proposal

It is proposed to make the change listed below to TR 22.870.

Begin changes

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Al Agent: an automated intelligent entity capable of e.g. interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, executing tasks (autonomously or in collaboration with other Al Agents) to achieve a specific goal.

Editor’s Note: This definition could be updated as the study goes on to align with the potential new understand of AI agent.

AI service: a 6G service (e.g., AI model inference) to support AI-related activities, which is provided by 6G core network to the 6G user, considering the required quality of the service (e.g., inference accuracy, latency).

NOTE 1:	The above term does not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN (compared to 5G).

Editor’s Note:	The definition of AI service is FFS.

Digital Twin: A real-time representation of physical assets in a digital world.

NOTE 2: This definition was taken from ITU-T Recommendation Y.3090 [113].

Donor Sensor: A sensor under certain conditions (e.g. region of interest) can offer non-3GPP sensing data which can be gathered and employed by third parties according to 3GPP standards.

Intelligent Communication Assistant: The virtual intelligent communication assistant located in operator network and interacts with the users through voice, video, text, gestures or other modalities. The assistant can be customized for each particular user by accessing user data stored in the network, with user’s consent. It can provide various communication services and support individual users based on user’s intention and requirement. One subscriber can have one or more Intelligent Communication Assistants.

Network Digital Twin: virtual replica of (part of) a mobile network to emulate (or simulate) the behaviour of the actual network.

Editor’s Note: it is FFS to update this definition.

Personal Data: any information relating to a user or subscriber that can be used to, either directly or indirectly, identify that user or subscriber, or to distinguish that user or subscriber from others.

Region of Interest: A three dimensional space for which sensing results are desired.

Sensing target density: total number of objects to be sensed per geographic area. It is a measure of how many objects the 3GPP system can detect, identify and/or track within a target sensing area.

Next Change

# 7	Integrated Sensing and Communication

Editor's Note: "Integrated Sensing and Communication" facilitates new applications and services that require sensing capabilities. It offers wide area multi-dimensional sensing that provides spatial information about unconnected objects as well as connected devices and their movements and surroundings.

## 7.X	Use Case X on enabling Non-3GPP Sensors

### 7.X.1	Description

This use case describes a new Teleservice (see [1]) for 6G, to enable discovery and acquisition of non-3GPP sensor data (e.g. from cameras, microphones, etc.) This use case does not consider non-3GPP wireless sensing, e.g. IEEE 802.11bf.

Non-3GPP sensors are deployed by MNOs. The MNOs offer their use, to acquire sensing data and/or sensing results, as a service.

In the Release 19 study of Integrated Sensing and Communication, use cases were added to TR 22.837 [9] to present the implications and requirements of non-3GPP sensing. These included use case 5.4 on Transparent Sensing and 5.19 on Sensor Groups. During consolidation of the agreed potential requirements, several aspects were not agreed to be included in the normative specification TS 22.137. [6] Both use cases included the following aspects:

-	Identifying (discovering) a set of relevant non-3GPP sensors, e.g. in proximity of the user or the user's UE

-	Accessing these sensors to acquire their sensor data

-	Processing the sensor data in the 3GPP system, combining sensing data, according to a set of standardized interfaces

-	Producing and exposing a sensing result, according to a set of standardized interfaces

There were two main considerations in the Release 19 study that led to the rejection of the associated functionality discussed above.

1.	It was asserted that 3GPP sensing was in scope of the Integrated Sensing and Communication study, and the functionality described above went beyond that, to consider non-3GPP sensing capabilities per se.

2.	The concept of the sensing group was deemed insufficiently precise, requiring further elaboration before consideration as a normative requirement.

In order to address these concerns, and also to investigate the utility of 3GPP service enablers to deploy and expose the results of non-3GPP sensors, a set of scenarios and associated use cases are elaborated.

The business case for this use case is charging for support of the following scenarios:

1)	A donor sensor is offered by a network operator. Customers (including third parties and subscribers) use this sensor as a service to collect non-3GPP sensing data. Example: Cameras positioned outdoors, in public spaces, or as a service in commercial spaces.

Other potential business cases, where the donor sensor is offered by a subscriber, third party or enterprise are not covered in this use case.

### 7.X.2	Pre-conditions

Actors in this use case include the

-	Donor Sensor Owner (DS-Owner): A deployed Donor Sensor has an owner. The owner determines who can access the sensor to obtain non-3GPP sensing data from the sensor.  The DS-Owner is the MNO in this use case.

-	Network Operator (NOP): A network operator (fixed or mobile), that enables authorized use of Donor Sensors, as a service.

NOTE:	The term Network Operator instead of Mobile Network Operator is used intentionally, as it is anticipated that this service could operate in fixed network scenarios as well as mobile networks.

-	Sensing Data Consumer (SDC): The Non-3GPP sensing data consumer, modelled as an Application Server (AS), obtains data and processes it as part of a service it provides to an application.

-	Sensing Result Consumer (SRC): The application provides some service to a User, for example XR media or high level information calculated from the sensing data and presented in any form, e.g. a diagram showing status information of a remote site.

Figure 7.X.2-1: Actors in Non-3GPP Sensing Service Enablers Use Case

Donor Sensor

DS-Owner

6G System

NOP

SDC

SRC

AS

Application

Sensor data

Sensor data

Sensing result

(e.g. XR Media)

Actions (see 7.X.3)

Determines use

Enables use

Uses sensor data

(in)directly initiates use, service

The Donor Sensor can be considered the terminal equipment of a UE, though in practice a single UE could support communication to multiple Donor Sensors.

This UE has a subscription with the network operator NOP. For simplicity, assume that the DS-Owner is the subscriber.

The Sensing Data Consumer (SDC) is considered in the service flows below to be a third party service provider, though it is not excluded that the SDC is operated by NOP.

The Sensing Result Consumer (SRC) is considered in the service flows below to be another third party, an application service user. It is not excluded that the SRC is a 3GPP subscriber, or even the DS-Owner, but this is not assumed.

It is not excluded that the SDC and SRC are one entity, e.g. an application deployed on a User’s UE that can directly consume Non-3GPP Sensing Data and produce Sensing Results.

NOTE:	In this use case, the Donor Sensor is offered by a network operator. Other scenarios are possible, where the Donor Sensor is offered by a mobile subscriber, a third party or an enterprise customer of a mobile network operator.

### 7.X.3	Service Flows

The User, making use of the services provided by the SRC effectively needs non-3GPP sensing data in a specific Region of Interest. In order to satisfy the User, a number of service flows are defined. (a) for the DS-Owner to establish available Donor Sensors, (b) for the SDC to discover Donor Sensors and get authorization to use them, (c) for the SDC to acquire sensing data from a Donor Sensor, (d) for the SDC to expose Sensor Results to an SRC.

a) provisioning, reconfiguration, deprovisioning

-	provisioning: The DS-Owner deploys a new Donor Sensor. The NOP is informed of the existence and configuration of the Donor Sensor. Configuration includes at least the localization (position and orientation) of the Donor Sensor, the target location (the physical space that can be sensed), the media type, and the access authorization restrictions.

-	reconfiguration: The DS-Owner can at any time modify the configuration of a provisioned Donor Sensor. The NOP is informed of the changed configuration.

-	deprovisioning: the DS-Owner decommissions a Donor Sensor. The NOP is informed of the Donor Sensor no longer being provisioned.

b) discovery and authorization

-	The SDC determines a need for sensing in a particular target location, e.g. because sensing results are needed by a specific SRC. This is the Region of Interest. The SDC requests Donor Sensors within this Region of Interest from the NOP.

-	The NOP identifies if the SDC is authorized to use any deployed Donor Sensors in the region of interest. If this is possible, the Donor Sensor configuration is provided to the SDC.

-	The SDC can request the NOP to use one or more Donor Sensors.

-	The NOP can authorize the SDC to use one or more Donor Sensors.

-	The NOP can at any time remove authorization of the SDC to use one or more Donor Sensors.

-	As the configuration of Donor Sensors can change (see (a), above), the SDC can be kept up to date when changes occur.

c) acquisition

- The SDC, authorized to use a Donor Sensor, can acquire sensing data. Depending on the Donor Sensor, this data may be ‘continuous’ (e.g. video, audio, etc.) or ‘discontinuous’ (e.g. captured images, captured data from a LiDAR pulse, etc.)

- If the data acquisition is continuous, the SDC (based on the Donor Sensor configuration) can adjust the acquisition (e.g. resolution, format, etc.) If the data acquisition is discontinuous, the SDC (based on the configuration) can adjust the frequency, etc.

- The 6G system supports a means by which this sensing data is treated efficiently and with appropriate communication resources (e.g. QoS policy).

-	The SDC, authorized to use a Donor Sensor, can cease acquiring sensing data at any time.

d) sensor result service enablement

Though the service experienced by the User is supported by the Non-3GPP Sensing Service, it is indirect. The service provided by the SDC to the SRC is not in 3GPP scope. It is proposed that the 3GPP standard could specify particular information to exchange between the SRC and SDC, such as known by the User (or the application that the User employs) and essential for the SDC to acquire and process results as expected.

-	The User employs some software (e.g. an application, accesses a resource via HTTP, etc.) that requires sensing results from a particular Region of Interest. The entity that requires the sensing result (e.g. XR media) is the SRC. The precise service interaction between the SRC and SDC is out of scope of 3GPP (occurs over ‘any network.’)

-	To enable the interaction between the SDC and the SRC, information can be exchanged between the SRC and SDC that conforms to the 3GPP standard (at the application layer) to identify such things as are identified at stage 2 technical consideration such as the Region of Interest, time when authorization is required, the identity of the User, as deemed relevant.

### 7.X.4	Post-conditions

It is possible for a user to consume sensing results as part of their application experience due to the new Non-3GPP Sensing Service employed by the SDC, to acquire data to be used to generate Sensing Results (e.g. XR media.) In this way, media acquired from a Region of Interest is therefore timely and completely suited to the service consumed by the User.

The Network Operator can charge for provisioning of Donor Sensors by DS-Owners. The Network Operator can also   charge the SDC for their authorized use of Donor Sensors: discovery, and fully supported communication and other services for data acquisition. The 3GPP standard also enables interaction between SRCs and SDCs to provide sufficient information to enable the SDC to discover and employ the appropriate (needed) Donor Sensors.

### 7.X.5	Existing features partly or fully covering the use case functionality

It is possible in 5G to support Sensor data acquisition over the top, where all aspects of discovery, configuration, authorization and sensor data communication are opaque to the 5G system.

Localization of the user and mapping is supported in Rel-19. This service extends the concept of localization to an arbitrary sensor (e.g. camera) that is not necessarily in the position and orientation of any particular User or UE.

### 7.X.6	Potential New Requirements needed to support the use case

[PR 7.X.6-1] Subject to operator policy, regulatory requirements and user consent, the 6G system shall support a means by which Donor Sensors can be provisioned, configured and deprovisioned by an authorized third party.

NOTE 0:	In this use case, the DS-Owner is the MNO. Authorization for provisioning by third parties could support third parties who would deploy and maintain sensor equipment on behalf of an MNO.

NOTE 1:	A provisioned Donor sensor is associated with configuration information by the authorized third party such as the Donor Sensor localization (position and orientation) of the Donor Sensor, the target location (the physical space that can be sensed), the media type of the sensing data obtained, and the access authorization restrictions (e.g. who can use or cannot use the Donor Sensor, when, etc.)

[PR 7.X.6-2] Subject to operator policy, regulatory requirements and user consent, the 6G system shall support a means by which authorized third parties can discover Donor Sensors based on their needs (e.g. Region of Interest, time when sensing data is needed, type of sensing data needed, etc.)

[PR 7.X.6-3] Subject to operator policy, regulatory requirements and user consent, the 6G system shall support a means by which authorized third parties can use one or more Donor Sensors.

NOTE 2:	It is possible that authorization to use a Donor Sensor can be removed at any time.

[PR 7.X.6-4] Subject to operator policy, regulatory requirements and user consent, the 6G system shall support a means by which authorized third parties can be provided with up-to-date information corresponding to their use of the Donor Sensor (e.g. Region of Interest, time when sensing data is needed, type of sensing data needed, etc.)

NOTE 3:	As information associated with a Donor Sensor can change at any time, the authorized third party needs to be made aware of the current information, over time.

[PR 7.X.6-5] Subject to operator policy, regulatory requirements and user consent, the 6G system shall support a means by which the sensor data offered by a Donor Sensor can be acquired by an authorized third party, according to suitable communication service policies (e.g. QoS policy) and mechanisms (e.g. media services to improve robustness and efficiency of uplink media transmission.)

[PR 7.X.6-6] Subject to operator policy, regulatory requirements and user consent, the 6G system shall support a means by which an authorized third party can acquire sensing data according to their needs, where the Donor Sensor supports different capabilities (e.g. data acquisition frequency, data resolution, etc.)

[PR 7.X.6-7] Subject to operator policy, regulatory requirements and user consent, the 6G system shall support application enablement mechanisms providing a standardized means to express requirements associated with sensing results needed, such as the time, Region of Interest associated with a service.

[PR 7.X.6-8] Subject to operator policy, the 6G system shall support charging for provisioning, configuration and deprovisioning Donor Sensors.

[PR 7.X.6-9] Subject to operator policy, the 6G system shall support charging for discovery, authorization and acquiring data from Donor Sensors.

End of changes