3GPP TSG-SA WG1 Meeting #110 	S1-252554

Fukuoka, Japan, 19-23 May 2025	(revision of S1-252351)

Source:	Nokia, Huawei, CATT

pCR Title:	Pseudo-CR on updates of use case 8.9

Draft Spec:	3GPP TR 22.870-021

Agenda item:	8.1.5

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 8.9 in FS_6G-REQ TR.

1. Introduction

This contribution proposes to change the pre-condition about UE registration, clarify the purpose of verification the non-3GPP positioning technologies and add an EN to KPIs for clarification.

2. Reason for Change

UE registration to TN is crucial for maintaining robust, low-latency and accurate positioning services in hybrid TN and NTN positioning. The purpose of the PR is unclear whether to verify the non-3GPP positioning results or improving positioning performance. The KPIs need to consider the challenges posed by long propogation delay in NTN, significant Doppler shifts for high speed UEs, and precise synchronization etc.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870-021.

* * * First Change * * * *

## 8.9.	Use case on hybrid TN and NTN positioning

### 8.9.1	Description

High-accuracy positioning with high availability is a key feature to enable increased efficiency and automation of operations, such as in mapping and transportation. This is typically achieved thanks to the use of GNSS with precise corrections, which can be provided by 3GPP networks through assistance data. Although GNSS and commercial corrections systems offer reliable services, their positioning solutions can still be affected by different sources of error, such as multipath or interferences. Thus, relative positioning solutions, such as sensors or vision-based navigation, are typically coupled with precise GNSS to enhance the positioning robustness. However, there are very limited mechanisms to verify the absolute positioning accuracy and availability of these precise GNSS solutions, especially when relative positioning solutions are not available or operational.

Dedicated 3GPP terrestrial networks (TN) can be deployed to enable high-accuracy positioning with high availability in enhanced service areas, while ensuring low latency. Nonetheless, the increased cost expenses of such dedicated deployments are not expected in commercial networks. Furthermore, the use of commercial TN deployments may not be sufficient to provide complementary 3GPP positioning for ground users, due to the expected blockage of the line-of-sight (LoS) from distant base stations, i.e., only one or two base stations may be available in LoS conditions. In case of aerial users, the commercial TN deployment may be sufficient to provide 3GPP horizontal positioning, however the similar altitude of the terrestrial base stations results in a poor accuracy in the height estimation. For both ground and aerial users, the deployment of 3GPP non-terrestrial networks (NTN) can then increase the number of available LoS links and the vertical geometrical diversity, with respect to only using 3GPP TN positioning.

Figure 8.9.1-1: Hybrid TN and NTN positioning with 3GPP technologies

Hybrid TN and NTN positioning includes the combination of TN and NTN to support 3GPP positioning technologies. Thus, hybrid TN and NTN positioning with 3GPP technologies can enable high-accuracy positioning with high availability and fast convergence time, which could be used as a verification mechanism of non-3GPP positioning technologies, such as GNSS with precise corrections. Considering GNSS state-of-the-art solutions, the verification mechanism is expected to operate with update rates between 2 Hz and 10 Hz and an availability above 99%.

One example application is the positioning verification of airport surface operations, where the aircraft on ground is required to meet safety navigation requirements in the taxiway. The aircraft follows the planned path based on the on-board navigation systems and visual operations, while the navigation requirements depend on the taxiway width of the airport, including the rapid exit phase with a ground aircraft speed up to 100 km/h. Low-visibility conditions can severely impair these aircraft taxi operations.

Another example application is the positioning verification of UAVs for facility monitoring. The UAVs fly over the installations for inspection (e.g. to capture photos and videos) or for surveillance, where the UAVs can achieve speeds up to 160 km/h.

### 8.9.2	Pre-conditions

The UE has terrestrial and non-terrestrial network coverage in the 6G system.

The UE is registered to the terrestrial network.

The UE can be under the coverage of multiple or multi-orbit satellite networks (e.g. LEO and GEO constellations) deployed by one or more operators and has subscription(s) for accessing these satellite networks.

The environment of use is outdoor static or moving with coverage provided by the terrestrial and non-terrestrial networks.

The UE is equipped with a native 6G communication and positioning function. The positioning function relies on 3GPP technologies delivered by the satellite network, to be able to verify non-3GPP positioning results.

### 8.9.3	Service Flows

- The 3GPP positioning service provided by the terrestrial and non-terrestrial networks can be initiated by a third party or by the end-user.

- This 3GPP positioning service can determine the UE location with a certain accuracy level.

- When triggering the 3GPP positioning service, if the UE fails to obtain the location data within a time limit or the location data is not accurate enough, the UE can select different terrestrial or non-terrestrial networks or improve the accuracy of location data by combining multiple networks.

- The UE location is used to verify the non-3GPP positioning results.

- The UE can report its UE location and positioning accuracy level.

### 8.9.4	Post-conditions

The user successfully obtains the 3GPP positioning service and can verify the non-3GPP positioning results.

### 8.9.5	Existing features partly or fully covering the use case functionality

TS 22.261 [14] clause 6.27 includes positioning and performance requirements for the 3GPP and non-3GPP positioning technologies, including the following requirement related to 3GPP terrestrial coverage:

The 5G system shall support mechanisms to determine the UE's position-related data for period when the UE is outside the coverage of 3GPP RAT-dependent positioning technologies but within the 5G positioning service area (e.g. within the coverage of satellite access).

However, this requirement does not ensure the combination of 3GPP terrestrial and non-terrestrial positioning technologies, which could be used to achieve high-accuracy positioning with high availability.

TS 22.261 [14] clause 7.3.2.2 also includes requirements for horizontal and vertical positioning service levels:

The 5G system shall be able to provide positioning services with the performances requirements reported in Table 7.3.2.2-1.

### 8.9.6	Potential New Requirements needed to support the use case

[PR 8.9.6-1]	The 6G system with terrestrial and satellite access shall be able to provide high-accuracy positioning service with high availability, by using 3GPP positioning technologies to verify the non-3GPP positioning results and fulfilling the following KPIs:

Table 8.9.6-1: Performance requirements for hybrid terrestrial and satellite positioning services

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| Airplane taxiway (NOTE 1) | [1] m | N/A | [99 %] | [0.1 - 0.5] s | Outdoor | Up to [100] km/h | Airplane mounted |
| UAV facility monitoring | [3] m | [3] m | [99 %] | [0.1 - 0.5] s | Outdoor | Up to [160] km/h | UAV mounted |
| NOTE 1: Airplane taxiway refers to an airplane on ground during taxi operations. |  |  |  |  |  |  |  |

Editor's Note:	 it is FFS whether KPIs are needed to verify non-3GPP positioning results, and if needed, how the accuracy requirements are justified in relation to the latency and UE speed.