3GPP TSG-SA WG1 Meeting #110 	S1-252559

Fukuoka, Japan, 19-23 May 2025	(revision of S1-252352)

Source:	Nokia, Huawei, CATT

pCR Title:	Pseudo-CR on updates of use case 8.10

Draft Spec:	3GPP TR 22.870-021

Agenda item:	8.1.5

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 8.10 in FS_6G-REQ TR.

1. Introduction

This contribution proposes to clarify the hybrid NTN and GNSS positioning in pre-condition and PR#1 and Network-based Assisted GNSS functions for better positioning performance in PR#2.

2. Reason for Change

To make clear distinction between use case 8.9 and 8.10 what the hybrid positioning is about and for which purpose, also considering that the hybrid TN and GNSS positioning is already supported.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870-021.

* * * First Change * * * *

## 8.10	Use case on hybrid NTN and GNSS positioning

### 8.10.1	Description

The ubiquitous availability of position knowledge is paramount to a plethora of use cases and services. GNSS fulfils most of these positioning needs. GNSS systems are typically designed with a global depth of coverage (DOC) of at least 4 satellites in open-sky conditions to enable standalone use of each constellation. Interoperability and standardisation of GNSS systems on signal structure, power levels, navigation message design and broadcasting relative time-offset has enabled modern smartphones to rely on Galileo, GPS, Beidou and Glonass increasing the number of visible satellites.

Yet, GNSS users can face availability challenges when buildings or natural objects obstruct the line-of-sight (LOS) view of satellites. The increased occurrence of jamming and spoofing event constitutes an additional challenge for continuity and availability of positioning for users. For cold-start standalone GNSS, users may take tens of second or more to obtain a first position fix.

Hybrid operation of NTN and GNSS positioning may enable better accuracies especially in challenging environments thanks to an increased total number of satellites in view. Hybrid operation could further offer improved continuity of satellite-based positioning through frequency diversity of NTN bands and GNSS L-band signals, especially in jamming and spoofing events impeding L-band. Lastly, NTN satellites have the potential of reducing time to first fix (TTTF), especially in cold-start scenarios by providing a fast data channel for satellite clock and ephemeris data (CED).

### 8.10.2	Pre-conditions

The UE has satellite network coverage in the 6G system.

The UE has access to GNSS signals (non-3GPP technology).

The UE can be unregistered or registered (e.g. in idle or connected mode) to the satellite network.

The UE can be under the coverage of multiple or multi-orbit satellite networks (e.g. LEO and GEO constellations) deployed by one or more operators and has subscription(s) for accessing these satellite networks.

The environment of use is outdoor static or moving with coverage provided by satellite network(s) and GNSS. The outdoor environment may have partly obstructed satellite visibility.

The UE is equipped with a native 6G communication and positioning function. The positioning function relies on 3GPP technologies delivered by the satellite network.

### 8.10.3	Service Flows

1. The 3GPP positioning service provided by the satellite network can be initiated by a third party or by the end-user.

2. This 3GPP positioning service with support from GNSS can determine the UE location with a certain accuracy level.

3. When triggering the 3GPP positioning service, if the UE fails to obtain the location data within a time limit or the location data is not accurate enough, the UE can select different satellite networks or improve the accuracy of location data by combining multiple networks.

4. The UE location is provided by the hybrid NTN and GNSS positioning service.

5. The UE can report its UE location and positioning accuracy level.

### 8.10.4	Post-conditions

The user successfully obtains its position based on 3GPP NTN positioning service and GNSS.

### 8.10.5	Existing features partly or fully covering the use case functionality

TS 22.261 [14] clause 6.27 includes positioning requirements for the 3GPP and non-3GPP positioning technologies. In addition, TS 22.261 [14] clause 7.3.2.2 includes performance requirements for horizontal and vertical positioning service levels for the 3GPP system:

The 5G system shall be able to provide positioning services with the performances requirements reported in Table 7.3.2.2-1.

NOTE: 	The requirements do not preclude any type of UE, including specific UE such as for example V2X, MTC.

Table 8.10.5-1 Performance requirements for Horizontal and Vertical positioning service levels (Table 7.3.2.2-1 from [14])

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99,9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0,3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0,3 m | 2 m | 99,9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0,2 m | 0,2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each other (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |

TS 22.261 [14] clause 7.3.2.1 does require in general the combination of 3GPP and non-3GPP positioning to achieve superior performance than the 3GPP system standalone:

The 5G system shall support the combination of 3GPP and non-3GPP positioning technologies to achieve performances of the 5G positioning services better than those achieved using only 3GPP positioning technologies.

However, no performance targets are provided with satellite access. Furthermore, no specific mention is made of hybrid NTN and GNSS positioning where signals used for ranging emanate from satellites in both systems.

TS 22.261 [14] clause 6.27.2 notes that hybrid positioning includes a combination of 3GPP and non-3GPP positioning methods, for example Network-based Assisted GNSS. However, no mention is made of provision of Network-based Assisted GNSS by means of NTN.

While not further specified in [14], Network-based Assisted GNSS is understood as the 5G system providing data that assists the UE in obtaining a GNSS position. This data can include for example GNSS navigation message data required for the UE to compute its position as is supported in LTE positioning protocol (LPP) for terrestrial networks.

TS 38.171 [100] specifies minimum expected GNSS performance. For outdoor environment, the performance of combined NTN and GNSS is expected to be superior to the minimum GNSS performance as specified in TS 38.171 [100], clauses 5 and 6.

### 8.10.6	Potential New Requirements needed to support the use case

[PR 8.10.6-1]	The 6G system with satellite access shall be able to provide a positioning service supported by hybrid NTN and GNSS positioning technologies.

Editor’s note: The KPIs are FFS.

[PR 8.10.6-2]	The 6G system with satellite access shall be able to support Network-based Assisted GNSS functions to improve positioning.