3GPP TSG SA WG 1 Meeting #110 	S1-252584

Fukuoka, Japan, 19 - 23 May 2025

(revision of S1-252571) 	(revision of S1-252552)	(revision of S1-252101)

Title:	Use case on Disaster relief

Agenda item:	8.1.5

Source:	Orange

Contact:	Philippe Lottin (philippe.lottin (@) orange.com)

Abstract: This contribution provides an update of the use case on Disaster relief taking into account the results of the deliverable D1.4 “6G Value, Requirements and Ecosystem” from the Hexa-X-II project.

1. Introduction

The use case on Disaster relief in TR 22.870 corresponds to scenario S3 Connectivity at Remote Locations developed in the Ubiquitous and Resilient Network | Representative Use Case in the Hexa-X-II project deliverable.

This contribution provides an update on the use case for Disaster relief in the Technical Report TR 22.870, taking into account the results of the Hexa-X-II project.

2. Reason for Change

The values in the key performance indicator table was consolidated in the Hexa-X-II project. It is proposed to add a key performance indicator table in the 3GPP TR 22.870 standard accordingly.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

### 8.5.6	Potential New Requirements needed to support the use case

[PR 8.5.6-1]: The 6G system using satellite access, shall be able to provide eMBB service to UE mounted on SWAP constrained moving platforms (e.g. flying drone, small vessels mounted).

[PR 8.5.6-2]: The 6G system shall be able to support the following KPIs:

Table 8.1.6-1: Disaster relief Network KPIs [112]

| End-to-end latency [ms] | Availability [%] | Reliability [%] | User experienced data rate [Mb/s] | Connection density [devices/m2] | Coverage [%] |
| --- | --- | --- | --- | --- | --- |
| [10-500] | [99.999] | [99.9 –99.999] | [0.1 –25] Downlink / [2] Uplink | [0,1] | [99.9] |

| Mobility [km/h] | Location accuracy [m] | Positioning availability [%] | Positioning latency [s] |
| --- | --- | --- | --- |
| up to [120] | Low (≈ [10]) | [99] | [1] |