3GPP TSG SA WG 1 Meeting #110 	S1-252764

19-23 May 2025, Fukuoka, Japan	(revision of S1-252217)

Source:		OPPO

Title:	Updating use case of 6G system assisted target object detection

Agenda item:	8.x.x

Document for:	agreement

Contact:	Yang Xu (xuyang@oppo.com)

Abstract: This contribution updates the use case of 6G system assisted target object detection.

********** Start of Change *********

6.14	6G system assisted target object detection

6.14.1	Description

Figure 6.14.1-1 6G system assisted data processing for a task

Aside from 6G system providing a data transmission (3GPP communication service), on top of the connectivity, some servers in the 6G CN or operator managed DN (e.g. edge compute domain) may be connected to provide data processing using its local computation resource. As the figure above shown, UE and one or more nodes in 6G CN, operator managed DN (e.g. edge compute domain) and Application server are involved. The 6G network can gurantee the overall e2e latency considering the apportionment of communication latency as well as the processing latency.

NOTE: The 6G system may adjust the resource of  the processing server in the 6G CN or in the operator managed DN (e.g. edge compute domain) considering the transmission latency to satisfy the needed the overall joint latency (communication latency plus processing latency).

This contribution introduces a use case where the data processing is performed by 6G system for a target object detection task,

6.14.2	Pre-conditions

Figure 6.14.2-1 6G system assisted target object detection

Vehicle:

support LiDAR sensor to sense the target car. It generate data in in point-cloud format and transmit the data to RAN node;

RAN node:

support RF sensor to sense the target car. It generates the measurement data in point-cloud format and send the data to core network.

NOTE: The RAN node has a overview information about the target and the vehicles surrounding the target car can get the detailed data about a specific direction to the target, which helps increase the inference accuracy.

Core network:

The core network is installed with aggregation model, which is used to process the input data by aggregating thet data from UEs and RAN. Then transmit the aggregated data to the Edge server supporting inference model for Car positioning.

Edge Server:

Support the inference model. Perform the inference result of the shape and position of the target object based on the aggregated data from core network and the video data from the infrastructure camera.

6.14.3	Service Flows

0) the aggregation model and inference model has been offline trained in cloud server and provisioned into CN node and edge computing server in 6G system.

1) When a car collision happens, the 6G system selects three vehicles which are surrounding the target object (i.e. the collided cars) and capable of doing LiDAR sensing service [89]. As example, the selection may consider one or more of the aspects: capabilities and computation resource of UEs and/or networks, the network congestion status, and the certain task request (e.g. the accuracy of a sensing request). Then, the vehicles generate the LiDAR sensing data meanwhile the RAN node generates the RF sensing data, both the vehicles and RAN node transmit the data in point-cloud format. The serving core network node (e.g. connected behind the user-plane function) generates the output (aggregated data) using aggregated model based on the measurement input from vehicles and RAN node.

2) The edge computing server received the video data sent by the infrastructure camera and received the aggregated data from CN node. The edge server then do the inference using the inference model to derive the accurate shape and position of the target object.

3) The edge server generates the output i.e. inference result, which could be updated into HD map for auto-driving service. The edge server may send the updated HD map to all vehicles so that the vehicle can perform the correct driving decision based on the updated HD map [90].

6.14.4	Post-conditions

Thanks to the in-time detection of the target object, it can quickly inference the accurate shape and position of the collided cars so that the HD map can be updated in time.

6.14.5	Existing features partly or fully covering the use case functionality

TS 22.261 [14] Clauses 6.40 and 7.10 describe the communication functionality and KPIs for AIML model transfer in 5GS (including UE member seleltions for FL). Instead of assuming to be performed as a mere transmission pipeline, the 6G system may get involved for an AIML operation by providing computation resources, data, model and enhanced connections.

The exisiting requirements for AMMT are assuming the AI application is from 3rd aparty and the data processing is done in application. However, this new use case is to addres MNO owned service such as sensing in which the data is generated within 3GPP system, thus it needs UE and 3GPP network (i.e. Core Network) to get involved of the data processing.

Thus, it requires more functionality and KPIs for 6G network to perform AIML operation as a native AI service.

6.14.6	Potential New Requirements needed to support the use case

Functionality Requirements:

[PR 6.14.6-1] Subject to operator policy, 6G network shall be able to select network entities (i.e. in 6G core network, or Service Hosting Environment) for a computing task.

[PR 6.14.6-2] Subject to user consent and operator policy, 6G network shall be able to guarantee an overall E2E latency for a computing task.

NOTE 1: the overall E2E latency includes the time for data transmission and data processing within 6G system.

NOTE 2: the above term does not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN (compared to 5G).

*******************End of Change*****************