3GPP TSG SA WG 1 Meeting #110 	S1-252882

SA1#110, 19-23 May, Fukuoka, Japan 	(revision S1-252799 S1-252312)

Source:	Ericsson, China Unicom

New use case title:	New use case on AI for disability support

Draft TS/TR:	3GPP TR 22.870v0.2.1

Agenda item:	8.1.3 AI

Document for:	Agreement

Contact:	Daniel Lönnblad (Daniel.lonnblad at ericsson.com)

Abstract: This is a use case that was presented in SA1#109. The use case “AI for disability support” focused on disability support that can be implemented using AI proposed for chapter 6 in TR 22.870

# 6	AI

## 6.x	Use case on AI for disability support

### 6.x.1	Description

As AI evolves, it will further improve human communication, enabling proactive communication with people, devices, and surroundings. This includes hearing aids with complete noise cancellation and the ability to tune in on specific sounds, visual aids enabling recognition beyond normal vision, and communication aids using eye movements or brain interfaces. This can have a revolutionary enabling effect for people with various disabilities and people in challenging situations where typical human capabilities are insufficient, allowing them to recuperate some of the impaired capabilities through the aid of AI-powered tools.

AI enhanced communication can be about person-to-person communication, for example a phone/video call, or person-to-AI function communication, where the AI function/agent assists the person in orientation, understanding, and information processing.

Key value impact analysis

Use of AI for disability support is valuable and has benefits within health & human wellbeing area. However, challenges regarding trustworthiness and energy needs to be considered

Energy resources

From the environmental perspective, energy-efficient AI models and increased reliance on renewable energy sources could help minimize the risks. Sources of electricity for AI devices and operations is an important factor as it could impact emissions to air, water and soils.

Trustworthiness

Ethical challenges include determining responsibility - for instance, assessing the impact and entity accountable in the event of technology malfunctions or if developers or providers decide to discontinue support. Determining the appropriate level of control over current and future technology functionality, given the technology’s intimate nature. Ensuring the technology’s value alignment, particularly concerning privacy and integrity as the technology is likely to collect, analyze, and store highly sensitive user data. Other issues in terms of value alignment are mitigating bias, ensuring that the technology works for everyone, and identifying the beneficiaries of the technology (e.g., those with the greatest need versus those with the greatest ability to pay). In evaluating trustworthiness, several aspects must be considered. These include the functionality, reliability, and benevolence (friendliness) of the technology itself, as well as external factors such as the trustworthiness of developers and providers, business models, data management practices and adherence to policy and regulations

### 6.x.2	Pre-conditions

The following pre-conditions are considered:

- A user has impaired vision, hearing, mobility, or orientation capacity, due to either disabilities or environmental factors that affect bodily functions.

- The user is equipped with a device capable of sending video and audio streams, such as smart glasses with camera and microphone. The device is also capable of receiving audio streams and optionally video streams.

### 6.x.3	Service Flows

The following service flow is considered, involving an application that can be part of the network (e.g. as an IMS application) or external:

- Immersive video (RGB and depth) and audio stream is captured on user device.

- In an application, the video and audio streams are analysed for user input (cues, intents)

  - User gesturing, audio cues, eye tracking, and field-of-view tracking (what the user is looking at).

- In an application, the video stream is further analyzed, and situational awareness is established.

- Based on audio, data input, or visual cues from the user, the application adds contextual information to the visual and/or audio stream or modifies the incoming audio stream. This may include:

  - Scene understanding: answering questions “What am I looking at?”, “Explain what is happening”. Adding digital tags in field-of-view.

  - Orientation: answering questions “Where am I?”, “How do I go to X?”. Adding digital arrows in field of view.

  - Visual aid: text in field-of-view read out in audio.

  - Audio aid: amplification, enhancing contrast, simplification or translation of audio stream.

  - Communication aid: assistance in communication with others, e.g. through translating user intents (gestures, eye movements, field-of-view tracking, etc.) to audio stream.

  - Reminders: assistance in remembering appointments etc., e.g. through audio or digital messages in field-of-view.

- A virtual overlay of contextual and modified information, in the form of video and/or audio streams, is transmitted to user device.

### 6.x.4	Post-conditions

Users get orientation, scene understanding, and adaptive communication support from the application through their devices and can thereby become more independent and capable.

### 6.x.5	Existing features partly or fully covering the use case functionality

Some forms of assistance can be implemented as an over-the-top service, on top of the existing mobile broadband connections. However, in the next subsection this contribution analyses the requirements for arrangements where the network involvement provides an enhanced service.

### 6.x.6	Potential New Requirements needed to support the use case

The following requirement for AI for Disability support are derived from the use case above.

[PR 6.x.6-1] Based on operator policy and user consent, the 6G system should enable the IMS services enhanced by AI capability, to enhance the audio and video stream for disability support in order for a disabled user to access IMS services.

NOTE: The disability support can be e.g., describing and explaining the surroundings, audio aid (modification of the audio stream i.e., enhancements, clarifications, translations, speech to text, text to speech, etc), visual aid (modification of the visual stream).

[PR 6.x.6-2] The following KPIs in Table 6.x.6-1 should be supported, in Dense Urban, Urban, and Rural indoor/outdoor scenario:

Table 6.x.6-1: Performance requirements

| Profile | Characteristic parameter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate down link | Bit rate up link | Application round trip latency | Communication service availability | Overall user density | Activity factor | UE speed |
| AI for Disability support | [1-2] Mbit/s | [50-100] Mbit/s Note 1 | [300-1000 ]ms Note 2 | Dense Urban [99.9]% Urban [99.9]% Rural [99]% | Dense Urban [2 500 ]/ km2 Urban [1 000] / km2 Rural [10] / km2 Note 3 | [50]% | 5km/h |
| Note 1: envisioned 4K resolution Note 2: Input stream (video, audio, data) to output stream (video, audio, data) 300-1000 ms dependent on task including the disability application Note 3: It is assumed that 10% of all users are using AI for Disability support |  |  |  |  |  |  |  |