3GPP TSG-SA WG1 Meeting #111	S1-253049

# Stor-Göteborg, Sweden, 25 – 29 August 2025

Source:	SK Telecom, LG Uplus, Verizon, Huawei, Hisilicon, China Unicom, China Telecom, vivo

Title:	Discussion on access category for Unattended Data Traffic

Agenda item:	New Work Items

Document for:	DISCUSSION

# 1	Introduction

We have submitted a CR [1] for TS 22.261 Table 6.22.2.3-1.

In this discussion paper, we provide the concerns on an excessive number of access attempts from Unattended Data Traffic in a congestion scenario and investigate the limitations of current UAC framework. Since the existing list of access categories do not cover the access attempts from Unattended Data Traffic, we propose the introduction of a new access category for Unattended Data Traffic.

# 2	Discussion

## 2.1	Concerns on Unattended Data Traffic in a congestion scenario

Unattended Data Traffic refers to data traffic of which the user is unaware he/she initiated, such as when the screen is locked or there has been no user input for a certain period [2]. One example is photo synchronization traffic, which is occasionally transmitted without the user’s awareness. Such traffic can be considered delay-tolerant, as its delayed transmission does not affect user QoE. Also, it is noticeable that a similar concept, Background traffic category, is also defined by GSMA [3] and has been specified as one of Connection Capabilities [4], [5] in 3GPP.

TS 22.101[2] Unattended Data Traffic: Data traffic of which the user is unaware he/she initiated, e.g. based on the screen/keypad lock being activated, length of time since the UE last received any input from the user, known type of app (e.g. an application monitoring a user's health – "mHealth" – may need its data never treated as Unattended Data Traffic.) Attended Data Traffic: Data traffic of which the user is aware he/she initiated, e.g. based on the screen/keypad lock being deactivated, length of time since the UE last received any input from the user, known type of application (e.g. an application monitoring a user's health – "mHealth" – which may need its data always treated as Attended Data Traffic.)

GSMA NG.141[3] Traffic Category; Description. Background; Any traffic that is not time-sensitive, e.g., firmware/ software updates over the air. This traffic has no critical requirements from latency or data rates perspective. This traffic should/can be subject of scheduling (e.g., at specific time of day) by the applications/networks..

While such traffic is generally not problematic under normal conditions, it can cause issues in congestion scenarios. In congestion scenarios such as subway stations during rush hour, it can be easily captured that excessive access attempts lead to lower RACH success rate (Figure 1). Also, when we investigated the types of access attempts, assuming that the call from Unattended Data Traffic shows a specific RRC pattern (e.g., a short connection duration after a relatively long idle duration), a significant portion of access attempts was found in live networks to originate from Unattended Data Traffic—that is, from UEs transmitting data without any active user interaction (Table 1). The large portion of these access attempts might be fine in idle hour (e.g., 4AM), since there are available RRC capability for the user-initiated traffic. In busy hour (e.g., 6PM), however, even a small portion of these access attempts from Unattended Data Traffic would degrade the overall service availability for user-initiated traffic.

[Figure 1. The variants of RRC connection success rate observed at the live network]

[Table 1. The comparison of the ratio of RRC connection attempts between background traffic and user-initiated traffic]

| Time | 5G/LTE | Ratio of RRC Connection Attempt |  |
| --- | --- | --- | --- |
|  |  | Unattended Data Traffic | User-initiated traffic |
| Idle Hour (4 AM) | 5G | 56% | 44% |
|  | LTE | 63% | 37% |
| Busy Hour (6 PM) | 5G | 28% | 72% |
|  | LTE | 37% | 63% |

- Unattended Data Traffic refers to data traffic of which the user is unaware he/she initiated, such as when the screen is locked or there has been no user input for a certain period. One example is photo synchronization traffic, which is occasionally transmitted without the user’s awareness. Such traffic can be considered delay-tolerant, as its delayed transmission does not affect user QoE.

- A significant portion of access attempts was found in live networks to originate from Unattended Data Traffic.

## 2.2	Limitations of current UAC framework

The 5G system will provide a single unified access control (UAC) where operators control accesses based on Access Identities and Access Categories. In unified access control, each access attempt is categorized into one or more of the Access Identities and one of the Access Categories. Based on the access control information applicable for the corresponding Access Identity and Access Category of the access attempt, the UE performs a test whether the actual access attempt can be made or not [6]. To be specific, in order for UE to determine the access category applicable for its access attempt, the NAS shall check the rules in table 4.5.2.2 [7], and use the access category for which there is a match for barring check.

Three approaches for controlling access attempts of Unattended Data Traffic can be considered based on the UAC framework, each with its inherent limitations:

- Access Category 1 (=delay-tolerant): Applicable only when the UE is configured for NAS signalling low priority. Typical UE, e.g. smartphones, do not meet this condition.

- Operator-defined Access Category via App ID: Specific applications known to generate Unattended Data Traffic are mapped to an operator-defined access category. This approach assumes that Unattended Data Traffic can be identified on a per-app basis, but unattended traffic is often context-dependent, not app-specific.

- Operator-defined Access Category via URSP Routing: With a specific URSP routing policy on Connection Capability, Background traffic (e.g., Connection Capability identifier "10101000" indicates background traffic) is routed to a dedicated slice and that slice is mapped to an operator-defined access category [5]. This approach requires dedicated slicing per traffic type, which imposes operational burden.

- Three approaches for controlling access attempts of Unattended Data Traffic can be considered based on the UAC framework, each with its inherent limitations.

## 2.3	New access category for Unattended Data Traffic

Since the existing list of access categories do not cover the access attempts from Unattended Data Traffic [6], we propose the introduction of a new access category for Unattended Data Traffic. As access category numbers 11–31 are reserved for future standardized use, access category 11 can be allocated for this purpose. The conditions related to UE can be all except for the conditions in access category 1. Also, the type of access attempt can be MO data consisting only of Unattended Data Traffic.

TS 22.261[6] Table 6.22.2.3-1: Access Categories Access Category number; Conditions related to UE; Type of access attempt. 0; All; MO signalling resulting from paging. 1 (NOTE 1); UE is configured for delay tolerant service and subject to access control for Access Category 1, which is judged based on relation of UE’s HPLMN and the selected PLMN.; All except for Emergency, or MO exception data. 2; All; Emergency. 3; All except for the conditions in Access Category 1.; MO signalling on NAS level resulting from other than paging. 4; All except for the conditions in Access Category 1.; MMTEL voice (NOTE 3). 5; All except for the conditions in Access Category 1.; MMTEL video. 6; All except for the conditions in Access Category 1.; SMS. 7; All except for the conditions in Access Category 1.; MO data that do not belong to any other Access Categories (NOTE 4) (NOTE 8). 8; All except for the conditions in Access Category 1; MO signalling on RRC level resulting from other than paging. 9; All except for the conditions in Access Category 1; MO IMS registration related signalling (NOTE 5). 10 (NOTE 6); All; MO exception data. 11; All except for the conditions in Access Category 1.; MO data consisting only of Unattended Data Traffic (NOTE 7) (NOTE 8). 12-31; Reserved standardized Access Categories. 32-63 (NOTE 2); All; Based on operator classification. NOTE 1: The barring parameter for Access Category 1 is accompanied with information that define whether Access Category applies to UEs within one of the following categories: a) UEs that are configured for delay tolerant service; b) UEs that are configured for delay tolerant service and are neither in their HPLMN nor in a PLMN that is equivalent to it; c) UEs that are configured for delay tolerant service and are neither in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list on the SIM/USIM, nor in their HPLMN nor in a PLMN that is equivalent to their HPLMN. When a UE is configured for EAB, the UE is also configured for delay tolerant service. In case a UE is configured both for EAB and for EAB override, when upper layer indicates to override Access Category 1, then Access Category 1 is not applicable. NOTE 2: When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is neither 0 nor 2, the UE applies the Access Category based on operator classification. When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is 0 or 2, the UE applies the standardized Access Category. NOTE 3: Includes Real-Time Text (RTT). NOTE 4: Includes IMS Messaging. NOTE 5: Includes IMS registration related signalling, e.g. IMS initial registration, re-registration, and subscription refresh. NOTE 6: Applies to access of a NB-IoT-capable UE to a NB-IOT cell connected to 5GC when the UE is authorized to send exception data. NOTE 7: It is up to UE implementation how to identify MO data as Unattended Data Traffic [6]. NOTE 8: When MO data consists of both Unattended and Attended Data Traffic, the access attempts shall be categorized as Access Category 7 rather than Access Category 11..

Also, it is up to UE implementation how to identify MO data as Unattended Data Traffic, which is denoted in NOTE 7. It is noticeable that TS 24.526 clause 4.2.1 NOTE 1 clarifies that the UE implementation is capable to map application-requested connection capabilities to standardized connection capability identifiers, which implies that UE is assumed to have the capability to identify traffic corresponding to unattended or background traffic [5].

Lastly, with the introduction of a new access category, it was considered to avoid any conflicts with the existing access categories as much as possible. Therefore, given that the access attempts corresponding to access category 11 are a subset of the MO data access attempts corresponding to access category 7, it is intended that the access attempts of MO data consisting only of Unattended Data Traffic are included in access category 11, as shown in Figure 2. In cases when MO data consists of both Unattended and Attended Data Traffic, the access attempts shall be categorized as Access Category 7 rather than Access Category 11, as denoted in NOTE 8. It is obvious that access category 11 has nothing to do with all other access categories, since the types of their access attempts are not Unattended Data Traffic.

[Figure 2. The relationship between access category 11 and access category 7]

- Since the existing list of access categories do not cover the access attempts from Unattended Data Traffic, we propose the introduction of a new access category for Unattended Data Traffic.

# 3 Conclusion

- Unattended Data Traffic refers to data traffic of which the user is unaware he/she initiated, such as when the screen is locked or there has been no user input for a certain period. One example is photo synchronization traffic, which is occasionally transmitted without the user’s awareness. Such traffic can be considered delay-tolerant, as its delayed transmission does not affect user QoE.

- A significant portion of access attempts was found in live networks to originate from Unattended Data Traffic.

- Three approaches for controlling access attempts of Unattended Data Traffic can be considered based on the UAC framework, each with its inherent limitations.

- Since the existing list of access categories do not cover the access attempts from Unattended Data Traffic, we propose the introduction of a new access category for Unattended Data Traffic.

# References

[1] S1-253047, “Access category for Unattended Data Traffic” SA1#111

[2] 3GPP TS 22.101 V20.0.0: “Service aspects; Service principles”

[3] GSMA PRD NG.141, version 2.0: “Guidelines for URSP”

[4] 3GPP TS 23.503 V19.4.0: “Policy and charging control framework for the 5G System (5GS); Stage 2”

[5] 3GPP TS 24.526 V19.3.0: “User Equipment (UE) policies for 5G System (5GS) Stage 3”

[6] 3GPP TS 22.261 V20.3.0: “Service requirements for the 5G system”

[7] 3GPP TS 24.501 V19.3.0: “Non-Access-Stratum (NAS) protocol for 5G System (5GS) Stage 3”