3GPP TSG-SA WG1 Meeting #111 	S1-253057

Gothenburg, Sweden, 25-29 August 2025	(revision of S1-252400)

Title:	Native AI integration

Agenda Item:	8.1.3

Source:	Nokia, NIST

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: Native AI integration is pivotal for 6G system, enabling e.g., enhanced automation. This transformative approach is essential to accommodate various use cases and to address future connectivity challenges.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

<< skip unchanged part >>

[X]	3GPP TR 22.850: "Study on 3GPP AI/ML Consistency Alignment".

* * * Next Change * * * *

---------- Use Case template ----------

### 5.7.x	Native AI integration

#### 5.7.x.1	Description

Several organizations stressed the importance of native AI support in 6G during 3GPP stage 1 workshop on IMT-2030 use cases. Native AI integration is essential for networks as critical national infrastructure, ensuring autonomy, efficiency, and adaptability to challenges like labour shortage, declining and aging populations. By embedding AI directly within the network, tasks such as resource allocation, anomaly detection, and network optimization can be automated, reducing operational demands and enhancing performance. AI enables intelligent network management, allows for increased network resiliency, enhances 6G architecture and air interfaces, and provides full automation across the network lifecycle, making AI crucial for advancing and sustaining 6G networks.

Furthermore, the 6G system design needs to enable adoption of AI native principles in a controllable and responsible manner. Main goal is to allow definition of an AI framework enabling use of AI techniques in all network domains without the need to constantly change the architecture and interfaces whenever these AI techniques evolve. The full life cycle of an AI component (e.g. a model) is supported, monitored and managed in an aligned way across all network domains. This emphasizes the needs for an underlying end-to-end and cross-domain AI enablement framework that defines common capabilities, with energy aware orchestration of AI workloads. These common capabilities can be embedded in the network functions or consumed as a service by network functions through common service APIs.

#### 5.7.x.2	Existing features partly or fully covering the use cases functionality

TS 22.261 [14] has defined requirements and KPIs for AI/ML model transfer.

TR 22.850 [Y] investigates and identifies AI/ML related activities of Rel-18 features and Rel-19 studies and work items in TSG CT, TSG RAN and TSG SA Working Groups. AI/ML-related features are inconsistent across WGs, particularly in the areas of training, inference, and data collection.

Regarding ML model training, SA1 has not defined any services or operations.

- SA WG2: Emphasizes a structured approach to ML model training services by defining a clear consumer-producer relationship. This enables specific entities to consume and produce these services, ensuring a well-defined and controlled environment for service utilization.

- SA WG5: Offers a more flexible approach by defining generic ML model training services. This allows for greater adaptability in implementation and usage, without the constraints of a specific consumer-producer relationship.

- SA WG6: Mirrors the approach of SA WG2, prioritizing a clear consumer-producer relationship for its defined services. This aligns with the structured approach advocated by SA WG2.

While SA2 and SA6 restrict the potential producers and consumers, SA WG5 emphasizes flexibility and adaptability. The choice of approach will depend on the specific needs and requirements of the individual service provider and consumer.

Regarding ML model inference, SA1 has not defined any services or operations.

- SA WG2: Defines analytics services through a clear consumer-producer relationship. It defines several analytics types in TS 23.288 [8], each one supported by the NWDAF/RE-NWDAF AnLF and requested/subscribed by the NWDAF/RE-NWDAF AnLF consumer using the defined analytics services.

- SA WG5: Defines generic analytics services without specific consumer-producer relationship in 3GPP TS 28.105 [9] and 3GPP TS 28.104 [71]. It defines several analytics types in TS 28.104 [71], each one supported by an MnS producer and requested by the MnS consumer using the defined analytics services.

- SA WG6: Defines individual analytics services for each analytics type. It defines several analytics types in 3GPP TS 23.436 [33].

While SA WG2 and SA WG6 restrict the potential producers and consumers, SA WG5 emphasizes flexibility and adaptability. Moreover, SA WG2 and SA WG5 defines several analytics types that can be supported by an entity and requested by another entity using the defined ML model inference services. However, in SA WG6, individual services are defined for each analytics type, lacking a generic analytics service definition as seen in SA WG2 and SA WG5.

Regarding data collection, SA1 has not defined any services or operations.

- SA WG2: Defines multiple network functions capable of producing data collection services and defines a function for data storage related services. Leverages event exposure framework and defines event exposure services for network functions that can be consumed by NWDAF (see 3GPP TS 23.288 [8] clause 6.2.2.1). Defines data collection for AI/ML services through a clear consumer-producer relationship.

- SA WG6: Defines network functions similar to those in SA WG2. Data collection services defined for A-DCCF are generic while data collection and data storage services defined for A-ADRF are a mix of generic and use case specific.

- RAN WG3: Defines data collection messages exchanged between two gNBs over the Xn interface, in a P2P manner. It is to be noted that procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”, which means that the intended use of the data (e.g., input, output, feedback) shall not be indicated.

While SA2 and SA6 both define data collection services, their approaches to data storage and retrieval are different. SA WG2 defines generic data storage and retrieval services that can be supported by an entity (ADRF) and requested by another entity but SA WG6 defines both generic and individual services (related to each analytics type) for storing and retrieving data. RAN WG3 operates independently and is unrelated to services defined in SA WGs and therefore can coexist.

Duplicated specification, implementation, and deployment efforts with limited cross-domain AI highlight the need for more cross-domain and end-to-end AI integration in 6G.

#### 5.7.x.3	Potential New Requirements

[PR 5.7.x.3-001] The 6G system shall enable usage of AI supported by data collection, machine learning operations and AI/ML life cycle management, e.g., for enhanced system automation.