3GPP TSG SA WG 1 Meeting #111 	S1-253320

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	Novamint, Thales, TNO, ESA

pCR Title:	Pseudo-CR on clarification of 6G system and satellite access

Draft Spec:	3GPP TR 22.870 V0.3.1

Agenda item:	8.1.1

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This contribution proposes to clarify what is the 6G system in relation with satellite access

1. Introduction

The 6G system natively supports satellite/NTN but some requirements are explitely making references to the 6G system with satellite access or other simlar phrasing making this unclear. The objective is to clarify this.

2. Reason for Change

NTN/Satellite is supported in 3GPP since release 17 and the 6G system includes natively the support of NTN/satellite.

However, there are different phrasing in the study that are referiing to the 6G system and the satellite access and it may be unclear what it means.

For example,

- Clause 5.10.2.2: “The 6G system with satellite access shall…”

- Clause 8.3.6:  “The 6G system via its satellite access, shall….”

- Clause 8.3.6:  “the 6G system using satellite access, shall….”

- Clause 8.9.6: “The 6G system with terrestrial and satellite access shall…”

The proposal is to clarify that satellite/NTN is part of the 6G system and to use “the 6G system using satellite access” when the following requirement applies specifically to the satellite access of the system.

3. Conclusions

None

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.3.1

* * * First Change * * * *

# 8	Ubiquitous Connectivity

Editor's Note: "Ubiquitous Connectivity" is intended to enhance connectivity to bridge the digital divide and enhance user experience, to address presently uncovered or scarcely covered areas, particularly rural, remote and sparsely populated areas, and indoor connectivity. “Ubiquitous Positioning” is intended to enhance the availability and continuity of positioning-enabled services.

## 8.0	General

Connectivity is foundational for providing access and delivering services.  The ITU-R, in [27] describes that further development of ubiquitous connectivity "… would provide digital inclusion for all by meaningfully connecting the rural and remote communities, further extending into sparsely populated areas, and maintaining the consistency of user experience between different locations including deep indoor coverage." Furthermore, ubiquitous positioning is a key capability to enhance the resilience, accuracy and availability of positioning-enabled services.

These use cases consider expanding network as well as improving user experiences, where possible, for advanced services (e.g. positioning, location). Connectivity could also be enhanced through interworking with other systems. Capabilities of existing satellite networks and normative requirements specified in [14] provide the foundation for these use cases.

Since Release 17, 3GPP introduced 3GPP NTN (Non-Terrestrial Network) to support satellite or HAPS based radio access network as part of 3GPP with 2 defined radio access technologies that have been enhanced with NTN capabilities:

- IoT-NTN as part of the Massive Internet of Things

- NR-NTN as part of the 5G NR radio interface family

As the 6G system includes NTN and satellite natively, the underlying assumption is that the 6G system will inherit from day 1 all existing NTN features and services developed previously in 3GPP NTN e.g.

- Discontinuous coverage

- Store & Forward Satellite Operation

- UE-Satellite-UE communication

- Support of multiple satellite orbit types such as GEO, MEO, LEO and Multi-Orbits combination

- Backhauling by satellite

The use of the clarifying phrase “the 6G system using satellite access” implies the following text applies specifically to the satellite access.

* * * Next Change * * * *

#### 5.10.2.2	Potential New Requirements needed to support the use case

[PR 5.10.2.2-1]: The 6G system using satellite access shall support UEs with the following performances:

Table 5.10.2.2-1: Performance requirements for 6G satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pedestrian (note 2) | [1] Mbit/s | [100] kbit/s | 1,5 Mbit/s/km2 | 150 kbit/s/km2 | [100]/km2 | [1,5] % | Pedestrian | Handheld | - |
| Vehicular connectivity (note 3) | [50] Mbit/s | [25] Mbit/s | TBD | TBD | TBD | 50 % | Up to 250 km/h | Vehicle mounted | - |

Editor’s note: The table above will be filled with new scenarios compared to 5G and with legacy scenarios for which the performance values need to be updated. The Table 7.4.2-1: “Performance requirements for satellite access” in TS 22.261 [14] is considered as a basis. Potential new scenarios include for example flying/surface drone-mounted, railway moving platforms and NGSO satellite-mounted devices.

* * * Next Change * * * *

### 8.2.6	Potential New Requirements needed to support the use case

[PR 8.2.6-1]: The 6G system using satellite access shall optimize the coverage per LEO satellite.

[PR 8.2.6-2]: The 6G system using satellite access shall support service continuity when UE is within the optimized coverage of the LEO constellation, e.g. in the sparse LEO scenarios.

[PR 8.2.6-3]: The 6G system using satellite access shall be able to support flexible configurations (e.g. parameters for satellite access) for both sparse satellite constellation and dense satellite constellation.

[PR 8.2.6-4]: The 6G system using satellite access shall be able to provide the location service meeting the regulatory requirements for both sparse satellite constellation and the dense satellite constellation.

Editor's Note: [PR 8.2.6-4] is FFS.

* * * Next Change * * * *

### 8.3.6	Potential New Requirements needed to support the use case

[PR 8.3.6-1]: The 6G system shall be able to ensure service continuity with minimum interruption for UEs during the transition between terrestrial and satellite access and vice versa.

[PR 8.3.6-2]: The 6G system using satellite access, shall support SMS delivery to a high density of UEs i.e. up to [1000] UE per km2.

[PR 8.3.6-3]: Subject to regulatory requirements, the 6G system using satellite access, shall be able to support PWS for broadcasting warning notifications to UEs in adverse propagation conditions e.g. light indoor conditions, dense forest.

* * * Next Change * * * *

### 8.4.6	Potential New Requirements needed to support the use case

[PR 8.4.6-1]	The 6G system using satellite access shall be able to provide the UE with positioning service with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS) with the following KPIs:

Table 8.4.6-1: Performance requirements for satellite-based positioning services

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| Airplane en-route | [50] m | [50] m | [99 %] | [1] s | Outdoor | Up to [1500] km/h | Airplane mounted |
| Airplane landing | [10] m | [10] m | [99 %] | [1] s | Outdoor | Up to [350] km/h | Airplane mounted |
| UAV positioning | [1] m | [1] m | [99 %] | [1] s | Outdoor | Up to [160] km/h | UAV mounted |
| NOTE: Multiple satellites can be used to support 3GPP positioning technologies. |  |  |  |  |  |  |  |

[PR 8.4.6-2]	Subject to regulatory requirements, and operator policy, and to user’s consent, the 6G system using satellite access shall be able to determine the UE location.

* * * Next Change * * * *

### 8.6.6	Potential New Requirements needed to support the use case

[PR 8.6.6-1]: The 6G system using satellite access shall be able to provide a mechanism for energy constrained devices to determine its position using 3GPP positioning technology.

[PR 8.6.6-2]: The 6G system using satellite access shall be able to provide positioning service with the following KPIs:

Table 8.6.6-1: Performance requirements for satellite-based positioning services for energy constrained devices

| Scenario | Battery Life Expectancy (note) | Horizontal Accuracy (95 % confidence level) | Positioning service availability | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- |
| Containers | 12 years | [100] m | [95 %] | Outdoor | Up to [37] km/h | Container mounted |
| Wagons | 20 years | [100] m | [95 %] | Outdoor | Up to [350] km/h | Train mounted |
| Pallets | 7 years | [100] m | [95 %] | Outdoor | Up to [100] km/h | Vehicle mounted |
| NOTE: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency, e.g. the number of messages per day by a typical user |  |  |  |  |  |  |

Editor’s note:  the source of the value in the table above will be provided later.

* * * Next Change * * * *

### 8.8.6	Potential New Requirements needed to support the use case

[PR 8.8.6-1] Subject to operator’s policy and agreement with 3rd party, the 6G network using satellite access shall be able to provide a suitable Service Hosting Environment on board satellite to a UAV using only satellite access e.g. considering the latency and satellite capabilities.

Editor's Note:	 this requirement is FFS.

Editor's Note:	 6G network or 6G system is FFS.

[PR 8.8.6-2] The 6G system shall be able to support communication service for UAV with variant altitudes (e.g.  from 0 to 3 km [99]) with satellite access and/or terrestrial access.

[PR 8.8.6-3] Subject to operator’s policy and user’s consent, the 6G system using satellite access shall be able to support data sharing among multiple UAVs.

NOTE: The shared data is sensing results processed by Service Hosting Environment on aboard satellite based on non-3GPP sensing data from the UAV.

Editor’s note: This NOTE is FFS.

* * * Next Change * * * *

### 8.9.6	Potential New Requirements needed to support the use case

[PR 8.9.6-1]	The 6G system using terrestrial and satellite access shall be able to provide high-accuracy positioning service with high availability, by using 3GPP positioning technologies and fulfilling the following KPIs:

Table 8.9.6-1: Performance requirements for positioning services based on hybrid TN and NTN positioning technologies

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| Airplane taxiway (note) | [1] m | N/A | [99 %] | [0.1 - 0.5] s | Outdoor | Up to [100] km/h | Airplane mounted |
| UAV facility monitoring | [3] m | [3] m | [99 %] | [0.1 - 0.5] s | Outdoor | Up to [160] km/h | UAV mounted |
| NOTE: Airplane taxiway refers to an airplane on ground during taxi operations. |  |  |  |  |  |  |  |

Editor's Note:	 it is FFS whether KPIs are needed, and if needed, how the accuracy requirements are justified in relation to the latency and UE speed.

* * * Next Change * * * *

### 8.10.6	Potential New Requirements needed to support the use case

[PR 8.10.6-1]	The 6G system using satellite access shall be able to provide a positioning service supported by hybrid NTN and GNSS positioning technologies.

Editor’s note: The KPIs are FFS.

[PR 8.10.6-2]	The 6G system using satellite access shall be able to support Network-based Assisted GNSS to improve NTN positioning.

* * * Next Change * * * *

### 8.13.6	Potential New Requirements needed to support the use case

[PR 8.13.6-1]	The 6G system using satellite access shall be able to provide time synchronization to UEs and applications using 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS).

* * * Next Change * * * *

### 11.4.6	Potential New Requirements needed to support the use case

[PR 11.4.6-1] The 6G system using satellite access shall provide a means to support immersive media services with the following KPIs:

Table 11.4.6-1: Performance requirements for immersive media service via satellite access

| Scenario | Service interruption time (ms) | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) | Area traffic capacity (UL) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Immersive media service via AAM/UAM (direction: from network to user, uncompressed) | [10ms] for 60 fps, [20ms] for 30 fps (note 1) | [1.3] Gbits/s (note 2) | N/A | TBD | N/A | [4]/km2 | N/A | Up to [300] km/h | UAM mounted |
| Immersive media service via AAM/UAM (direction: from network to user, compressed) | [10ms] for 60 fps, [20ms] for 30 fps (note 1) | [25] Mbits/s (note 3) | N/A | TBD | N/A | [4]/km2 | N/A | Up to [300] km/h | UAM mounted |
| NOTE 1: It is assumed that the interruption time is less than a single frame (16ms for 60 fps, 32 ms for 30 fps) plus a margin of an order of millisecond (e.g. for other processing time). NOTE 2: Uncompressed (0.8 M points per frame, total 300 frames are considered: i.e. 43.2 Mbits/s, 30 fps) NOTE 3: Compressed (Video-based Point Cloud Compression (V-PCC), bpip (bits per input points) total 1.14 (Color 0.9, Geometry 0,11, Occupancy 0.13) are considered: 25 Mbits/s) NOTE 4: It is assumed that the number of passengers using immersive media service per UAM is up to 2. |  |  |  |  |  |  |  |  |  |

[PR 11.4.6-2] The 6G system using satellite access shall provide a means to obtain and expose application service interruption information (e.g. expected interruptions to the service) to authorized third party to minimize interruption for immersive media services.

* * * Next Change * * * *

### 11.5.6	Potential New Requirements needed to support the use case

[PR 11.5.6-1] The 6G System using satellite access shall support high-data rate aircraft communications with the following KPI requirements.

Table 11.5.6-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) | Area traffic capacity (UL) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Airplanes connectivity (note) | 3 Gbit/s per plane | 1.5 Gbit/s per plane | N/A | N/A | N/A | 40% | Up to 1500 km/h | Airplane mounted |
| NOTE: - Required experienced peak data rate corresponding to the aggregated passenger traffic at aircraft level - Based on an assumption of 450 seats, average take rate of 75% (free model) and load factor of 85% - Assumption of 2:1 Downlink / Uplink ratio, anticipating future usages - The Downlink & Uplink throughput can be achieved using one or multiple satellite links |  |  |  |  |  |  |  |  |

[PR 11.5.6-2] The 6G System using satellite access shall support high data rate aircraft communications for on-board UEs with the following KPI requirements.

Table 11.5.6-2: Performance requirements for on-board UEs

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) | Area traffic capacity (UL) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Airplanes connectivity | 50 Mbit/s | 25 Mbit/s | 6 Gbit/s/plane | 3 Gbit/s/plane | 400/plane | 30 % | Users in airplanes (up to 1500 km/h) | On-board UE (note) |
| NOTE: These KPIs are related to UEs inside the aircraft where the UE can be connected to on-board base station. These KPIs enable the case of XR UEs for applications in highly dynamic environments, where UEs offload processing to an edge-cloud. |  |  |  |  |  |  |  |  |

[PR 11.5.6-3] The 6G System using satellite access shall support high data-rate aircraft communications with massive data transfer when the aircraft is on the ground or near the ground with the following KPI requirements.

* * * End of Change * * * *