3GPP TSG SA WG 1 Meeting #111 	S1-253342

Goteborg, Sweden, 25 -29 August 2025	(revision of S1-253211)

Source:	ESA, Fraunhofer, Airbus, TNO, Thales, Novamint

Title:	New use case on detection of ships in the open sea

Draft Spec:	3GPP TR 22.870 V0.3.1

Agenda item:	8.1.4 Integrated Sensing and Communication (FS_6G_REQ)

Document type:	pCR

Document for:	Discussion/Approval

Contact:	relja.djapic@tno.nl

Abstract: This document proposes to address integrated sensing and communication from space using NTN and introduces a new use case on detecting ships in the open sea.

1. Introduction

Integrated Sensing and Communication (ISAC) is an important element of 6G as reflected in Chapter 7 of TR 22.870. However, the use cases so far explicitly only refer to the use of terrestrial networks (TN) for ISAC. As the field of ISAC from space is rapidly emerging ([X1], [X2]) it becomes apparent that also non-terrestrial network (NTN) can support integrated sensing application.

The following table highlights the ISAC use cases currently in TR 22.870 V0.3.1 that would benefit from NTN contributions in particular (but not restricted to) remote areas:

| ID | Use case on | Remarks |
| --- | --- | --- |
| 7.1 | coordination of search and rescue missions in large disaster areas | The satellite assets of a NTN Satellites can provide significant contributions to coordination in disaster situations in particular in remote areas and if the operation of the TN is impaired by the disaster (e.g. loss of terrestrial base stations, …) |
| 7.3 | high-resolution topographical maps | NTN can provide additional sensor layers to improve accuracy of topographical maps |
| 7.6 | road digitalization | NTN ISAC can complement by provisioning suitable traffic flow data in remote areas with a scarcity of terrestrial base stations. |
| 7.9 | advanced modern city transportation system | Analogously to 7.6 NTN can complement TN ISAC services for providing additional data to Intelligent Transportation Systems |
| 7.10 | stored sensing data handling | NTN can provide additional sensing data upon request especially in barely accessible remote regions |
| 7.14 | infrastructure collapse monitoring | Infrastructure monitoring and change detection is an estalbished Earth Observation use cas, NTN can contribute to pre-event monitoring through wide area surveillance and automated change detection |
| 7.15 | multi-sensor fusion based sensing for UAV takeoff and landing | NTN ISAC results can be fused into the results from other (terrestrial) sensors to improve accuracy |
| 7.17 | safe & economic UAV transport | UAV transport is in particular viable in rural regions where TN coverage can be sparse. NTN coverage can complement with sensing data for safe UAV operation even in these areas |
| 7.20 | autonomous driving based on network-assisted sensing | Sensing data obtained by the 3GPP network – both TN and NTN – together with the vehicle’s onboard sensors can very advantageous for the safe operation of the autonomous vehicle |

The use case 7.8 on  the detection of ships on the coast or in rivers is relying on ISAC with terrestrial base stations, which are limited in reach. Utilizing NTN ship detection can also be performed in remote oceanic regions.

Consequently, this contribution proposes a new use case on the detection of ships in the open seas, based on ISAC from space, i.e., based on the usage of  NTN domain.

2. Reason for Change

Ensure that the capabilities of Integrated Sensing and Communications from space, i.e. by NTN, will be supported by 6G networks.

3. Conclusions

Add a new use case on Integrated Sensing and Communications from space (NTN) in TR 22.870.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.3.1.

* * * 1st Change * * * *

# 2	References

[X1]	M. Corici et al. "Repurposable payloads: concept, design, and initial results," 41st International Communications Satellite Systems Conference (ICSSC 2024), Seattle, USA, 2024, pp. 120-124, doi: 10.1049/icp.2024.4622.

[X2]	J. Kim et al, “Integrated Sensing and Communication from Space – Satellite Payload Hardware Aspects”, Proceedings of 2025 IEEE Radar Conference, Krakow, Poland, 2025.

* * * 2nd Change* * * *

## 7.x	Use case on detection of ships in the open sea

### 7.x.1	Description

Maritime transportation is the primary means of cargo transport worldwide and the key enabler of global supply chains. Over two-thirds of global international trade volume and approximately 90% of China's import and export freight are transported container ships.

Ship detection plays an important role in the management of ship traffic, cargo transportation and the maintenance of efficient supply chains. But maritime travel on the oceans is still not without danger, and wide area surveillance on the open seas can be an important element in efficient search and rescue activities in case of accidents, possibly saving human life. Oceans cover 70 % of the Earth’s surface, NTN is an enabler for efficient global maritime surveillance. In this task it needs to cope with a range of ship sizes from smaller recreational vessels (length about 15 m) to huge container ships with lengths of more than 300 m.

In summary, ship detection is a crucial scenario. NTN has the potential to provide large coverage and ship detection services on the open seas.

### 7.x.2	Pre-conditions

Satellite Network Operator MM provides “ocean ship detection and tracking” service with 6G system.

Satellite Network Operator MM can access 6G NTN base stations possibly also complemented by TN base stations to monitor ships on the open seas.

Company NN subscribes to this ocean ship detection and tracking service from Network Operator MM..

### 7.x.3	Service Flows

- Company NN operates a fleet of ships to transport goods.

- The 6G NTN base stations, accessible by the Satellite Network Operator MM, could transmit and receive the sensing signal. The 6G network could generate the sensing results based on these measurements.

- Based on the sensing results, 6G network could sense and track Company MM’s ships. Within the sensing service with high accuracy and resolution, 6G network could derive the position and velocity information of Company NN’s ship allowing to precisely estimate the estimated time of arrival at its destination.

- Company NN has frequent updates of the location and status of their ships and act quickly on deviations of the trip itinerary and schedule and also on emergencies.

### 7.x.4	Post-conditions

Company NN delivers the goods safely and forbidden zones are observed. Thanks for the 6G system with ship tracking and ship recognition function, Company NN is always aware of the status of its fleet of ships.

### 7.x.5	Existing features partly or fully covering the use case functionality

In TS 22.137 [6], the basic procedure of sensing has been described, including capability reporting, data collection, data transmission, data processing, and capability exposure, listed below:

- The 6G system shall be able to provide sensing service to detect, and/or track one or more objects (e.g. UAVs, birds) and the environment around the object(s).

- Based on operator’s policies, operator’s control and regulation, the 6G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

- Subject to operator’s policy and regulation, the 6G system shall be able to provide secure means for a trusted third-party to receive sensing results with contextual information.

### 7.x.6	Potential New Requirements needed to support the use case

[PR 7.x.6-1] The 6G system shall be able to provide ship sensing services on the open sea as described in the following table:

Table 7.8.6-1: Performance requirements for 6G ship detection and tracking in the open sea

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [s] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Ship detection and tracking | Large container vessel (270 m x 30 m x 30 m) | 95 | ≤100 | NA | NA | NA | ≤100 | NA | 60 | 1 | 5 | 5 |  |
|  | Small ocean faring ship (15 m x 5 m x 10 m) | 95 | ≤100 | NA | NA | NA | ≤100 | NA | 60 | 1 | 5 | 5 |  |