3GPP TSG-SA WG1 Meeting #111 	S1-253356

Goteborg, Sweden 25-29 August 2025	(formerly S1-25xxxx)

Source:	Xiaomi

pCR Title:	pCR for New use case UE sensor groups

Draft Spec:	TR 22.870 v0.3.1

Agenda item:	8.1.4

Document for:	Approval

Contact:	Gordon Young <gordonpetery@xiaomi.com

Abstract: this paper proposes a new use case for sensor groups for a UE

7.X Autonomous driving service with a UE using multiple sensors.

7.X.1 Description

Vehicles equipped with a UE for 3GPP sensing service (may or may not be a 3GPP sensor) may have one or more sensors installed in a distributed manner.

Sensor additional information for each sensor may include know offset from UE localisation target point (e.g. UE 3GPP positioning /communication antenna), in addition to yew, pitch and roll of the sensor at each sensing measurement may be included as additional sensing information, along with the measured sensor data. In one alternative the vehicle may pre-process the respective sensor measurement data creating a fused sensing result for the UE. The UE reports the respective sensor measurement data for each sensor or fused measurement result in accordance with the configured sensing service configuration.

Sensing target area for a sensor is directly related to the location of the sensor, the orientation of the sensor in regards to the true position of the UE and the aperture of the sensor.  The aperture for the sensor may be tuned to provide optimum performance in a narrow field of view, e.g. not omni-directional (in fact the material and position of the sensor mount may mean significant areas of an omni-directional field of view are obscured). Each sensor may for example be positioned on the vehicle so that it provides distinct or different sensing target area, e.g. on a car or transport vehicle sensors may provide forward facing and rear facing views.

For road users operating in a sensing service target area the choice of activating a UE sensor is determined by the 6G network function controlling the authorized sensing service. The sensing service for example may be provided by a trusted third party.

In this case the sensors may be considered as a sensor group [TR22.837] wherein they have a know proximity location to one UE which is responsible for managing the sensors, and communicating with the sensing service.

On discovery for available 3GPP sensors the MNO P sensing service identifies the road users to determine available UEs supporting 6G sensing service the 6G Sensing function may discover that a vehicle has more than one sensor, or more than one target sensing area.

In order to ensure network efficiency rather than have the UE send all available sensing data selecting one or more of the sensors individually is advantageous. In some instances one of the sensors may not be facing the target sensing area, or as a receiver may not be facing in sufficient direction of a sensing transmitter to provide meaningful sensing measurement data.

In this case the sensor connections to the UE are considered to be ideal (no loss) and can be wireless or wired.

7.X.2 Pre-conditions

Vehicles a, b, c and d are registered subscribers to Autonomous driver service X with provides V2X services and sensing services of different level of autonomy e.g. mapping, warning and sensing services to aid drivers of vehicles a, b, c and d.

Autonomous driving service X has an agreement with MNO P for provision of sensing service and V2X services including localisation and user privacy for reported service results.

Vehicles a, b, c and d are registered with MNO P and are in coverage and connected.

Vehicle a has 2 sensors fitted 1 front and 1 rear. Service X is aware of the two sensors and has provided this capability information to sensing service of MNO P. Initially only the forward sensor is activated and providing sensing information.

Vehicle d is stationary.

7.X.3 Service Flows

- The vehicles a, b, c and d are travel in a rural area and approaching a T-junction. Traffic is approaching from both the left and right and vehicles a and b, need to turn into the road passing the T-junction safely.

- The visibility at the junction is poor due to high hedges and poor light etc., so vehicle travel is more reliant on the service X for safe passage, and avoiding of traffic.

Figure 7.X.3-1 Vehicles approaching T-junction – and after vehicle a has turned

- Approaching the junction the sensing service X determines that it wishes to have the rear sensor on vehicle a activated, so that it can have an enhanced view of vehicle c, after vehicle a has entered the same lane of traffic. Also the rear view will give a clear view of vehicle d, including capturing its size and if it starts moving, to better inform the sensing service of vehicle d potentially causing a problem for vehicle b which is intending to turn in the opposite direction to vehicle a, and into the same lane as vehicle d.

- Vehicle a provides sensor measurement data for both sensors to the MNO P sensing service being operated and delivered to service provider X.

- The MNO P sensing service with knowledge regarding the rear sensor orientation (field of view) is able to fuse the sensing data of the additional sensor from vehicle a with the other received sensing data from vehicles a, b, c and d to create the required sensing result.

- Vehicle d moves off, gaining speed and approaches the junction.

- MNO P forwards the sensing result in accordance with the sensing service requirements to service provider X.

- Sensing service provider X delivers autonomous driving service combined with sensing results to vehicles a, b, c and d.

7.X.4 Post Conditions

Sensing service provider detects and determines the size of vehicle d early as vehicle a pass it, and is able to ensure vehicles a and b navigate the junction safely.

7.X.5	Existing features partly or fully covering the use case functionality

7.X.6	Potential New Requirements needed to support the use case

[PR 7.X.6-1] Based on third-party request, the 6G system shall be able to discover a sensing group in the proximity of the UE that is requesting the service from the AS.

NOTE:	This requirement assumes that the additional rear sensor on vehicle a is not preconfigured as a normal configuration for the vehicle, and is triggered to discover the new configuration.

[PR 7.X.6-2] Subject regulatory requirements and the operator’s policy, the 6G network shall be able to provide secure means to enable trusted third-party to request discovering a sensing group related to a UE in a sensing service area as indicated from application server.