3GPP TSG-SA1 Meeting #111	S1-253366

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0851 | rev | - | Current version: | 20.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ordering Charging Requirements in TS 22.261 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SMARTER_Ph2, TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Although the Rel-15 version of TS 22.261 included a clause for charging requirements (clause 9), subsequent additions have not follow the original sttructure making more challenging to find such requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Creating new sections under Clause 9 Charging for features/services with charginig requirments already included in TS 22.261. Moving those requirements to clause 9, and in the cases where this is not prossible, making a link to the section where to find charging requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some charging requirments for some features may be oeverlooked by downstream working groups. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.21, 6.31, 6.40, 6.50, 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.21	NG-RAN Sharing

### 6.21.1	Description

The increased density of access nodes needed to meet future performance objectives poses considerable challenges in deployment and acquiring spectrum and antenna locations. RAN sharing is seen as a technical solution to these issues.

In RAN Sharing operations, NG-RAN resources can be used by multiple network operators. For the NG-RAN resources deployed onboard satellites, they can be used by both satellite mobile network operators and terrestrial network operators together by proper network operations.  Indirect Network Sharing is one of the possible sharing methods.

In addition, Indirect Network Sharing can offer more options for an operator to provide services to UEs via Shared NG-RAN, when a Disaster Condition occurs in a geographic area, which causes the loss of its UEs’ previous network connection, until the Disaster Condition is not applicable anymore.

NOTE:     This scenario assumes that NG-RAN and/or core network of at least one operator is unavailable but that the NG-RAN and core network of at least another operator is available in the disaster area. In case the affected operator has multiple core networks in different geographic areas, when a core network of that operator has broken down in the specific disaster area, the other core network from the different area can be used temporarily in this case.

During NG-RAN sharing, the security and privacy of shared networks, non-shared networks, and subscribers need to be maintained without negative effects. Especially in the case of Indirect Network Sharing, where the involvement of the core network of the hosting operator e.g. for signalling exchange between the users and the core network of the participating operator could cause exposure of the subscriber’s information to the hosting network, an extra scrutiny of the security mechanism is expected to avoid sharing the information that is not needed for the Indirect Network Sharing operation (e.g. network topology) and protect the information that is needed for the Indirect Network Sharing operation between the hosting operator and the participating operator.

### 6.21.2	Requirements

#### 6.21.2.1	General

Requirements related to NG-RAN sharing are described in 3GPP TS 22.101 [6], mainly in clause 28.2.

A 5G satellite access network shall support NG-RAN sharing.

#### 6.21.2.2	Indirect network sharing

The 5G system shall be able to support Indirect Network Sharing between the Shared NG-RAN and one or more Participating NG-RAN Operators’ core networks, by means of the connection being routed through the Hosting NG-RAN Operator’s core network.

NOTE 1:	Requirements of Indirect Network Sharing assume no impact on UE.

NOTE 2: 	For more information on Indirect Network Sharing see Annex I.

Indirect Network Sharing shall be transparent to the user.

NOTE 3:	This requirement is aligned with the existing requirement in 3GPP TS 22.101 [6] clause 4.9.

The following existing service requirements related to network sharing in 3GPP TS 22.101 [6] are applied to Indirect Network Sharing:

-	clause 4.2.1,

-	clause 28.2.3, and

-	clause 28.2.5.

Subject to regulatory requirements or operator policy, the 5G network shall support a PLMN operator to be made aware of the failure or recovery of NG-RAN and/or core network in other PLMN(s) in the same area when the Disaster Condition applies, or when the Disaster Condition is not applicable anymore.

Subject to regulatory requirements or operator policy, the 5G network shall support a PLMN operator to be made aware of the availability of other PLMN(s) as Hosting NG-RAN Operator(s) via Indirect Network Sharing in the same area when the Disaster Condition applies.

Subject to the agreement between the hosting and participating operator, the 5G system shall support a means to

enable a UE of the Participating NG-RAN Operator to:

-	access their subscribed PLMN services when accessing a Shared NG-RAN, and/or,

-	obtain its subscribed services, including Hosted Services, of participating operator via a Shared NG-RAN.

NOTE 4: The above requirement is applicable to Disaster Condition via a Shared NG-RAN.

Based on operator policy, the 5G system shall support a mechanism to enable an authorized UE with a subscription to a Participating Operator to select and access a Shared NG-RAN.

Based on operator policy, the 5G system shall support access control for an authorized UE accessing a Shared NG-RAN and be able to apply differentiated access control for different Shared NG-RANs when more than one Shared NG-RAN are available for the Participating Operator to choose from.

Based on operator policy, the 5G network shall be able to apply differentiated access control for an authorized UE connecting to Participating Operator’s core network e.g. depending on specifc geographic areas within the coverage of a shared satellite NG-RAN.

Based on operator policy, the 5G network shall minimize network congestion caused by Indirect Network Sharing when a Disaster Condition applies.

NOTE 5: Population density in the different areas where Disaster Condition apply needs to be considered.

Based on operator policy, the 5G system shall enable the Participating Operator to provide steering information in order to assist a UE with access network selection amongst the Hosting Operator’s available Shared RAN(s).

Subject to the agreement between operators, the 5G network shall enable the Participating Operator to assist an authorized UE with access network selection when this UE is located both within the coverage of a shared satellite NG-RAN and within the coverage of terrestrial NG-RANs.

The 5G system shall support service continuity for UEs that are moving between different Shared NG-RANs and/or between a Shared NG-RAN and a non-Shared NG-RAN.

The 5G system shall be able to provide a UE accessing a Shared NG-RAN network with positioning service in compliance with regulatory requirements.

Subject to regulatory requirements and mutual agreement between the participating operators and the hosting operator, the requirements to support regulatory services, e.g., PWS or emergency calls apply to Indirect Network Sharing.

Subject to agreement between operators the 5G system shall enable the Shared NG-RAN of a hosting operator to provide services for inbound roaming users.

The 5G network shall be able to enable Indirect Network Sharing only when the Disaster Condition applies in a specific area and disable it when no longer applicable.

NOTE 6: It is assumed operators can have sharing agreement for Disaster Condition in the area.

NOTE 7: It is assumed that during a Disaster Condition, previous network communication is temporarily disabled.

The 5G network shall be able to provide a means for a UE to return to the PLMN used prior to Indirect Network Sharing, when a Disaster Condition is no longer applicable.

* * * Next Change * * * *

## 6.31	Minimization of Service Interruption

### 6.31.1	Description

A mobile network can fail to provide service in the event of a disaster (for example a fire.) The requirements listed in this clause provide the 5GS with the capability to mitigate interruption of service. UEs can obtain service in the event of a disaster, if there are PLMN operators prepared to offer service. The minimization of service interruption is constrained to a particular time and place. To reduce the impact to the 5G System and EPS of supporting Disaster Roaming, the potential congestion resulting from an influx or outflux of Disaster Inbound Roamers is taken into account.

Scenarios where network failures render the network subject to a disaster unable to authenticate its subscribers are excluded.

### 6.31.2	Requirements

#### 6.31.2.1	General

Subject to regulatory requirements or operator's policy, 3GPP system shall be able to enable a UE of a given PLMN to obtain connectivity service (e.g. voice call, mobile data service) from another PLMN for the area where a Disaster Condition applies.

Subject to regulatory requirements, operator's policy or UE capabilities, the 3GPP system shall be able to support a UE, with 5G-only national roaming access to a VPLMN, to obtain 4G connectivity service  from that VPLMN in the area where a Disaster Condition applies.

NOTE:	In the above scenario, voice call service is provided by IMS in HPLMN.

Subject to regulatory requirements or operator's policy, in case of shared RAN between participating PLMNs, the 3GPP system shall be able to support a UE of a given PLMN to obtain connectivity service (e.g. voice call, mobile data service) from another participating network when a Disaster Condition applies to the UE’s PLMN.

#### 6.31.2.2	Disaster Condition

The 3GPP system shall enable UEs to obtain information that a Disaster Condition applies to a particular PLMN or PLMNs.

NOTE:	If a UE has no coverage of its HPLMN, then obtains information that a Disaster Condition applies to the UE's HPLMN, the UE can register with a PLMN offering Disaster Roaming service.

The 3GPP system shall support means for a PLMN operator to be aware of the area where Disaster Condition applies.

The 3GPP system shall be able to support provision of service to Disaster Inbound Roamer only within the specific region where Disaster Condition applies.

The 3GPP system shall be able to provide efficient means for a network to inform Disaster Inbound roamers that a Disaster Condition is no longer applicable.

Subject to regulatory requirements or operator’s policy, the 3GPP system shall support a PLMN operator to be made aware of the failure or recovery of other PLMN(s) in the same country when the Disaster Condition is applies, or when the Disaster Condition is not applicable.

#### 6.31.2.3	Disaster Roaming

The 3GPP system shall be able to provide means to enable a UE to access PLMNs in a forbidden PLMN list if a Disaster condition applies and no other PLMN is available except for PLMNs in the forbidden PLMN list.

The 3GPP system shall provide means to enable that a Disaster Condition applies to UEs of a specific PLMN.

The 3GPP system shall be able to provide a resource efficient means for a PLMN to indicate to potential Disaster Inbound Roamers whether they can access the PLMN or not.

Disaster Inbound Roamers shall perform network reselection when a Disaster Condition has ended.

The 3GPP system shall minimize congestion caused by Disaster Roaming.

The 5G system and EPS shall support a mechanism for the HPLMN to control whether a UE, with HPLMN subscription, should apply Disaster Roaming when a Disaster Condition arises (in the HPLMN or a VPLMN).

* * * Next Change * * * *

## 6.40	AI/ML model transfer in 5GS

### 6.40.1	Description

Artificial Intelligence (AI)/Machine Learning (ML) is being used in a range of application domains across industry sectors. In mobile communications systems, mobile devices (e.g. smartphones, automotive, robots) are increasingly replacing conventional algorithms (e.g. speech recognition, image recognition, video processing) with AI/ML models to enable applications. The 5G system can at least support three types of AI/ML operations:

-	AI/ML operation splitting between AI/ML endpoints

The AI/ML operation/model is split into multiple parts according to the current task and environment. The intention is to offload the computation-intensive, energy-intensive parts to network endpoints, whereas leave the privacy-sensitive and delay-sensitive parts at the end device. The device executes the operation/model up to a specific part/layer and then sends the intermediate data to the network endpoint. The network endpoint executes the remaining parts/layers and feeds the inference results back to the device.

-	AI/ML model/data distribution and sharing over 5G system

Multi-functional mobile terminals might need to switch the AI/ML model in response to task and environment variations. The condition of adaptive model selection is that the models to be selected are available for the mobile device. However, given the fact that the AI/ML models are becoming increasingly diverse, and with the limited storage resource in a UE, it can be determined to not pre-load all candidate AI/ML models on-board. Online model distribution (i.e. new model downloading) is needed, in which an AI/ML model can be distributed from a NW endpoint to the devices when they need it to adapt to the changed AI/ML tasks and environments. For this purpose, the model performance at the UE needs to be monitored constantly.

-	Distributed/Federated Learning over 5G system

The cloud server trains a global model by aggregating local models partially-trained by each end devices. Within each training iteration, a UE performs the training based on the model downloaded from the AI server using the local training data. Then the UE reports the interim training results to the cloud server via 5G UL channels. The server aggregates the interim training results from the UEs and updates the global model. The updated global model is then distributed back to the UEs and the UEs can perform the training for the next iteration.

### 6.40.2	Requirements

#### 6.40.2.1	Requirements for direct network connection

Based on operator policy, the 5G system shall be able to provide means to allow an authorized third-party to monitor the resource utilisation of the network service that is associated with the third-party.

NOTE 1:	Resource utilization in the preceding requirement refers to measurements relevant to the UE’s performance such as the data throughput provided to the UE.

Based on operator policy, the 5G system shall be able to provide an indication about a planned change of bitrate, latency, or reliability for a QoS flow to an authorized 3rd party so that the 3rd party AI/ML application is able to adjust the application layer behaviour if time allows. The indication shall provide the anticipated time and location of the change, as well as the target QoS parameters.

Based on operator policy, 5G system shall be able to provide means to predict and expose predicted network condition changes (i.e. bitrate, latency, reliability) per UE, to an authorized third party.

Subject to user consent, operator policy and regulatory constraints, the 5G system shall be able to support a mechanism to expose monitoring and status information of an AI-ML session to a 3rd party AI/ML application.

NOTE 2:	Such mechanism is needed for AI/ML application to determine an in-time transfer of AI/ML model.

5G system shall be able to provide event alerting to an authorized 3rd party, together with a predicted time of the event (e.g., alerting about traffic congestion or UE moving into/out of a different geographical area).

NOTE 3:	A 3rd party AI/ML application may use the prediction information to minimize disturbance in the transfer of learning data and AI/ML model data.

The 5G system shall be able to expose aggregated QoS parameter values for a group of UEs to an authorized service provider.

The 5G system shall be able to support an authorised 3rd party to change aggregated QoS parameter values associated with a group of UEs, e.g. UEs of a FL group.

Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to expose information (e.g. candidate UEs) to an authorized 3rd party to assist the 3rd party to determine member(s) of a group of UEs (e.g. UEs of a FL group).

#### 6.40.2.2	Requirements for direct device connection

Based on user consent, operator policy and trusted 3rd party request, the 5G system shall support a means to authorize specific UEs to transmit data (e.g. AI-ML model data for a specific application,) via direct device connection in a certain location and time.

Based on user consent, operator policy, and trusted 3rd party’s request, the 5G system shall be able to provide means for an operator to authorize specific UEs who participate in the same service (e.g. for the same AI-ML FL task) to exchange data with each other via direct device connection, e.g. when direct network connection cannot fulfil the required QoS.

Based on user consent, operator policy and trusted 3rd party request, the 5G system shall be able to dynamically add or remove specific UEs to/from the same service (e.g. a AI-ML federated learning task) when communicating via direct device connection.

Based on user consent and operator policy, the 5G system shall be able to provide means for the network to configure and modify remote UEs’ communication QoS, when a relay UE is involved, e.g., to satisfy end to end latency for proximity-based work task offloading.

NOTE:	for proximity-based work task offloading, the data packet size transmitted over the sidelink and Uu parts of the UE indirect network connection can be different.

Subject to user consent and operator policy, the 5G system shall be able to support configuration of the QoS (e.g., latency, reliability, data rate) of a communication path using direct device connection, e.g., for AI-ML data transfer.

Based on user consent, operator policy and trusted 3rd party request, the 5G system shall be able to support means to monitor the QoS characteristics (e.g. data rate, latency) of traffic transmitted via direct device connection or relayed by a UE, and 5G network expose the monitored information to the 3rd party.

NOTE:  The monitoring information doesn’t include user position-related data.

Subject to user consent, operator policy and trusted 3rd party request, the 5G system shall be able to provide means the network to predict and expose QoS information changes for UEs’ traffic using direct or indirect network connection (e.g., bitrate, latency, reliability).The 5G system shall be able to support a mechanism for a trusted third-party to negotiate with the 5G system for a suitable QoS for direct device connections of multiple UEs exchanging data with each other (e.g. a group of UEs using the same AI-ML service).

Based on user consent, operator policy and trusted 3rd party’s request, the 5G system shall be able to support and provision an aggregated QoS for multiple remote UEs served by a relay UE.

Based on user consent, operator policy and trusted 3rd party’s request, the 5G system shall be able to support configuring specific QoS limitations applied to multiple UEs communicating via direct device connection (e.g. part of a joint AI-ML inference task).

NOTE: the above requirement assumes unicast type of communication.

Subject to user consent, regulation, trusted 3rd party’s request and operator policy, the 5G network shall be able to expose information to assist the 3rd party to determine candidate UEs for data transmission via direct device connection (e.g. for AIML model transfer for a specific application).

NOTE: the information does not include user’s specific positioning and can include QoS information

Subject to user consent, operator policy, regulation and trusted 3rd party’s request, the 5G network shall be able to expose information of certain UEs using the same service to the 3rd party (e.g. to assist a joint AIML task of UEs in a specific area using direct device communication)

NOTE: the information does not include user’s exact positioning information.

* * * Next Change * * * *

## 6.50	Traffic steering and switching over two 3GPP access networks

### 6.50.1	Introduction

The following requirements cover scenarios and functionalities for supporting enhanced traffic steering and switching of a DualSteer device’s user data (for different services) across two 3GPP access networks, assuming the ability to differentiate the two connections for the same device and minimize impacts to CN, O&M or IT systems.

Target scenarios cover two 3GPP access networks belonging to the same PLMN, or between two different PLMNs, or between one PLMN and one PLMN-integrated NPN, over same or different RAT, which can use terrestrial and/or satellite access (including the case of two different satellite orbits). Scenarios may also include traffic steering and/or switching across E-UTRA/EPC and NR/5GC, with anchoring in 5GC.

Traffic policies are intended to be in full control of the home network operator.

The requirements below can apply to different DualSteer device types (e.g., smartphones, IoT, UAV, VSAT devices).

### 6.50.2	Requirements

For the requirements below, the following applies:

- a subscriber with two subscriptions/SUPIs, sharing one subscription profile from the same operator;
- for simultaneous transmission over two networks, a DualSteer device is assumed to include two separate UEs.

#### 6.50.2.1	General

Subject to HPLMN policy and network control, the 5G system shall be able to support mechanisms to enable traffic steering and/or switching of a DualSteer device’s user data (for different services) across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN), assuming data anchoring in the HPLMN and non-simultaneous transmission over the two networks.

Subject to HPLMN policy and network control, the 5G system may be able to support mechanisms to enable traffic steering and/or switching with simultaneous transmission of a DualSteer device’s user data (for different services) across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN), assuming data anchoring in the HPLMN.

Subject to HPLMN policy and network control, the 5G system shall be able to support mechanisms to enable traffic steering and/or switching of a DualSteer device’s user data (for different services) across two 3GPP access networks belonging to two PLMNs, assuming a business/roaming agreement between PLMN operators (if different), data anchoring in the HPLMN and non-simultaneous transmission over the two networks.

Subject to HPLMN policy and network control, the 5G system may be able to support mechanisms to enable traffic steering and/or switching with simultaneous transmission of a DualSteer device’s user data (for different services) across two 3GPP access networks belonging to two PLMNs, assuming a business/roaming agreement between PLMN operators (if different) and HPLMN data anchoring.

NOTE 1: Inter-PLMN requirements can apply also to PLMN-NPN scenarios assuming a PLMN-integrated NPN (NPN hosted by a PLMN or offered as a slice of a PLMN).

For traffic steering and/or switching of user data across two 3GPP access networks, the 5G system shall be able to allow a HPLMN to provide policies and criteria for a DualSteer device to connect to an additional PLMN/NPN, or an additional RAT within the same PLMN.

NOTE 2: The above requirements assume configuration of traffic policies, under HPLMN control or negotiated between the HPLMN and other network operators, considering e.g., user subscription, application/traffic type, service preference, QoS requirements, location, time, UE capabilities, mobility, connectivity conditions.

For any particular service, at any given time, the DualSteer device shall transmit all traffic of that service using only a single 3GPP access network.

#### 6.50.2.2	Mobility and connectivity changes

Subject to HPLMN policy and network control, the 5G system shall be able to support mechanisms to minimize service interruption when switching a DualSteer device’s user data, for one or multiple services, between two 3GPP access networks.

Subject to HPLMN policy and network control, for traffic steering and/or switching of user data across two 3GPP access networks, the 5G system may be able to support mechanisms to change one 3GPP access network to the non-3GPP access network of the same subscription (and vice versa).

#### 6.50.2.3	Other aspects

Subject to home network operator policy and network control, the 5G system shall be able to support traffic steering and/or switching of a DualSteer device’s user data between a NPN and a PLMN, for one or more a DualSteer devices with a NPN subscription accessing NPN services, to meet specific QoS requirements for each device, assuming non-simultaneous transmission over the two 3GPP access networks.

NOTE 2: The above assumes a NPN hosted by a PLMN or offered as a slice of a PLMN, data anchoring in the NPN, and a business/roaming agreement between the PLMN and the NPN operator (if different).

* * * Next Change * * * *

# 9	Charging aspects

## 9.0 	Introduction

This clause addresses the charging aspects of the different services/features described in the present document. Functional service requirements for the respective services/features can be found in respective subclause titles under clause 6.

## 9.1	General

The following set of requirements complement the requirements listed in 3GPP TS 22.115 [11]. The requirements apply for both home and roaming cases.

The 5G core network shall support collection of all charging information on either a network or a slice basis.

The 5G core network shall support collection of charging information for alternative authentication mechanisms.

The 5G core network shall support collection of charging information associated with each serving MNO when multi-network connectivity is used under the control of the home operator.

The 5G core network shall support charging for services/applications in an operator’s Service Hosting Environment.

The 5G core network shall support charging for content delivered from a content caching application.

The 5G core network shall support collection of charging information based on the access type (e.g. 3GPP, non-3GPP, satellite access).

The 5G core network shall support collection of charging information based on the slice that the UE accesses.

The 5G system shall be able to generate charging information regarding the used radio resources e.g. used frequency bands.

The 5G core network shall support collection of charging information based on the capacity and performance metrics.

The 5G system shall be able to support an indirect network connection even when the UE is in E-UTRAN or NG-RAN coverage.

The 5G system shall be able to support mechanisms to differentiate charging information for traffic carried over satellite backhaul.

For service function chaining (see clause 10) the collection of charging information associated to the use of service functions and the chain of service functions requested by third parties shall be supported.

The 5G system shall be able to support collection of charging information for a group of UEs, e.g. UEs of a AI/ML FL group.

The 5G system shall be able to support charging mechanism for multiple UE exchange data for the same service using the direct device connection.

## 9.2	5G LAN

A 5G core network shall support collection of charging information for a 5G LAN-type service based on resource usage (e.g. licensed or unlicensed spectrum, QoS, applications).

The 5G core network shall support collection of charging information for a 5G LAN-type service when a UE joins or leaves a specific private communication.

The 5G core network shall support collection of charging information for a 5G LAN-type service for both home and roaming UEs based on the UE’s HPLMN.

## 9.3	5G Timing Resiliency

The 5G system shall be able to collect charging information based on the timing source (e.g., the source in use, start and stop of source usage).

The 5G system shall be able to collect charging information per UE for use of a timing source (e.g., start/stop time and source used by a UE, timing source used by UE, holdover capability).

The 5G system shall be able to collect charging information on 5G system timing resiliency (e.g., resiliency KPIs, holdover capability, number of UEs using a certain timing source).

The 5G system shall be able to collect charging information per application using 5G timing resiliency, including 3rd party application, (e.g., timing resiliency KPIs, holdover capability, number of UEs using a certain timing source).

## 9.4	Satellite Access

In a 5G system with satellite access, charging data records associated with satellite access(es) shall include the location of the associated UE(s) with satellite access.

NOTE: The precision of the location of the UE can be based on the capabilities of the UE or of the network.

A 5G system with satellite access supporting S&F Satellite operation shall be able to collect charging information per UE or per application (e.g., number of UEs, data volume, duration, involved satellites).

A 5G system with satellite access shall be able to collect charging information for a UE registered to a HPLMN or a VPLMN, for UE-Satellite-UE communication.

A 5G system with satellite access supporting multiple satellite orbit types with different characteristics (e.g., altitude, orbital characteristics, satellite capabilities) shall be able to collect and distinguish charging information related to user data traffic across satellites with different orbit types having different characteristics (e.g., LEO, MEO, GEO).

The 5G system with satellite access supporting connectivity over multiple satellite orbit types with different characteristics (e.g., altitude, orbital characteristics, satellite capabilities) shall be able to collect and distinguish charging information related to the different satellite access traffic carried via a mobile base station relay.

## 9.5	Energy efficiency as a Service Criteria

Charging requirements for “Energy efficiency as a Service Criteria” can be found in clauses 6.15a2, 6.15a3 and 6.15a6 of this TR.

## 9.6	NG-RAN Sharing

The 5G core network shall be able to support collection of charging information associated with a UE accessing a Shared NG-RAN using Indirect Network Sharing, which refers to the resource usage of hosting operator’s core network.

NOTE: When Shared NG-RAN is a shared satellite NG-RAN, the charging information can include the information specific to the shared satellite NG-RAN, e.g. the location of UE, involved satellites.

The 5G network shall be able to collect charging information for a UE accessing a Shared NG-RAN using Indirect Network Sharing in Disaster Condition.

## 9.7 	Minimization of Service Interruption

3GPP system shall be able to collect charging information for a Disaster Inbound Roamer with information about the applied disaster condition.

## 9.8 	Personal IoT Networks and Customer Premises Networks

Charging requirements for “Personal IoT Networks and Customer Premises Networks” can be found in clause 6.38.2.8 of this TR.

## 9.9 	AI/ML model transfer in 5GS

The 5G system shall be able to support charging mechanisms for multiple UEs exchanging data for the same service using the direct device connection (e.g. for AI-ML applications).

## 9.10 	Providing Access to Local Services

Charging requirements for “Providing Access to Local Services” can be found in clause 6.41.2.8 of this TR.

## 9.11 	Mobile base station relays

Charging requirements for “Mobile base station relays” can be found in clause 6.42.2 of this TR.

## 9.12	Traffic steering and switching over two 3GPP access networks

Subject to HPLMN policy and network control, the 5G system shall be able to collect charging information related to traffic steering and/or switching of a DualSteer device’s user data across two 3GPP access networks.

NOTE 1: Charging information should be collected for both 3GPP access networks; in case the two 3GPP access networks belong to different PLMNs, or a PLMN and NPN, a proper business/roaming agreement among network operators is assumed.

* * * End of Changes * * *