3GPP TSG SA WG 1 Meeting #111 	S1-253528

25-29 August 2025, Goteborg, Sweden	(revision of S1-253037r2)

Source:	ESA, Airbus, Echostar

pCR Title:	Pseudo-CR on “8.10 Use case on hybrid NTN and GNSS positioning”

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	Felix Abel, felix.abel@esa.int

Abstract: The Editor’s Note is proposed to be replaced by performance requirements.

1. Introduction

In 3GPP TR 22.870, The use case on hybrid NTN and GNSS positioning includes an Editor’s Note on the KPIs, which needs to be replaced by performance requirements.

2. Reason for Change

The KPIs are currently FFS.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.3.1.

* * * First Change * * * *

## 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[25]	European Union Agency for the Space Programme (EUSPA): "Report on Aviation and Drones – User Needs and Requirements", 2024. (Report on Aviation and Drones - User Needs and Requirements.pdf)

…

[X1]	ETSI TS 103 246-3: "Satellite Earth Stations and Systems (SES); GNSS Based Location Systems; Part 3: Performance Requirements".

[X2]	European Union Agency for the Space Programme (EUSPA): "Report on Maritime and Inland Waterways – User Needs and Requirements", 2024. (Report on Maritime and Inland Waterways - User Needs and Requirements.pdf)

* * * Second Change * * * *

## 8.10	Use case on hybrid NTN and GNSS positioning

### 8.10.1	Description

The ubiquitous availability of position knowledge is paramount to a plethora of use cases and services. GNSS fulfils most of these positioning needs. GNSS systems are typically designed with a global depth of coverage of at least 4 satellites in open-sky conditions to enable standalone use of each constellation. Interoperability and standardisation of GNSS systems on signal structure, power levels, navigation message design and broadcasting relative time-offset has enabled modern smartphones to rely on Galileo, GPS, Beidou and Glonass increasing the number of visible satellites.

Yet, GNSS users can face availability challenges when buildings or natural objects obstruct the LoS view of satellites (such as in urban canyons with partly obstructed sky as in “Annex A.3.2” of [X1]). The increased occurrence of jamming and spoofing event constitutes an additional challenge for continuity and availability of positioning for users. For cold-start standalone GNSS, users may take tens of second or more to obtain a first position fix.

Hybrid operation of NTN and GNSS positioning may enable better accuracies especially in challenging environments thanks to an increased total number of satellites in view. Hybrid operation could further offer improved continuity of satellite-based positioning through frequency diversity of NTN bands and GNSS L-band signals, especially in jamming and spoofing events impeding L-band. Lastly, NTN satellites have the potential of reducing time to first fix, especially in cold-start scenarios by providing a fast data channel for satellite clock and ephemeris data. The target performance of hybrid NTN and GNSS positioning is expected to be aligned with positioning service level 1 and 2 of Table 7.3.2.2-1 in TS 22.261 [14], considering outdoor urban environments and requirements from airborne [25] and maritime [X2] scenarios, respectively.

* * * Third Change * * * *

### 8.10.6	Potential New Requirements needed to support the use case

[PR 8.10.6-1]	The 6G system with satellite access shall be able to provide a positioning service supported by the combination of  NTN and GNSS positioning technologies, including Network-based Assisted GNSS.

[PR 8.10.6-2]	The 6G system with satellite access shall be able to provide positioning service with the following KPIs:

Table 8.10.6-1: Performance requirements for positioning services with satellite access

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| UE with partly obstructed sky | [10] m | [3] m | [95 %] | [1] s | Outdoor rural and urban | Up to [250] km/h | Handheld, vehicle mounted, IoT |
| Airborne and maritime | [3] m | [3] m | [99 %] | [1] s | Outdoor rural | Up to [500] km/h | Airborne or maritime mounted |