3GPP TSG SA WG 1 Meeting #111 	S1-253531

25-29 August 2025, Goteborg, Sweden	(revision of S1-253236r2)

Source:	SoftBank Corp., FirstNet, MITRE, Hytera communications

pCR Title:	Update on 8.12 “Use case on HAPS-based rapid deployable network for public safety and disaster response”

Draft Spec:	3GPP TR 22.870 V0.3.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	Shiro Fukumoto (shiro.fukumoto01@g.softbank.co.jp)

1. Introduction

This contribution proposes an update to 8.12 “Use case on HAPS-based rapid deployable network for public safety and disaster response.”

It (i) replaces Figure 8.12.3-1 with an updated diagram that clearly depicts both fixed-wing and airship HAPS platforms and the associated radio links—HTS (HAPS-to-Satellite), HTH (HAPS-to-HAPS), HTG (HAPS-to-Gateway)—as well as direct-to-device connectivity in the disaster-affected area, and (ii) introduces a new PR to reflect dynamic coverage/quality adaptation in disaster scenarios.

2. Reason for Change

The current figure does not explicitly distinguish HAPS platform types or show that HTS/HTH/HTG are radio links. The proposed figure improves readability and technical fidelity.

Reflect operational situation of HAPS platform’s flight behavior (e.g. loiter/turn patterns) that changes position and orientation, which in turn affects coverage and link budgets.

The proposed PR codifies the capability for dynamic reconfiguration of coverage footprint and service quality driven by (a) first-responder demand and evolving recovery status, and (b) HAPS flight area/behavior—ensuring service continuity for prioritized users in disaster response.

Editorial alignments based on the HAPS explanatory texts proposal (S1-253235)

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.3.1.

* * * FIRST Change * * * *

8.12	Use case on HAPS-based rapid deployable network for public safety and disaster response

8.12.1	Description

This use case focuses on leveraging HAPS, specifically HIBS (base station on board a HAPS), to rapidly establish or restore communication services in areas affected by natural disasters (earthquakes, floods, hurricanes, wildfires) or other emergencies where terrestrial infrastructure is damaged, destroyed, or overloaded. HAPS provides a service link to standard UEs over wide-area (e.g. 50-100 km radius per platform) and can prioritize emergency traffic for first responders. Its stratospheric position makes it immune to ground-level events. Multiple HAPS can be deployed to cover larger areas or provide redundancy.

8.12.2	Pre-conditions

•	A disaster or emergency event has occurred, disrupting terrestrial communications.

•	HAPS platform(s), which can be Heavier-than-Air (HTA) types like fixed-wing aircraft or Lighter-than-Air (LTA) types like airships, are available for deployment or are already airborne near the affected region.

•	Authorized spectrum is available for emergency use.

•	Emergency response agencies and targeted user groups are equipped with compatible devices (standard smartphones or dedicated terminals).

•	A mechanism exists to establish feeder link for the HAPS (e.g. via satellite link (HTS), a connection to an intact ground gateway outside the disaster zone (HTG), or through another nearby HAPS (HTH)).

•	Coordination with relevant authorities (e.g. aviation, emergency management) is established.

8.12.3	Service Flows

- Upon notification/authorization, HAPS platform(s) are deployed/re-tasked to the target area.

- HAPS activates communication payload, establishing service links over the designated area.

- Establish the feeder link (HTS, HTG and/or HTH) The service link between HAPS and UEs is then activated over the designated area.

- Authorized UEs (first responders, public safety devices, potentially public UEs) register and connect to the network via HAPS.

- Voice, data, and potentially location services are provided. HAPS should prioritize traffic for emergency services. HAPS can also provide communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network (e,g., push-to-talk for first responders.)

- Network monitoring and management are performed remotely.

Figure 8.12.3-1 shows a structure of HAPS-based rapid deployable network based on the above flows.

HAPS

HAPS

Satellite

Disinterred area

Core NW

HTS

HTH

HTG

Gateway

Direct access

Figure 8.12.3-1: HAPS-based rapid deployable network

8.12.4	Post-conditions

•	Essential communication services are established/restored in the affected area via HAPS.

•	First responders can communicate effectively.

•	Affected populations can access emergency information or contact help.

•	Network status is monitored, and operational logs are maintained.

8.12.5	Existing features partly or fully covering the use case functionality

•	Public Safety features (e.g. priority access [63], MCPTT [53]/MCData [56]/MCVideo [55]).

•	Location Services (LCS) [61].

•	Basic connectivity, QoS framework.

•	Mobility Management (Handover, Cell Reselection), especially in case the TN network is just partially or locally damaged, some level of mobility between TN and temporary HAPS coverage is expected.

•	Multi-connectivity/redundancy features with regard to multi-link backhaul connectivity.

8.12.6	Potential New Requirements needed to support the use case

[PR 8.12.6-1]: The 6G system shall support communication between UEs under the coverage of base stations onboard HAPS platforms, without the user traffic going through the ground network.

* * * End of Change * * * *