3GPP TSG-SA WG1 Meeting #111 	S1-253043r1

Goteborg, Sweden, 25-29 August 2025

Source:		NICT, Deutsche Telekom, Thales

pCR Title:		Use Case on Enhanced Multi-Access Connectivity

Draft Spec:		3GPP TR22.870 v0.31

Agenda Item:			8.1.5 Ubiquitous connectivity

Document for:		Approval

Contact:			Hikaru Kawasaki (h_kawasaki@nict.go.jp)

Hideshi Murai (hideshi.murai@nict.go.jp)

Abstract: We propose a new use case on enhanced multi-access connectivity.

1. Introduction

This contribution proposes a new use case on enhanced multi-access connectivity to enhance the reliability and/or data rate performance in uncovered or scarcely covered areas, such as rural, remote, indoor, and sparsely populated areas as well as in areas subject to terrestrial networks due to disasters.

2. Reason for Change

Existing features in 5G partially cover the proposed use case but requires evolution in 6G to support enhanced multi-access functionality.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.31.

* * * First Change * * * *

8.x	Use Case on Enhanced Multi-Access Connectivity

8.x.1	Description

Enhanced multi-access connectivity enables ubiquitous connectivity mainly in rural, remote, indoor, and sparsely populated areas as well as in areas subject to terrestrial networks due to disasters, supporting dynamic selection and combination of several access networks (TNs and/or NTNs), potentially with significant performance disparities (e.g., service rate, latency, etc…) and across different network operators for applications such as telehealth, and remote control of robots or drones. The support of multi-access connectivity includes traffic flow control (including the ability to steer traffic flow) across two or more simultaneous sessions to meet performance requirements like high availability, throughput, reliability, low latency, and time jitter stability, which a single operator network may not provide due to coverage limitations, bandwidth constraints, and diverse network deployments.

8.x.2	Pre-conditions

The environment consists of heterogeneous access networks (including TNs and NTNs) in rural, remote, and sparsely populated areas as well as in areas subject to terrestrial networks due to disasters. The system supports access to networks from different operators where feasible, to enhance coverage. Users are authorized to access these networks, and UE supports multi-access functionality for dynamic network selection and traffic flow control across two or more simultaneous sessions.

8.x.3	Service Flows

The UE  combines the multiple (access) networks composed of TNs and NTNs (potentially from different operators) mainly in rural, remote, indoor, and sparsely populated areas as well as in areas subject to terrestrial networks due to disasters, using multi-access functionality to apply traffic flow control across two or more simultaneous sessions considering the performance disparities to meet performance requirements such as network availability, throughput, reliability, low latency, time jitter stability. For example:

- In a telehealth scenario, the multi-access connectivity provides high reliability and bandwidth to support the high quality video communication for remote healthcare.

- In the remote control of robots/drones, the multi-access connectivity provides the necessary high reliability.

- In a heavy data usage service scenario (e.g., streaming or gaming), the multi-access connectivity (e.g., NTN and TN) provides high data rate services and perceived low latency for best user experience.

8.x.4	Post-conditions

The UE achieves ubiquitous connectivity mainly in rural, remote, indoor, and sparsely populated areas as well as in areas subject to terrestrial networks due to disasters through enhanced multi-access functionality, including traffic flow control across multiple network sessions (TNs and NTNs, potentially from different operators where feasible). This ensures the target performance such as network availability, throughput, reliability, low latency, time jitter stability, for applications like telehealth or remote control of robots/drones.

8.x.5	Existing features partly or fully covering the use case functionality

The 5G supports multi-access functionality, as defined in TS 22.261 [14] Clause 6.3.2 (multi-access technologies requirements) and Clause 6.50.2 (traffic steering and switching over two 3GPP access networks).

The ATSSS feature (distribution user traffic over the access technologies in use) in previous releases enables multi-access between a 3GPP network and a non-3GPP network. Enhancements are needed that enable the UE to select, manage, and efficiently provide services over non-3GPP access when the 3GPP access is not available or there is poor coverage.

The DualSteer feature in previous releases aims to enable multi-access support between two 3GPP access networks. This feature in 5G partially covers the proposed use case but requires evolution in 6G to support enhanced multi-access functionality. Enhancements are needed for traffic flow control across TNs and NTNs, such as a mechanism for the operator to manage network selection.

8.x.6	Potential New Requirements needed to support the use case

[PR8.x.6-1] The 6G system shall support means for a UE to use multiple access networks, including TNs and NTNs, which may belong to the same or different network operators. This may also refer to traffic flow control for two simultaneous sessions.

[PR8.x.6-2] The 6G system shall support means for the HPLMN to provide configuration information for a UE to select one or multiple additional networks, including a list of allowed additional networks in priority order. The configuration information may include optional restrictions (e.g., service area, time).

NOTE 1: additional network is the network (PLMN/access technology combination) that a UE may select in addition to the initial network (i.e. an existing network connection).

[PR8.x.6-3] The UE shall only select an additional network which is part of the list and which is different from the current network which was initially selected. If the UE changes the initial network (i.e. moves to a new network), this may result in new configuration information provided to the UE.

NOTE 2:	The additional network(s) might be from the same or a different country than the initial network.

[PR8.x.6-4] The HPLMN shall be able to configure whether the configuration information shall remain valid when the initial network changes and no new configuration is received.

[PR8.x.6-5] While the UE is registered on an additional network it shall periodically attempt to select a higher priority additional network, according to a timer value set by the HPLMN.

NOTE 3:  If no additional network is available, the UE can periodically re-attempt network selection.

[PR8.x.6-6] Based on operator policy, the 6G system shall enable the UE to connect to the supported access network and access services over 3GPP access and/or non-3GPP access independently.

[PR8.x.6-7] Based on operator policy, the 6G system shall be able to optimally distribute user traffic over the access technologies in use.

End of CHANGES