3GPP TSG SA WG 1 Meeting #111 	S1-253195r1

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	CATT, CSCN, China Unicom

pCR Title:	Use Case on 6G Satellite Backhaul Service

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Qiyao Xiang xiangqy@chinasatnet.com.cn

Abstract: This contribution proposes a use case about enabling satellite backhauling based on 6G networks with satellite access.

1. Introduction

The satellite-based backhauling plays a critical role in the mobile market, as it can not only extend coverage to underserved regions with limited terrestrial infrastructure (e.g. Africa, South America), but also enhance network resilience in urban areas by alleviating congestion and solving disaster-induced failures. With the trend toward integrated satellite and terrestrial network in 6G, satellite backhauling will increasingly become a necessary backhaul option, offering unprecedented flexibility for 6G networks and improving the cost efficiency of the satellite-based 6G infrastructure.

However, existing satellite backhauling is independently of the 3GPP system which hinders coordination in optimizing the route and the reliability of E2E connectivity. Moreover, maintaining two independent systems (satellite backhaul and 3GPP satellite) increases the CAPEX and OPEX for satellite operators, who aim to maximize the utilization of satellite resources and improve Return on Investment (ROI).

Therefore, 6G can be expected to integrate satellite backhauling into 3GPP system to enable flexible satellite based transmission and stimulate potential new business models for the MNOs supporting satellite access. This contribution illustrates the motivation of enabling satellite backhauling based on 3GPP satellite-as-a-service and the use case on serving various types of 6G users who obtain 3GPP services via satellite access.

2. Reason for Change

Introduce a use case about satellite backhauling based on 3GPP satellite as a service.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.870 v0.3.1

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[8.X-1]	Avanti Space: White Paper - Connectivity Through Backhaul (https://www.avanti.space/wp-content/uploads/2022/05/Satellite-Backhaul-whitepaper-2022.pdf)

[8.X-2]	Y. Turk and E. Zeydan, "Satellite Backhauling for Next Generation Cellular Networks: Challenges and Opportunities," in IEEE Communications Magazine, vol. 57, no. 12, pp. 52-57, December 2019

* * * Next Change * * * *

8.X	Use case on 6G satellite backhaul service

8.x.1	Description

Satellite backhauling first emerged in the early 2000s as a solution for connecting remote cellular base stations to the backbone network when terrestrial connections were unavailable or unreliable. As cellular networks have evolved from 2G to 5G, the satellite backhauling technology has advanced significantly – starting with GEO satellites for 2G voice/SMS to High Throughput satellites (HTS) for 3G/4G data and more recently, to LEO satellites for low-latency data service.[8.X-1] Today, the satellite based backhauling plays a critical role in the mobile market, as it can not only extend coverage to underserved regions with limited terrestrial infrastructure (e.g. Africa, South America), but also enhance network resilience in urban areas by alleviating congestion and solving disaster-induced failures.

Figure 8.X-1 Legacy Satellite Backhauling with Traditional Ground Stations

As outlined by ITU-R [27], IMT-2030 is expected to connect humans, machines and various other things together with further development of ubiquitous connectivity and enhanced resilience in design, deployment and operation. With the trend toward integrated satellite and terrestrial network in 6G, satellite backhaul will increasingly become a necessary backhaul option, offering unprecedented flexibility for 6G networks and improving the cost efficiency of the satellite-based 6G infrastructure. However, existing satellite backhaul is treated as an independent system from 3GPP system. The isolation between each system has caused a series of issues.

- Extra CAPEX and redundancy: As [8.X-2] mentioned, the ownership of dedicated satellite equipment (VSAT modem and hub) used for satellite backhaul is obligatory to be MNOs (e.g.in Turkey). If the mobile traffic is carried over satellite equipment owned by 3rd party satellite providers, the providers need to have customized mobile communication licenses by local government agencies. Such regulations require extra investment from MNOs or satellite operators to enable satellite backhauling. Moreover, the satellite backhaul network (using DVB) and 3GPP satellite network are architecturally decoupled, resulting in duplicated onboard components (e.g. transponders) on the satellites capable of satellite access and backhauling, or additional single-purpose satellites, or redundant ground infrastructures.

- Inefficient resource management: for the satellites supporting both access and backhauling, the independent operations of two payloads may lead to resource inefficiency issues such as the spectrum interference when using overlapping Ka/Ku bands, static bandwidth partition without dynamic allocation, etc. The lack of collaboration in resource allocation may reduce the chance to ensure the user experience of 3GPP services carried on the satellites especially when the backhauling congestion could be solved by using the bandwidth provisioned for 3GPP satellite access, or further, sharing the satellite link used for wireless access. Compared with traditional terrestrial transmission, the transmission cost per bit via satellite links is higher. Thus, the sharing of the satellite links either among different kinds of services (e.g., access, backhauling) or multiple operators can be a promising business model to maximize resource utilization and reduce OPEX.

- Inflexible network topology and backhauling: 6G base stations mounted on various locations (e.g. ground, vehicles, vessels, drones, HAPS and satellites) exhibit distinct mobility characteristics, which raise demands for switching the service link of the satellite backhauling to adapt to their specific mobility patterns as illustrated in Figure 8.X-1. Besides, the movement of NGSO satellites used for backhauling intensifies the demand for dynamic link switching. However, one satellite may have only one feeder link to a traditional ground station (e.g. using DVB) located in a specific area, limiting the possibility to choose a suitable ground station or feeder link for optimal backhaul link especially adapting to service link switching. Moreover, in scenarios such as disaster areas or remote areas like oceans and deserts, there is a strong demand to deploy one ground station for temporary usage, in order to quickly setup a localized network enabling communications for UAVs and emergency devices. However, the legacy ground stations that rely on the provisioning cannot easily meet such plug-and -play demand.

- Lack of coordination for the reliability and resilience of E2E connectivity: traditional terrestrial backhauling is generally assumed to be reliable and stable in the design 3GPP systems. However, satellites mobility and inherent instability of feeder links may compromise the reliability and continuity of the backhaul link, which need to be taken into account to ensure reliable E2E connectivity. Store and forward operation defined in 5G could be treated as a workaround to mitigate the impact of instable satellite links on the service delivery but its applicability is limited to the latency-insensitive service. Currently, the satellite backhauling is managed independently, and there are no standardized mechanisms to coordinate the satellite backhauling with 3GPP network. The lack of coordination hinders the rapid adaption of backhaul links such as replacement, re-configuration and further impedes the optimization of E2E QoS, or the fulfilment of divergent backhaulquality requirements (e.g. via one or multiple links, by dynamic configuration).

6G can be expected to solve the above pain points by the integration of satellite backhaul into 6G system as Figure 8.X-2. And this may also stimulate a new business model for the MNOs supporting satellite access, to offer 6G satellite backhaul service to other MNOs based on the same satellite infrastructures.

This use case illustrates an example that the 6G system serves various types of 6G users using satellite access through 6G satellite backhaul.

Figure 8.X-2 6G Satellite Backhauling based on 3GPP Satellite

8.x.2	Pre-conditions

As Figure 8.X-3, Network Operator OpSAT provides 6G backhauling service via its deployed satellite based 6G network (6G SAT NW), which may include multiple orbit types of satellites (e.g. GEO, MEO, LEO) and distributed ground stations(e.g. GND Station#1) registered for the usage.

Network Operator OpMNO#1 and OpMNO#2 have agreements with OpSAT for backhauling services

- They can provide satellite access to their individual subscribers via dedicated satellite-based RANs or shared satellite-based RANs with OpSAT to ensure ubiquitous connectivity.

- They have individual ground core network (6GC#1 for OpMNO#1 and 6GC#2 for OpMNO#2).

OpMNO#2 has owned a vehicle mounted ground station with the subscription of OpSAT for emergency usage.

Bob (with UE#1) is a travel blogger, and subscribes to OpMNO#1for the global reliable communication services.

A UAV (with UE#2) is used for remote search and rescue, and subscribes to OpMNO#2 for 3GPP services (e.g. communication service, sensing service and computing service).

Figure 8.X-3 Serve 6G users with 6G Satellite Backhauling

8.x.3	Service Flows

Based on the agreement with OpMNO#1 and OpMNO#2, OpSAT enables the backhauling services for 6GC#1 and 6GC#2 through the ground stations within their respective serving areas, establishing dedicated backhaul links for each operator’s network.

Sub-Scenario A: Efficient service delivery via dynamic backhauling links

- Bob traveled to Tanzania to photograph the Great Migration of animals. Upon arriving at East Africa, UE#1 camped on base station on 6G SAT(GEO) and connected to 6GC#1 through the satellite backhaul link#1, enabled by 6G SAT(GEO) and GND Station#1.

- To report the safety, Bob initiated a voice call with his family using UE#1. The voice traffic was identified and routed to 6GC#1 via the satellite backhaul link#1 regarding the operator’s policy and the coordination of 6GC#1 and 6G SAT NW.

- When Bob switched to a video call, the change was identified and the video traffic was routed to 6GC#1 through the satellite backhaul link#2, enabled by 6G SAT(GEO), 6G SAT(LEO) and GND Station#1 regarding the operator’s policy and the coordination of 6GC#1 and 6G SAT NW.

- During the video call, UE#1 can maintain the user experience with minimum interruption although the satellite backhaul link#2 keeps changing due to the movement of LEO satellite.

- After one day of photography, Bob uploaded all the photos and videos to the remote cloud using UE#1. The data traffic was identified and successfully routed to 6GC#1 through the satellite backhaul link#2 regarding the operator’s policy and the coordination of 6GC#1 and 6G SAT NW.

Sub-Scenario B: Resilient 3GPP services via temporary backhaul links

- UAVs were launched to a mountain area to search for the lost travelers. Upon reaching the target area. UE#2 connected to 6GC#2 through the satellite ing link#3, enabled by 6G SAT(GEO), 6G SAT(LEO) and GND Station#1 regarding the policy and the coordination of 6GC#2 and 6G SAT NW.

- UAV and the remote control platform exchanged the information of the tasks, the control and command messages and the real-time imaging data for localizing the travelers within the expected time via the satellite backhaul link#3.

- When GND Station#1 met the failure issue caused by the local flood, GND station#2 on an emergency vehicle was powered on and quickly registered to 6G SAT NW for backhauling service. The satellite backhaul link#4 was rapidly setup enabled by 6G SAT(GEO), 6G SAT(LEO) and GND Station#2, and continually served UE#2 for the rescue tasks.

8.x.4	Post-conditions

Thanks to the backhauling service, OpMNO#1 can provide Bob with reliable communication services globally in a cost-efficient manner. OpMNO#2 can support UAVs to complete the search-and-rescue tasks through flexible and resilient backhauling links.

8.x.5	Existing features partly or fully covering the use case functionality

1. Wireless Self-backhaul

TS 22.261clauses 6.12 has specified a series of requirements about wireless self-backhaul for 5G systems as below.

The 5G network shall enable operators to support wireless self-backhaul using NR and E-UTRA.

The 5G network shall support flexible and efficient wireless self-backhaul for both indoor and outdoor scenarios.

The 5G network shall support flexible partitioning of radio resources between access and backhaul functions.

The 5G network shall support autonomous configuration of access and wireless self-backhaul functions.

The 5G network shall support multi-hop wireless self-backhauling.

NOTE 1: This is to enable flexible extension of range and coverage area.

This feature mainly focuses on the terrestrial network, and intends to solve the challenge caused by the increased density of terrestrial access nodes by the functionalities like dynamic spectrum sharing, relays/IAB, SON and so on. However, satellite backhauling has distinct challenges, not addressed by the above solutions.

2. Satellite Backhaul

TS22.261 also specifies the requirements about routing, QoS handling and charging for 5G system using satellite backhauling as below.

The 5G network can also support multiple wireless backhaul connections (e.g. satellites and/or terrestrial), and efficiently route and/or bundle traffic among them.

The 5G system shall support a mechanism to determine suitable QoS parameters for traffic over a satellite backhaul, based e.g. on the latency and bandwidth of the specific backhaul.

NOTE 3: The case where a backhaul connection has dynamically changed latency and/or bandwidth needs to be considered.

The 5G system shall be able to support mechanisms to differentiate charging information for traffic carried over satellite backhaul.

However, the 5G system can just adapt to the impact of used satellite backhaul passively but cannot coordinate the backhaul system to be adjusted to improve the end-to-end connectivity. Moreover, one bottleneck of flexibility is ground station, which has not been taken into account in 5G.

8.x.6	Potential New Requirements needed to support the use case

[PR 8.x.6-1] Subject to operator’s policy, the 6G system shall be able to enable satellite backhaul using 3GPP access technology to connect the satellite and ground station serving 6G core networks.

NOTE: The 6G network with satellite backhaul and the onboard network enabling satellite backhaul can belong to the same operator or different operators under SLA.

[PR 8.x.6-2] Subject to the regulatory requirements, the 6G system shall support a mechanism to authenticate and authorize a ground station provisioned for satellite backhaul registered to the satellite onboard network for providing backhauling.

[PR 8.x.6-3] Subject to the regulatory requirements and operator’s policy, the 6G system shall support a mechanism to select a satellite backhaul link for the traffic transmitted to the ground core network, taking into account the satellite capabilities (e.g., latency, data rate) and/or the associated ground station’s location and availability.

[PR 8.x.6-4] Subject to the regulatory requirements and operator’s policy, the 6G system shall support a mechanism to maintain user experience with minimum interruption when the serving satellite backhaul link changes caused by the movement of satellites and/or the switching of ground stations serving 6G core networks.

* * * End of Changes * * * *