3GPP TSG SA WG 1 Meeting #111 	S1-253247

25-29 August 2025, Goteborg, Sweden

Source:	ZTE Corporation, China Unicom

pCR Title:	Pseudo-CR on addressing EN in clause 9.5

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.6

Document for:	Approval

Contact:	Lijuan Chen, chen.lijuan@zte.com.cn

Abstract: This contribution proposes to addressing issue #119, EN in in clause 9.5

1. Introduction

<Introduction part >

2. Reason for Change

The proposed KPI values were submitted in pCR S1- 252490 and discussed during SA1#110.  An EN with “FFS” was introduced because there was no enough time for further discussion and clarification.

This  pCR tries to address the EN with supporting reference. The purpose is to serving the scenario of getting low-complexity and low-latency encoding and keeping as much as details of video stream for 4K, 8K solutions at camara side to ensure immersive experience.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

## 9.5	Use case on collaborative service in multi-site involved immersive communication

### 9.5.1	Description

Compared to AR/VR services under existing 5G networks, 6G immersive XR services aim to provide an ultimate user experience through capabilities such as ultra-high resolution, high frame rate, wide color gamut, high dynamic range, wide field of view, and advanced encoding/compression techniques. It supports more natural interaction methods (e.g. voice interaction, gesture interaction, head interaction, eye tracking) to achieve complex perception of users and environments. The 6G system and service platform need to collaborate to complete functions such as rendering, synchronization, encoding, distribution, storage and management of immersive XR services, and support real-time transmission and processing of panoramic video data, field of view video data, etc.

6G immersive XR services present new challenges for platforms, networks and terminals. Based on 3GPP R17 XR project research, current XR service data rates are 30/45Mbps downlink and 10 Mbps uplink for AR scenarios; future requirements with increased video resolution and frame rate can reach Gbps levels. While data volumes increase, immersive XR services have data characteristics of variable packet size, non-integer period, jitter and multiple streams. Concurrent traditional data flows (signaling flows, audio streams, video streams) also exist in the network. In addition to stronger computing capabilities, ensuring real-time transmission of relevant data within the 6G system between various processing nodes is also a key aspect. The timeliness of data transmission will impact the success rate and quality of service, so 6G networks must ensure efficient transmission of large volumes of data to maintain service level agreements

This case demonstrates the 6G network’s capability to provide computing service for enabling a multi-site involved immersive communication scenario.

### 9.5.2	Pre-conditions

- Alice, Bob and Lisa, Tom are customers of Network Operator A, a 6G service provider.

- On New Year's Eve, a public event will jointly held across two cities - City A and City B. Participants had the option to attend in-person at the event venues for massive gatherings, or experience an immersive viewing through different presentation methods (e.g. outdoor large screens, mobile phones/pads, VR/AR glasses)  to feel as if present.

- Alice attended on-site at the City A venue. Bob attended on-site at the City B venue while Tom watched the event in the bus from City B to City A. Lisa experienced the event remotely from her home in City C through a wireless VR device.

- To meet the needs of multi-dimensional perspectives and long-distance follow-up shooting, the event organizer chooses wireless cameras to capture video streams. For saving the time of encoding at camera side and keeping as much as details of video stream, low-compression and even uncompressing ([161], [64], [67]) are selected for cameras on-site. There are also multiple sensors on site collecting signals. Network Operator A’s network provides wireless transmission for video streaming and sensors data.

- Besides providing communication service to enable this use case, Network Operator A's 6G network also provides computing service to the organizer’s platform for stream processing to fulfill demands on immersive presentation. Rendering, encoder/decoder, and other capabilities are also pre-deployed in the network on the event venues in City A and City B.

### 9.5.3	Service Flows

Sub-scenario 1 (Immersive Program Viewing):

For example, two singers perform the same song on stages located in City A and City B respectively. Through the immersive viewing service, Alice, Bob and Lisa, Tom - attending from different physical locations using different means - perceive the experience as if the two singers were performing on a single stage together.

The following Figure 9.5.3-1 illustrates stream flows between City A and City B venues during the live interaction show.

Figure 9.5.3-1: Stream flows between two venues during the live interaction show

NOTE:	Not all entities and control message are shown in above figure for the sake of simplicity.

- 1, 1`: Local performances are presented to on-site audiences in City A and City B event venues. Meanwhile, multi-dimensional data (such as audio, video, stage view, audience seating view, 3D models, motion, etc.) from the both venues is being acquired. And sending to on-site core network.

- 2, 2`: The core network selects corresponding processing nodes (e.g. core network nodes with computing resources, edge server) to process data (such as data cleaning, compression, encoding, decoding or rendering) and routing data traffics between processing nodes based on the work load of the nodes and computing service requirement received from organizer’s platform.

- 3, 4: The rendered data from City B is being transmitted to City A through the 6G network and is holographically presented to the on-site audience.

- 3, 4`: In the opposite direction, the rendered data from City A is being transmitted to City B through the 6G network and is holographically presented to the on-site audience.

- The 6G network also needs to collaborate with remote audience service VR application servers to render and fuse the data processed from Cities A and B into a global stage scene data and then distribute it to wireless VR devices located in different locations for remote audiences.

- All these data are used to reconstruct through immersive services and presented via Lisa’s wireless VR device.

- During Tom’s bus moves from City B to City A, Tom can keep receiving the render data from 6G network with the requested quality of immersive presentation on VR devices, and enjoy the live show as he is in the event venue.

Sub-scenario 2 (regular Communication Services Remain Unaffected):

Throughout the duration of the event, audiences' regular communication services such as making phone calls, sending text messages, and IMS multimedia services are unaffected.

- Alice shares her experience at the event venue with friends through a video call, and the video interaction remains smooth without issues like audio-video desynchronization or stuttering.

- Bob successfully receives congratulatory New Year greeting messages sent by his friends from all the country, without delays.

- A phone call from Lisa's friend reaches her successfully, and the voice calling service quality does not degrade.

During this event, there are substantial concurrent up-streams and down-streams of multiple data types (user interaction sensing data, AI algorithm data, AR/VR interaction data) which poses challenges for 6G networks. 6G networks must not only function as a data conduit but also enable required real-time data processing operations to ensure user experience.

### 9.5.4	Post-conditions

- Alice, Bob, Tom and Lisa have an equivalent viewing and interactive experience during the event.

- Alice, Bob, Tom and Lisa's regular communication services are unaffected during their participation in the event.

### 9.5.5	Existing features partly or fully covering the use case functionality

Currently, communication requirements as described in TS 22.261 [14] clauses 7.1, 7.6.1 and 7.10 are provided. And it is assumed that there are always sufficient computing resources at the selected one node for running rendering server. 5G network lack mechanisms for intermediary data for collaborative tasks in large-scale rendering.

An "Audio and Video Production" use case which uses HD and UHD resolutions deployed over 5G system is introduced in TS 22.104 [64]. Some normative requirements related to support of media service are in TS 22.263 [67].

### 9.5.6	Potential New Requirements needed to support the use case

[PR 9.5.6-1] Subject to operator’s policy and regulation, the 6G network shall be able to provide computing service controlled by the core network of 6G system to an authorized third-party for supporting rendering.

[PR 9.5.6-2] Subject to operator’s policy, the 6G network shall be able to maintain user experience (e.g. for immersive communication) with minimum interruption when the selected computing resources changes within the Service Hosting Environment used for the computing service for XR rendering e.g. during UE mobility.

[PR 9.5.6-3]: 6G system shall be able to support immersive media content production with the KPI requirements summarized in below Table 9.5.6-1.

Table 9.5.6-1: Performance requirements for immersive media content production via wireless link

| Use Cases | Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate (UL) | Service bit rate: user-experienced data rate (DL) | Reliability | # of Cameras | Camera Speed | Service Area |
| Immersive media content production via the wireless link | [100ms] (note 3) | (Note 1) [2.65 Gbps] (4K, 60fps, 10 bits per color, compression ratio of 4) [1.1 Gbps] (4K, 60fps, 10 bits per color, compression ratio of 10) [7.96 Gbps] (8K, 60fps, 10 bits per color, compression ratio of 6) [4.8 Gbps] (8K, 60fps, 10 bits per color, compression ratio of 10) | N/A | [99,99 %] | 4 (Note 2) | Stationary, Pedestrian | 30 m x 30 m |
| NOTE 1: This bit rate is assuming to use the JPEG-XS [9.5-1] pre-compression encoding method to ensure both high-quality images and high encoding efficiency, while also realizing lower encoding delays [161]. Which is intended as a targeted value per camera and not a strict requirement. NOTE 2: 4 cameras are used in simple stage [9.5-2], more cameras may be needed for condition of larger number of performers, with complex stage and lighting conditions. It depends on the deployment. NOTE 3: One-way delay that is from camera to holographic player. |  |  |  |  |  |  |  |

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[9.5-1]	https://en.wikipedia.org/wiki/JPEG_XS

[9.5-2]	https://www.bbc.co.uk/rd/articles/2024-09-immersive-augmented-virtual-reality-metaverse-3d