3GPP TSG-SA WG1 Meeting #111 	S1-253293r2

25-29 August 2025, Goteborg, Sweden	(revision of S1-253293, S1-252861)

Source:	     Huawei, HiSilicon, China Telecom, TOYOTA, China Mobile, China Unicom,

KPN, UIC, Turkcell, NEC, Turk Telekom

New use case title:	New use case on network-based intelligent assistance for autonomous driving

Draft TS/TR:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	Shuang ZHANG, zhang.shuang2 AT Huawei DOT com

Xu Xia, xiaxu AT chinatelecom DOT cn

Menghan Yu, yumh1 AT chinatelecom DOT cn

Abstract: This contribution proposes a new 3GPP service – intelligent assistance service in the context of assisting safe navigation and autonomous driving. The general assistance is provided by an operator’s 3GPP network-native intelligent assistant (i.e., an AI Agent), whose role includes to interpret Intent-based navigation queries received from the UEs, and in response provide specific services (e.g.: to provide occluded-view assistance, assist safe routing, etc.) from the 3GPP network, by collecting and processing the 3GPP sensing data, and by loading and executing various AI algorithms available (e.g. stored in a dedicated AI toolbox).

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

<Skip>

[257]	3GPP TS 28.538: “Management and orchestration; Edge Computing Management”.

[1x]	Industry Report: Global ADAS And Autonomous Driving Components Market. Emergen Research (URL: https://www.emergenresearch.com/press-release/global-adas-and-autonomous-driving-components-market)

[2x]	Industry Report: Autonomous Vehicle Market Size, Share & COVID-19 Impact Analysis, By Level (L1, L2, & L3 and L4 & L5), By Vehicle Type (Passenger Cars and Commercial Vehicles), and Regional Forecast, 2023-2030. Fortune Business Insights. URL: https://www.fortunebusinessinsights.com/autonomous-vehicle-market-109045

[3x]	Market Analysis Report: Automotive Artificial Intelligence (AI) Market Size, Share & Trends Analysis Report By Component, By Level Of Autonomy, By Technology (Machine Learning, Natural Language Processing), By Vehicle Type, By Region, And Segment Forecasts, 2023 - 2030. Grand View Research (URL: https://www.grandviewresearch.com/industry-analysis/automotive-artificial-intelligence-market-report)

[4x]	5GAA C-V2X Use Cases and Service Level Requirements Vol II. (URL: https://5gaa.org/content/uploads/2020/10/5GAA_White-Paper_C-V2X-Use-Cases-Volume-II.pdf)

[5x]	Vehicle-to-Network-to-Everything (V2N2X) Communications; Architecture, Solution Blueprint, and Use Case Implementation Examples. 5GAA Automotive Association. (URL: https://5gaa.org/vehicle-to-network-to-everything-v2n2x-communications-architecture-solution-blueprint-use-cases/)

* * * Second Change * * * *

# 3	Definitions, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

intelligent assistance service: a 3GPP service to help subscribers and third-party applications perform their tasks or services, e.g., using an AI Agent.

NOTE: 	For example, intelligent assistance service in the context of autonomous driving can be to support collision avoidance, parking assistance, emergency trajectory alignment, automated intersection-crossing, etc.

* * * Third Change (new text) * * * *

## 6.x	Use case on network-based intelligent assistance (e.g., for autonomous driving) by a network-native AI Agent

### 6.x.1	Description

The business market for solutions that provide navigation and assistance to traffic participants, particularly in smart cities is growing rapidly. For example, the overall market of Advanced Driving Assistance Systems (ADAS) is estimated to grow from USD 40.98 billion in 2022 to USD 186.29 billion in 2032 with a CAGR of 16.3% [1x]. Furthermore, the global autonomous vehicle market size was valued at USD 1,921.1 billion in 2023 and is projected to exceed USD 13,632.4 billion by 2030, exhibiting a CAGR of 32.3% during the forecast period [2x]. Finally, the global automotive AI market size, valued at USD 2.99 billion in 2022, is projected to grow to USD 14.92 billion in 2030 with a CAGR of 22.7% [3x]. These very promising forecasts suggest that AI-driven navigation support for various traffic participants (vehicles, cyclists, pedestrians, etc.) could become a good business opportunity for 3GPP operators.

For a range of applications related to autonomous driving (e.g. taxis, consumer vehicles, transportation delivery, etc.), the fleets will potentially have service providers involved in testing, mapping, driver assistance, cloud services, and communication. To them, the 3GPP operators have the opportunity to provide network-based wide-area intelligent services and capabilities, integrated in 3GPP network and in an efficient, collaborative, and dynamical way.

There exist several advantages of providing the general intelligent assistance service via the 3GPP network:

- Through sensing capabilities, 3GPP networks have timely access to unique and relevant information about the wide-area physical environment (e.g. terrain features, traffic patterns, weather characteristics). Such data together with the 3GPP network information (e.g. 3GPP service provisioning data, network configuration and deployment information, O&M data, subscription data, 3GPP monitoring data, 3GPP data analytics results) are not available at 3rd party clouds or at UEs. Therefore, a 3GPP network has the advantage in training and inferencing AI/ML models using these data to provide improved and timely assistance services.

- Thanks to the wide-area connectivity and the capabilities to perform distributed AI tasks (e.g., distributed AI/ML model training and inference), the 3GPP network can perform various tasks more reliably and efficiently than UEs. For example, the 3GPP network can use a split ML model to process jointly multi-modal data from its distributed sensors, in order to produce a real-time unified representation of the environment that is needed for this type of use cases.

- By processing data closer to end users in the Service Hosting Environment and/or in 3GPP core network, the 3GPP network can offer reduced service latency and enhanced data privacy compared with purely cloud-based solutions. This is crucial for real-time and privacy-sensitive applications (e.g., for autonomous driving, collaborative robotics, in privacy-restricted areas).

- Providing intelligent assistance services in some usage scenarios (e.g., autonomous driving, collaborative robots) requires integration of communication, AI, and sensing, which can be realized by the 3GPP network in a self-contained way.

Even though this use case is explained in the context of autonomous driving, the intelligent assistance service is based on a general set of capabilities of the 3GPP network that is applicable to support a wide range of applications and industry sectors. In the context of this use case, the intelligent assistance service is provided to the vehicle’s driving application (e.g., provided by the OEM manufacturer) which is responsible for taking the final decisions.

Figure 6.x.1-1 illustrates an example UE integration in vehicle’s onboard control system. The integrator can be the very provider of the on-board control system for car manufacturers. The integrator has full control of what is onboard of the vehicle, and is responsible for enabling UE’s access to vehicle’s onboard sensors, etc. Based on [5x], V2X App exchanges C-ITS messages with V2X AS or with other V2X Apps; OEM App (which integrates services from OEM AS into vehicle) is provided by OEM manufacturer, the driving application (which may be the “Driving Automation System” in [5x]) in the vehicle interacts with the V2X UE via OEM App. Additionally, as the OEM App has access to vehicle’s Human-Machine interface (HMI) of vehicle’s infotainment system, the driving application can additionally interact with the driver through HMI if needed.

Figure 6.x.1-1: An example of the intelligent assistance service for autonomous driving.

NOTE: The vehicle’s onboard control system shown in grey is out of 3GPP scope and for illustrative purposes only.

Categories of intelligent assistance services offered by the 3GPP system:

The intelligent assistance service of the 3GPP network may (based on what is expressed in the Intent) involve providing various more specific types of services (e.g., related to Categories 1, 2, and 3 described below) to the driving application and/or the driver. Each of these specific services are characterized by their own requirements in terms of processed input data, management complexity, radio and computational resources. Based on these different requirements, the specific services could be grouped into three categories, e.g.:

- Category 1:  the network assistance is done based on (i) subscriber vehicle’s on-board sensing information and (ii) available 3GPP network information. This category of services focuses on improving the vehicle’s sensing capabilities. In this category of services, the 3GPP network provides selected algorithms and trained AI/ML models to the vehicle for local inferencing using the vehicle’s sensor data. The provided algorithms could add new sensing capabilities by extracting and processing new features from the vehicle’s available sensing data, or augment the existing sensing capabilities when provided models are better suited for local driving environment and conditions (e.g. terrain features, heavy snow, etc.). As detailed further, such AI/ML models and algorithms could be obtained, e.g., by re-training general AI/ML models (e.g. from OEM, 3rd parties) with wide-area 3GPP sensing data and 3GPP network data. When it comes to deployment of the algorithms at the vehicle, the 3GPP network is still responsible for monitoring the algorithms’ performance to ensure that they meet the pre-defined performance specifications (processing latency, accuracy, reliability, etc.). This can be done, e.g., by sending periodically performance analytics data from the UE to the 3GPP network for evaluation such as comparing the local model’s output with the high-quality results available at the 3GPP network. Additionally, compared with services of Category 2 and 3 described below, this category of services offers relatively limited assistance, but introduces very low operational costs to the 3GPP network operator because the vehicle bears the cost of running the algorithms locally.

- Category 2: the network assistance is done is based also on (iii) 3GPP network-based sensing data. In this category of services, the 3GPP network uses its own sensing capabilities to provide additional assistance to the vehicle (e.g., occluded-view assistance, infrastructure assisted environmental perception, etc.), possibly via UE-3GPP collaborative or split inference. This category of services offers more comprehensive assistance compared with Category 1 services, but it introduces moderately increased costs to the 3GPP operator (e.g. using and maintaining 3GPP sensing infrastructure, performing inference using sensing data, etc.).

- Category 3: different from services in Cat.2, in this category the network assistance is done also using (iv) 3GPP network knowledge databases and external data sources (e.g., Internet, public databases). The 3GPP network provides assistance by solving complex tasks requiring access to external data (e.g., advanced routing which requires accessing information about traffic conditions in a big city and executing best-path optimization algorithms via executing training and inferencing of AI/ML models). This category of service capabilities introduces the most comprehensive and reliable assistance, but introduces higher costs for the 3GPP operator (maintaining and updating network with external knowledge databases, processing much larger quantities of data).

Whenever a specific service requires providing an AI/ML model or algorithm for local inferencing, the integrator (with full control of the vehicle on-board control system) of a car manufacturer is responsible for integrating 3GPP intelligent assistance service (based on AI inferencing locally done in V2X UE) with the driving application in the vehicle.

For Category 2/3, the existing V2X framework and V2X-related specifications can be followed, such as architecture enhancements work in SA2 (e.g. TS 23.287), and SA6 V2XAPP (e.g. TS 23.286 on application layer support aspects). For the intelligent assistance service in this use case, the collaborative inference results (e.g. intermediate results from the UE) are further processed/combined at the AI/ML endpoint in the 3GPP network (or in Edge Compute Domain), before being fed (as a new source input) to the autonomous driving application servers (in the cloud side, managed by OEM manufacturer) that will further interact with the driving application in the vehicle.

As 3GPP Network controls delivery of 3GPP services, service requests shall be received here are parsed and acted on by performing authentication, authorization, policy control, various management (session, mobility, connectivity to services in DNs, etc.), etc. To enable the 3GPP intelligent assistance service, the objectives/intention of Intent-based service requests are received by an AI Agent in the 6G Network. The AI Agent interprets the Intent, uses its knowledge of 3GPP network (available services, network capabilities, specific services provided by the fine-tuned AI/ML models saved in the 3GPP CN model pool) to reason and plan specific services falling in Cat.1/2/3.

As a number of 3GPP services (e.g. communication, sensing, AI inference, etc.) are needed to support objectives expressed in the Intent and there is dependency among the constituent 3GPP services as needs and conditions change, the 6G network, if with AI Agent, would need to in runtime orchestrate, configure, instantiate and adapt these 3GPP services dynamically to provide the desired service and experienced to the subscribers or the third-party applications.

Main service components and their functionalities in the 3GPP network:

AI Toolbox – a warehouse of various pre-trained AI/ML models, datasets, and algorithms which are stored by the 3GPP operator in the 3GPP network, and which are used to provide specific services (e.g., related to Categories 1, 2, 3). This may include, e.g., an AI/ML model fine-tuned for object detection in snowy conditions using a vehicle’s front camera, another AI/ML model for object detection using vehicle’s radar data, a dataset of environment sensing data collected by the 3GPP operator, a shortest-path search algorithm, a calculator, etc. Each of these models and algorithms are verified (by the 3GPP operator or a 3rd party company) to meet pre-defined performance specifications such as processing latency, inference accuracy, reliability, etc. The 3GPP operator can obtain these AL/ML models and datasets by (i) purchasing fine-tuned AI/ML models and datasets from a 3rd party source, or (ii) re-training general AI/ML models and collecting datasets using own environment sensing and network data (e.g. any meta-data in the 3GPP network, including the configuration data of the deployed 3GPP services and 3GPP network, subscriber data, 3GPP network analytics, information from external systems but stored in the 3GPP network, etc.). Furthermore, the 3GPP operator decides which specific services should be supported by the AI Toolbox, depending on the local market potential.

intelligent assistant – an AI Agent in the 3GPP network that understands the Intent and uses an AI interface to assist with digital tasks. It provides general intelligent assistance to the subscriber as a means to consuming the 3GPP services, i.e., it is responsible for (i) understanding the Intent of the subscriber (i.e., translating the need into technical tasks), (ii) evaluating how the 3GPP network capabilities could provide the desired intelligent assistance to the subscriber (e.g., based on the received Intent, 3GPP network status and capabilities, the available AI Toolbox, and the environment information), (iii) activating and executing selected services and functions from the AI Toolbox, (iv) monitoring performance of activated services (e.g. as described for Category 1 services, the network-side inference results using higher-performance models can be used as a reference to check the local inference accuracy by vehicles using smaller dedicated models) and applying corrections if needed, and (v) providing relevant assistance information to the subscriber, i.e., sending data related to the activated services that are helpful for solving the subscriber’s Intent, and which can be expressed in a format and language suitable for the subscriber.

To perform its tasks, the intelligent assistant should be able to intelligently interpret Intent (in order to understand the Intent and translate it into technical tasks, i.e., AI/ML instructions and/or actions), should have awareness of the physical environment and the network (in order to decide which specific 3GPP services out of many should be activated, and to evaluate if the Intent has been accomplished), and should be able to activate and monitor selected 3GPP services enabled by the AI Toolbox (incl. 3GPP sensing service and algorithms running locally at the subscriber’s side). The intelligent assistant is AI-based, but does not have to be a large language model (LLM). For example, the 3GPP operator can deploy the intelligent assistant by adapting and re-training a general-purpose AI Agent from a 3rd party or open sources. Furthermore, the AI Agent could be re-trained to interpret Intents and provide assistance in more than one application or industry sector.

Main service components at the UE:

Intelligent Assistance Application Entity – pre-configured and provided by the 3GPP network operator. In general, it can be deployed on smart devices such as smart phones, in-vehicle on-board control system, etc. This application client provides the subscriber a service interface with the intelligent assistant (e.g., to relay subscriber’s queries to the intelligent assistant, to visualize the intelligent assistant’s response, to receive data from the 3GPP network, etc.). Furthermore, given the subscriber’s consent, the Intelligent Assistance Application Entity can receive a dedicated fine-tuned AI/ML model from the 3GPP network and use the vehicle’s on-board sensing capability to provide services of Category 1. It can also perform basic monitoring of locally deployed AI/ML models and send analytics data to the 3GPP network for evaluation.

### 6.x.2	Pre-conditions

1. A subscriber UE (e.g., a vehicle) has 6G connectivity. Optionally, the UE is equipped with on-board sensors (e.g., cameras, radars, LIDAR, GPS, etc.), has computing capabilities, and can receive AI/ML models from the 3GPP network for performing local inference based on on-board sensing data.

2. AI Toolbox stored in the 3GPP network contains AI/ML models (incl. AI/ML models trained with 3GPP network data and 3GPP sensing data from the deployed 3GPP network), algorithms, and datasets which are useful to provide the intelligent assistance expected by the subscriber.

3. The 3GPP network maintains real-time awareness of the physical environment and of the network state which is needed to pre-select and execute specific services.

### 6.x.3	Service Flows

1. A subscriber UE registers to the 3GPP network and establishes a connection.

2. The subscriber requests (in the context of navigation) intelligent assistance service and the UE receives the Intelligent Assistance Application Entity from the 3GPP network.

3. A subscriber uses the received Application Entity to input an Intent (e.g. in a natural language) that indicates its on-board sensing capabilities.

Example: a vehicle sends the Intent “I am a vehicle of type X. I carry heavy objects. I would like to safely get to place A.”, and lists its on-board sensor capabilities: a front camera, a GPS localizer, a rear radar.

4. Via the Application Entity, the intelligent assistant at the 3GPP network receives the Intent. Then, the assistant evaluates the Intent and decides which specific services out of all possible specific services offered by the 3GPP network are most relevant and useful to the subscriber in order to provide the required assistance. The intelligent assistant sends back to the subscriber a list of proposed specific intelligent assistance services, e.g., object detection, collision avoidance, parking assistance, emergency trajectory alignment, automated intersection-crossing, infrastructure assisted environmental perception, together with their performance specifications.

Example cont’d: the intelligent assistant responds with an adapted list of proposed specific services for the vehicle: (i) Category 1 specific service: a pre-trained ML model for detecting objects recorded by the vehicle’s front camera, and which is re-trained to be better adapted to local environmental and topological conditions, (ii) Category 2 specific service: obstructed-view assistance at intersections, done by the 3GPP network using 3GPP sensing and computing in 3GPP network (e.g., conduct AI model inferencing to generate obstructed-view assistance results), (iii) Category 3 specific service: safe routing that avoids steep inclines and abrupt turns, which additionally requires the 3GPP network to access external information about traffic conditions in the whole city to solve complex AI tasks.

5. The subscriber may use the Application Entity to select services in the list.

Example cont’d:  the vehicle accepts services (ii) and (iii), but declines the service (i) because it already has a built-in AI/ML model for dangerous object detection with similar performance specifications.

6. Depending on the selected specific service, the 3GPP network provides necessary AI/ML models and data to the Application Entity on the subscriber’s side.

Example cont’d:  the 3GPP network does not provide any pre-trained AI/ML model, because the vehicle declined the service (i).

7. The 3GPP network activates selected services, and the subscriber starts to receive from the 3GPP network the assistance information until the Intent is accomplished or cancelled. The assistance is provided via the subscriber’s Application Entity. The 3GPP network monitors the performance of deployed services in real-time (e.g., to ensure smooth hand-overs, to manage radio and computing resources, etc.).

Example cont’d:  the vehicle (e.g. Driving Application in Figure 6.x.1-1) receives intelligent assistance related to services (ii) and (iii) from the 3GPP network until the vehicle reaches place A.

### 6.x.4	Post-conditions

The subscriber is able to request, interpret, consume, and explore (i.e. view a catalogue of )  the provided 3GPP intelligent assistance service to improve its safety and efficiency.

### 6.x.5	Existing features partly or fully covering the use case functionality

1. 5G Advanced (Rel-18) allows for positioning accuracy of around 10 cm for 90% of the cases given specific (i.e., good) channel conditions. For the proposed use case, reliability of positioning accuracy should be significantly improved, such that centimetre-level positioning accuracy of moving objects is available for more than 90% of the cases, in various channel conditions, and with small delay.

2. TS 22.261 [14] Clauses 6.40.2.1 and 7.10.1 describe the communication service requirements for AI/ML model transfer in 5GS via direct network connection, the KPIs include split AI/ML inference, federated learning, and AI/ML model downloading between UE and AF/AS. The 5G network is regarded as a data pipeline to support 3rd party AI/ML applications.

Therefore, the current standards do not provide network-native real-time intelligent assistance to subscribers (e.g. for autonomous driving) utilizing 3GPP sensing of the environment, though that could be implemented at an application layer as an isolated system. For the proposed use case, not only does it reply on sensing that must meet stringent accuracy and latency requirements, but also on 3GPP network-based/native complex problem solving (using AI) whilst providing the required reliability and latency (e.g. joint latency from AL/ML model inference and communication).

3. 5G and 5G Advanced (Rel-16, Rel-17, and Rel-18) consider some specific services mentioned in this use case (e.g., for Vehicle-to-Anything [4x]). However, current 3GPP standards do not provide the general intelligent assistance service (applicable in different context, not just autonomous driving) that adaptively combines many specific services from 3GPP network, and which is available over a wide area (e.g., with hand-overs), and which meets stringent reliability and latency constraints (e.g. the intelligent assistant timely response to Intent, timely providing obstructed-view assistance results via guaranteed joint latency from inference and communication).

4. V2X specification, such as architecture enhancement work TS 23.287 (SA2), and V2XAPP work in TS 23.286 on application layer support aspects (SA6). Corresponding use cases include tele-operated driving despite being not yet commercialized, automated valet parking, etc.

5. Rel-18, 3GPP TR 28.912 “Study on enhanced intent-driven management services for mobile networks” and 3GPP TS 28.312 “Intent driven Management Service for Mobile Network phase 2” define features of Intent Management Network (IMN) which allows subscribers to express their network-oriented intents (typically human-readable) about, e.g., delivery of a specific communication service or desired properties of the network. TS 28.312 has the following definition:

intent: expectations including requirements, goals and constraints given to a 3GPP system, without specifying how to achieve them.

TS 28.312 clause 4.1.3 (“intent expectations for different types of management needs”) clarifies intent expectation is for “delivering network and service related object (e.g. network, service, slice)” and for “network and service related object (e.g. network, service, slice) performance”. Clause 4.5.1 (“intent expectation”) further clarifies the following:

The object may be a 3GPP managed object like a network slice, subnetwork (e.g. radio network) or other objects like a service.

6. 5G Advanced (Rel-18) defines application awareness (AA) capability, primarily for XR services. The awareness is achieved by exchanging additional information among network elements of the system, such as user equipment (UE), RAN, user plane function (UPF), core network (CN), and application function (AF). 5G AA helps the 3GPP network to better adapt radio resources and QoS (e.g., latency, scheduling, handovers) without compromising the XR service quality experienced by the subscriber. For the proposed use case, the AA capability should be significantly extended, including a capability to activate and adapt specific AI services in order to maintain high service quality.

7. 5G Advanced (R19) TS 22.137 specifies in clause 5.2.1 the stage 1 requirement of 5G wireless sensing service to be provided by 5GS in a target sensing service area, as well as in clause 5.2.3 the stage 1 requirements on network exposure, specifically for 5G network to securely report sensing results (incl. the combined sensing result derived from the joint processing of the 3GPP sensing data and non-3GPP sensing data) to a trusted 3rd party.

### 6.x.6	Potential New Requirements needed to support the use case

NOTE 1: 	The intelligent assistance service depends on the context. For autonomous driving, the provided 6G services (e.g. communication, sensing, AI inference) support the autonomous driving application, e.g. collision avoidance, parking assistance, emergency trajectory alignment, automated intersection-crossing, etc.

[PR 6.x.6-1] The 6G system shall be able to support mechanisms (e.g. AI capabilities/mechanisms like Gen-AI, AI Agents) in the 6G network to provide services to the user/subscribers (including authorized 3rd party application/service providers).

NOTE 1a: 6G network architecture is to be discussed in stage2, AI Agent does not imply any specific solution or architecture.

[PR 6.x.6-2] The 6G network shall support suitable means to support the training of AI/ML models in the Service Hosting Environment, using 6G network data (e.g., configuration data of the deployed 6G network/services, network analytics data) and 3GPP sensing data collected from wide areas, if needed.

NOTE 2: 	this requirement considers among others the availability of some of the above-mentioned 6G network data and 3GPP sensing data collected from wide area, when available in the Service Hosting Environment, or/and in the 6G network, necessary for training the intelligent assistance service-related AI/ML models.

[PR 6.x.6-3] The 6G system shall support mechanisms (e.g. AI capabilities/mechanisms like Gen-AI,  AI Agents) in 6G network to provide suitable combination of multiple 3GPP/6G services (e.g. communication service, sensing service, inferencing, etc. requested by user or authorized 3rd party application via Intent, if needed.

[PR 6.x.6-4] Subject to operator policy, the 6G network shall be able to provide and expose AI inference service to the user/subscriber, and ensure required joint (inference, communication) latency.

[PR 6.1.6-5] The 6G network shall support charging information collection for intelligent assistance services per user/subscriber, considering the usage of network resources related to multiple 3GPP services (e.g. communication service, sensing service, inferencing, etc.).

[PR 6.1.6-6] The 6G network shall support charging information collection for intelligent assistance services per user/subscriber, considering the usage of AI models stored or/and retrained in the operator’s Service Hosting Environment.