3GPP TSG SA WG 1 Meeting #112 Ad Hoc - e	S1-260042r2

12-16 January 2026, electronic meeting	(revision of S1-26xxxx)

Source:	Telefonica (Moderator)

pCR Title:	Consolidation of KPI requirements on ISAC

Draft Spec:	3GPP TR 22.870 1.0.1

Agenda item:	1.4

Document for:	Approval

Contact:	Jesus Martin, jesusmaria dot martingarcia at telefonica dot com

Abstract: Update of the KPI table of Consolidated performance requirements for ISAC

1. Introduction

Based on the approved pCRs from SA1#112, this contribution provides an update of the KPI table of Consolidated performance requirements for Integrated Sensing and Communication. Other two KPI tables of Consolidated performance requirements are included: one for the scenario of Network assisted transportation and another for the scenario of robots collaborating in sensing in smart factories

2. Reason for Change

As requested by the SA1 Chairman for KPIs consolidation, moderators need to update their KPI tables with new input from SA1#112

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to approve the following changes to 3GPP TR 22.870.

r1: Colours and revision marks on latest changes are removed.

ms units removed from some values of velocity resolution

r2: Clean versión with the table skeleton ready for approval + ENs pending for resolution

* * * First Change * * * *

All new text

## 14.2.2	Consolidated performance requirements for Integrated Sensing and Communication

The 6G system shall be able to provide sensing with the following performance requirements:

NOTE: The definitions of the terms used in the following KPI tables are defined in [6].

Table 14.2.2-1: Consolidated performance requirements for Integrated Sensing and Communication.

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing area (sensing target and/or description) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| High topology mapping (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor |
| Low-altitude UAV supervision |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor (UAV trajectory tracking) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor (UAV intrusion detection) |
| Environment object reconstruction |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor (Building (NOTE 2) as in Table 7.6.1-1) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor (Vehicle (NOTE 2) as in Table 7.6.1-1) |
| Road digitalization |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor (crossroad, as in Table 7.7.1-1) (NOTE 5) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor (highway, as in Table 7.7.1-1) (NOTE 6) |
| Ship detection and tracking (NOTE 7) |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor, (Detection of ship on the near shore waters and offshore waters) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor, (Detection of ship on the harbour entrances, harbour approaches, and port. and coastal waters) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor, (Detection of ship on the inland waterway (e.g. river, lake)) |
| Structural health monitoring |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor (e.g. detection of corner reflectors on bridges, buildings in urban scenario) |
| UAV detection, classification and counting |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor |
| Safety Assistance for vulnerable pedestrian |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor (Crossing) |
| Collaborative Robots (NOTE 12) (NOTE 24) |  |  |  |  |  |  |  |  |  |  |  |  | Indoor/outdoor |
| Infrastructure collapse monitoring |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor (e.g. Detecting sudden collapse on infrastructure such as highway, railway, road, flyover, rural areas, farmland) |
| UAV takeoff and landing (NOTE 24) |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor (NOTE 14) |
| Gestures Recognition (NOTE 23) (NOTE 24) |  |  |  |  |  |  |  |  |  |  |  |  | Indoor, Factory Environment, (hand gesture) (NOTE 31) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Indoor, Factory Environment, (head gesture) (NOTE 32) |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Indoor, Factory Environment, (body and limb gesture) |
| Robots collaborating in Sensing. Sensing of Surroundings (NOTE 26) (NOTE 29) |  |  |  |  |  |  |  |  |  |  |  |  | Indoor, Factory Environment |
| Robots collaborating in Sensing. Detection of Pathways, Openings (NOTE 26) (NOTE 29) |  |  |  |  |  |  |  |  |  |  |  |  | Indoor, Factory Environment |
| Enhanced XR navigation |  |  |  |  |  |  |  |  |  |  |  |  | Indoor, Factory Environment, (10 m2) |
| Safe and economic UAV transport |  |  |  |  |  |  |  |  |  |  |  |  | Outdoor (NOTE 18) |

EN: Further classification of use cases in sensing service categories	 is FFS

EN: Velocity resolution for Low-altitude UAV supervision is FFS

EN: Further work to capture sensing density on the table is FFS

EN: KPI values in this table are FFS

* * * End of changes * * * *