3GPP TSG SA WG 1 Meeting #112 Ad Hoc - e 	S1-260053

12-16 January 2026, electronic meeting	(revision of S1-260043r1, S1-254327)

Source:	Nokia (Moderator)

pCR Title:	Consolidation of KPI requirements on Ubiquitous Connectivity

Draft Spec:	3GPP TR 22.870

Agenda item:	1.4

Document for:	Approval

Contact:	feifei.lou@nokia.com

Abstract: this contribution provides an update of the performance requirements consolidation on ubiquitous connectivity, based on the approved pCRs from SA1#112.

Changes in 0053:

- Mark with the colors agreed in Thursday call.

- Change the title of Clause Y.2.x.2 and Table Y.2.x.2-1 to “Consolidated performance requirements for communication”.

- Add one EN to Table Y.2.x.1-1: differentiation between hybrid positioning is FFS.

- Capture the response to some of the comments.

Changes in r1:

- Reverse the changes in the title of Clause Y.2.x.1 and Table Y.2.x.1-1 to include back “satellite-based”. [Airbus]

- Change the title of Clause Y.2.x.2 and Table Y.2.x.2-1 to “Consolidated performance requirements for ubiquitous connectivity”. [Huawei]

1. Introduction

The consolidated performance requirements for clause 8 are categorised into two parts.

The consolidated performance requirements for positioning services covers the following use cases and PRs. The scenarios in the KPI table are sorted from less demanding to more demanding cases.

- [PR 8.5.6-1] from use case on resilient positioning in satellite networks

- [PR 8.7.6-2] from use case on low-energy positioning in satellite networks (updated in SA1#112)

- [PR 8.10.6-1] from use case on hybrid TN and NTN positioning

- [PR 8.11.6-2] from use case on hybrid NTN and GNSS positioning

The consolidated performance requirements for NTN covers the following use cases and PRs. Based on SA1#112 discussion, all of the KPIs in TS 22.261 Table 7.4.2-1 Performance requirements for satellite access are included.

- [PR 8.2.6-1] from use case on ubiquitous and resilient network (EN addressed in SA1#112)

- [PR 8.4.6-2] from use case on service continuity for wearable mobile devices

- [PR 8.6.6-2] from use case on disaster relief

- [PR 8.8.6-2] from use case on global mobile video (updated in SA1#112)

- [PR 8.9.6-2] from use case on low-altitude logistics supported by NTN

- [PR 11.5.6-1] from use case on immersive media services for advanced air mobility (AAM) enabled by 6G NTN (from industry and verticals)

- [PR 11.6.6-1] from use case on high-rate aircraft communication services in 6G (from industry and verticals)

2. Summary of changes

The update is from the following PRs, including the performance requirements for satellite access for UAM and Airplane from industry and verticals.

- [PR 8.7.6-2]

- [PR 8.2.6-1]

- [PR 8.8.6-2]

- [PR 11.5.6-1]

- [PR 11.6.6-1]

3. Proposal

It is proposed to approve the following changes to 3GPP TR 22.870.

* * * First Change * * * *

All new texts

## Y.2.x	Consolidated performance requirements for Ubiquitous Connectivity

### Y.2.x.1	Consolidated performance requirements for satellite-based positioning services

Table Y.2.x.1-1: Consolidated performance requirements for satellite-based positioning services

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Positioning service area/Environment of use | MAX UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |  |
| Containers | [100] m | N/A | [95] % | N/A | Outdoor | [37] km/h | Container mounted | Battery life expectancy (note 1) 12 years |
| Pallets | [100] m | N/A | [95] % | N/A | Outdoor | [100] km/h | Vehicle mounted | Battery life expectancy (note 1) 7 years |
| Wagons | [100] m | N/A | [95] % | N/A | Outdoor | [350] km/h | Train mounted | Battery life expectancy (note 1) 20 years |
| Airplane en-route (note 2) | [50] m | [50] m | [99] % | [1] s | Outdoor | [1500] km/h | Airplane mounted | - |
| Airplane landing (note 2) | [10] m | [10] m | [99] % | [1] s | Outdoor | [350] km/h | Airplane mounted | - |
| UE with partly obstructed sky | [10] m | [3] m | [95] % | [1] s | Outdoor rural and urban | [250] km/h | Handheld, vehicle mounted or IoT | - |
| Agriculture & farming (e.g. livestock, equipment tracking) | [5-10] m [376] |  | [95 %] |  | Outdoor (Rural/remote areas) | [30] km/h | Animal mounted (e.g. collar) or passive asset/equipment mounted (e.g. non-powered harvesting crates, trailers, wheelbarrows) | Battery life expectancy (note 1) 10 years |
| Maritime | [3] m | [3] m | [99] % | [1] s | Outdoor rural | [500] km/h | Maritime mounted | - |
| UAV facility monitoring (note 3) | [3] m | [3] m | [99] % | [0,1-0,5] s | Outdoor | [160] km/h | UAV mounted | - |
| UAV positioning (note 2) | [1] m | [1] m | [99] % | [1] s | Outdoor | [160] km/h | UAV mounted | - |
| Airplane taxiway (note 3, 4) | [1] m | N/A | [99] % | [0,1-0,5] s | Outdoor | [100] km/h | Airplane mounted | - |
| NOTE 1: Battery life expectancy is to be assumed in all coverage conditions and is based on typical message size value and typical frequency. The frequency assumed here is the number of times per day a device is involved in a positioning operation using terrestrial/6G Satellite Positioning, with the required accuracy anywhere within the required coverage area, which could be followed by a communication request from the UE to send this position into a message to a third party application server, unless this information can be obtained from the 6G network itself. In this table, frequency is assumed to be 1h (that is approximately 24 operations per day) and typical message size around 200 bytes. NOTE 2: Positioning services are provided with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS). Multiple satellites can be used to support 3GPP positioning technologies. NOTE 3: Requirements for Airplane taxiway and UAV facility monitoring are in [25]. NOTE 4: Airplane taxiway refers to an airplane on ground during taxi operations. |  |  |  |  |  |  |  |  |

Editor’s Note: maritime mounted UE speed is FFS.

Editor’s Note: differentiation between hybrid positioning is FFS.

### Y.2.x.2	Consolidated performance requirements for communication

Table Y.2.x.2-1: Consolidated performance requirements for communication

| Scenario | Experienced data rate | Area traffic capacity (note 1) | Overall user density | Activity factor | UE speed | UE type | Reliability | Service availability | End-to-end latency | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Disaster relief (note 2) | N/A | N/A | Up to [100] devices/km2 for SMS only | N/A | 3 km/h | Handheld | [99,9-99,999] % | Up to 99,9 % | Up to [600] ms | Peak data rate DL: Up to [20] Mbit/s UL: Up to [2] Mbit/s |
| Ubiquitous and Resilient Network (note 3) | DL: [0,1-25] Mbit/s UL: [2] Mbit/s (note 4) | N/A | [0,1] devices/m2 | N/A | up to [120] km/h | Handheld | [99.9-99.999] % | [98.5] % | [10-100] ms (note 4) [10-500] ms | Coverage [99.9] % |
| Global video service in remote area (note 5) | High video resolution DL: [2-5] Mbit/s UL: [1] Mbit/s Low video resolution DL: [2] Mbit/s UL: [250] kbit/s | DL: Up to [10] kbit/s/km2 (note 6) | [< 0,1] person/km2 | 2% | N/A | Handheld | N/A | N/A | N/A | - |
| Global video service in deep rural area (note 5) | High video resolution DL: [2-10] Mbit/s UL: [2] Mbit/s Low video resolution DL: [2] Mbit/s UL: [250] kbit/s | DL: [1] Mbit/s/km2 (note 7) | [0,1-10] person/km2 | 4% | N/A | Handheld | N/A | N/A | N/A | - |
| Immersive media service via AAM/UAM (direction: from network to user, uncompressed) | DL: [1,3] Gbit/s (note 9) (note 11) | DL: TBD | - | - | Up to 260 km/h (note 14) | UAM mounted | - | - | - | Service interruption time [10] ms for 60 fps, [20] ms for 30 fps (note 8) |
| Immersive media service via AAM/UAM (direction: from network to user, compressed) | DL: [25] Mbit/s (note 10) (note 11) | DL: TBD | - | - | Up to 260 km/h (note 14) | UAM mounted | - | - | - | Service interruption time [10] ms for 60 fps, [20] ms for 30 fps (note 8) |
| Airplanes connectivity (note 12) | DL: 6 Gbit/s/plane UL: 3 Gbit/s/plane | - | - | - | Up to [1500] km/h | Airplane mounted (note 13) | - | - | - | - |
| NOTE 1: Area capacity is averaged over a satellite beam. NOTE 2: Positioning accuracy is low (≈ [100] m). Positioning service availability is up to [99] %. Positioning service latency is [1] s. NOTE 3: Location accuracy is low (≈ [10] m). Positioning availability is [99] %. Positioning latency is [1] s. NOTE 4: When using base station on board HAPS for NTN, end-to-end latency should be up to 10 ms (operating at 20 km altitude) and user experienced data rate should be up to [500] Mb/s for DL and [50] Mb/s for UL to enable eMBB service [374]. NOTE 5: Reduced video quality is acceptable in the remote areas and deep rural area, see chapter 3.9.1.3 and 3.9.1.5 in [31]. NOTE 6: The DL area traffic capacity corresponds to an average user density of 0,1 person/km2. NOTE 7: The DL area traffic capacity corresponds to an average user density of 5 persons/km2 equally shared by two operators. The area traffic is here split between two operators, with [2] Mbit/s/km2 in total. NOTE 8: It is assumed that the interruption time is less than a single frame (16ms for 60 fps, 32 ms for 30 fps) plus a margin of an order of millisecond (e.g. for other processing time). NOTE 9: Uncompressed (0.8 M points per frame, total 300 frames are considered: i.e. 43.2 Mbits/s, 30 fps) NOTE 10: Compressed (Video-based Point Cloud Compression (V-PCC), bpip (bits per input points) total 1.14 (Color 0.9, Geometry 0,11, Occupancy 0.13) are considered: 25 Mbits/s) NOTE 11: It is assumed that the number of passengers using immersive media service per UAM is up to 2. NOTE 12: - Required experienced peak data rate corresponding to the aggregated passenger traffic at aircraft level - Based on an assumption of 450 seats, average take rate of 75% (free model) and load factor of 85% - Assumption of 2:1 Downlink / Uplink ratio, anticipating future usages - The Downlink & Uplink throughput can be achieved using one or multiple satellite links NOTE 13: This airplane mounted UE is operated as a mobile base station relay, providing connectivity to normal UEs used by passengers. These KPIs enable the case of XR UEs for applications in highly dynamic environments, where UEs offload processing to an edge-cloud. NOTE 14: The number is based on [408], [409]. |  |  |  |  |  |  |  |  |  |  |

[CPR Y.2.x.2-1]: The 6G system with satellite access, shall support SMS delivery to UEs with up to [1000] devices/km2 density.

[CPR Y.2.x.2-2] The 6G system with satellite access shall be able to support communication service for UEs (e.g. UAV) at the altitudes from 0 to 3 km via satellite access and/or terrestrial access.

* * * End of changes * * * *