3GPP TSG-SA WG1 Meeting #112 Ad Hoc - e	S1-260064

12-16 January 2026, electronic meeting 	(revision of S1-260054 of S1-260044r2)

Title:	Updated consolidation of KPI requirements on AI section

Agenda Item:

Source:	Moderator (China Unicom)

Contact:	Qun Wei, weiqun5@chinaunicom.cn

Abstract: Prepare and propose a way for consolidation on KPI requirements on AI section.

1. Introduction

Based on the meeting preparations, this document only provides the consolidation way forward and discussion regarding the AI KPIs, based on TR 22.870 and do not include performance requirements with editor’s note.

2. Reason for Change

To provide communication performance requirements contribution and reflect key points regarding the performance requirements of AI sections.

3. Proposal

It is proposed to agree the following changes to new version of 3GPP TR 22.870.

Discussion Part

Besides the basic template consolidation, the following comments need to be considered：

General:

CM1: Table y.1-2 the “joint e2e latency” is probably to be properly defined or decoupled?

CM2: Table y.1-2 Joint E2E latency value needs to be divided into a communication value and a compute value.

CM3: Table y.1-1 primarily consists of communication KPIs. Reflect the table name.

CM4: The latency title in table y.1-2 need to align with “Max allowed end-to-end latency” but also need to highlight the difference from existing KPIs.

CM5: Is the table y.1-2 related with computing service?

CM6: A definition needed for Joint E2E latency.

CM7: “Joint e2e latency” is more from service level of SA1.

UC 6.48 Use case on service robot for power grid

The value and note 3 updated based on approved S1-254342.

* * * First Change * * *

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

……

OP1:

Joint E2E latency: Joint latency of round-trip end-to-end latency, and AI inference latency in Service Hosting Environment, and UE is only considered to contribute to the communication service latency.

New wording:

Max-allowed Joint Latency: this refers to latency of 6G AI service under the operator’s control, including the end-to-end communication latency (from the UE sending inference input to the network), plus the processing latency (e.g. AI inference latency) in the Service Hosting Environment, plus the end-to-end communication latency (from the network sending the inference result back to the UE).

OP2: HW

joint E2E latency: the performance parameter for 6G AI Service when performing AI inference, comprised of round-trip communication latency (e.g. communication latency for UE transferring inference input to AI model in Service Hosting Environment, as well as the communication latency for transferring inference result from the AI model back to the UE), plus the AI inference latency (e.g. TTFT) inside the Service Hosting Environment.

OP3: Nokia

User experienced roundtrip time: Maximum acceptable roundtrip time for the user, which may combine the latencies of multiple 3GPP services under the operator’s control  eg communication , computing, AI, sensing,  in both directions between the UE and the network/SHE. The latency of each service contributing to this roundtrip time can vary [or “is flexible”].

* * * Second Change * * *

Y Consolidated potential KPIs

y.1. Performance requirements for AI service

[CPR y.1-1] The 6G system shall provide various communication and AI services with the following KPIs.

Table y.1-1: Communication performance requirements for burst traffic

| Use Cases | Burst size | Max Allowed end-to-end latency for a burst NOTE B-0 | Service bit rate: user-experienced data rate | transmission latency of a packet | Average packet size | UE speed | Service Area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| . NOTE B-0: Max allowed end to end latency for a burst: max latency for sending out the whole packets within a burst. |  |  |  |  |  |  |  |

Editor’s Note: Average packet size is FFS.

Table y.1-2: Joint Performance requirements for communication and AI services

| Use Cases | Traffic type | video frame size (byte) | Transfer interval | Service bit rate: user-experienced data rate | Max-allowed Joint latency/ Service latency/ Max allowed user experienced RTT |  |  | Reliability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Expected end-to-end latency （UL, or DL） (E2E Communication latency) | Non-Communication latency | Total value of the service latency |  |

Editor’s Note: Joint is FFS.

Editor’s Note: video frame size is FFS.

Editor’s Note: Max-allowed Joint latency/Service latency/Max allowed user experienced RTT/none is FFS

Editor’s Note: Non-Communication latency is FFS.

Editor’s Note: Total value of the service latency is FFS.

Editor’s Note: Expected end-to-end latency (UL, or DL)/E2E Communication latency is FFS.

* * * End of changes * * *