3GPP TSG SA WG 1 Meeting #112 Ad Hoc - e	S1-260066r2

12-16 January 2026, electronic meeting	(revision of S1-260066r1, 260071r2)

Source:	CATT, CEWiT

pCR Title:	Pseudo-CR on Resolve EN of 8.19

Draft Spec:	3GPP TR22.870 v1.0.1

Agenda item:	1.3

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Ankita Lachhwani ankita.lachhwani@cewit.org.in

Abstract: The contribution intends to resolve the editor’s note of clause 8.19 left in the last meeting.

1. Introduction

The use case about traffic offloading with HAPS was introduced in SA1#112 meeting. There is an editor’s note added for the potential requirements. This contribution intends to update the wording of the requirement to resolve it.

2. Reason for Change

HAPS has been introduced as an deployment option of RAN node since 5G, but there still exists some issues to be solved for efficient and workable deployment, such as in the mobility procedures, QoS adaption when the service is switched from terrestrial network to HAPS according to the description.

The PR1 tries to capture this above gap.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR22.870 v1.0.1.

* * * First Change * * * *

8.19	Use case on ensuring essential services using real time traffic offloading with HAPS in a time specific requirement on a geographically challenging area

8.19.1	Description

During mass gatherings or temporary events in geographically challenging or remote areas, existing terrestrial network infrastructure often becomes insufficient to meet the surge in service demand. The deployment of additional or backup infrastructure in such ad-hoc scenarios is typically not cost-efficient due to high logistical and operational complexities. Furthermore, the increased load significantly raises energy consumption, making traditional approaches less sustainable. Following challenges are faced during these situations

Limited Scalability for Sudden Traffic Surges:

•	Terrestrial networks are typically designed based on average or predictable usage patterns.

•	During large gatherings or events, the sudden influx of users can overwhelm the available network capacity, leading to congestion, degraded performance, and service outages.

High Deployment and Operational Costs:

•	Expanding coverage or capacity temporarily (e.g. using portable base stations) involves significant logistical and financial resources.

•	These solutions are often not cost-effective for short-duration or one-time events.

Challenging Terrain and Inaccessible Locations:

•	In mountainous, forested, or remote areas, installing or upgrading terrestrial infrastructure is complex, slow, and expensive.

•	It may require building roads, laying fiber cables, or using helicopters, all of which increase costs and delays.

Energy Constraints:

•	Increased load on the network leads to higher energy consumption, especially in areas without access to reliable power grids.

•	Temporary power solutions (e.g. diesel generators) are not sustainable and contribute to environmental concerns.

Limited Coverage and Service Availability:

•	Even with portable units, the coverage radius is limited, and service quality may be inconsistent, especially with obstructions or interference.

•	Ensuring seamless connectivity across large or irregular terrains becomes technically difficult.

Long Setup Times and Low Flexibility:

•	Traditional deployments require extensive planning, permitting, and physical setup time.

•	They lack the flexibility needed for rapid deployment or dynamic scaling during unpredictable situations.

To address these challenges, this use case proposes the deployment of High Altitude Platform Stations (HAPS) to rapidly establish or restore communication services. HAPS mounted base station can provide supplemental or standalone coverage with minimal ground infrastructure, especially in areas where:

•	Network congestion leads to service degradation

•	Temporary events result in short-term spikes in demand

HAPS can be deployed on-demand, providing temporary or supplemental coverage as needed. HAPS platforms can operate using solar energy, aligning with sustainability requirements. HAPS mounted base station can be utilized to provide at least essential voice and data connectivity to the users in such scenarios

To enable this, it is required to establish wireless backhaul links with core network. It may also be required to control the enablement of HAPS mounted base stations for energy optimization, solving congestion of terrestrial network, etc. In addition, HAPS mounted base station may not be capable of providing all kinds of services as  terrestrial network so that the service adaption of UE(s) may be required  to ensure service continuity.

8.19.2	Pre-conditions

•	A mass gathering event has occurred, overloading terrestrial communications.

•	HAPS platform(s) are available for deployment or are already airborne near the designated area.

•	The base station on the ground and the base station on board a HAPS platform are connected to the same 6G Core Network.

•	Authorized spectrum is available for the usage.

•	A mechanism exists to establish backhaul connectivity for the HAPS (e.g. via satellite link (HTS), a connection to an intact ground gateway outside the zone, or through another nearby HAPS (HTH)).

•	Coordination with relevant authorities (e.g. aviation, emergency management) is established.

8.19.3	Service Flows

1.	Upon notification/authorization, HAPS platform(s) are deployed/re-tasked to the target area.

2.	Backhaul connectivity (HTS, HTG and/or HTH) is established.

3.	HAPS activates communication to the 6G Core Network.

4.	Coordinated by the 6G Core Network, UEs connect to the core network via HAPS mounted base station.

5.

Voice, data, and potentially location services are provided. HAPS should prioritize traffic for emergency services.

8.19.4	Post-conditions

•	Essential communication services are established or restored in the desired area via HAPS.

•	Network status is monitored, and operational logs are maintained.

•	The essential services (voice, data) are provided with acceptable QoS.

•	The emergency services are provided to the user.

•	Thanks to solar power usage and access activation of HAPS mounted base station, the sustainability of the system is maintained.

8.19.5	Existing features partly or fully covering the use case functionality

Table 8.19.5-1: Gap analysis for this use case

| Specifications and clauses | Examples of Existing Requirements | Gap Analysis |
| --- | --- | --- |
| TS 22.261 [14] clause 6.3.2 clause 6.5.2 clause 6.12.2 clause 6.15a.2 clause 6.15a.6 | Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic (e.g. from 3GPP RAT to non-3GPP access technology), taking into account traffic load and traffic type. The 5G system shall be able to support mobility between the supported access networks (e.g. NG-RAN, WLAN, fixed broadband access network, 5G satellite access network). The 5G network can also support multiple wireless backhaul connections (e.g. satellites and/or terrestrial), and efficiently route and/or bundle traffic among them The 5G network shall support autonomous configuration of access and wireless self-backhaul functions. Energy consumption can be monitored and considered through O&M as part of network operations [47], as well as a service criteria. One of the strategies to save energy within mobile networks is to shut down some RAN nodes at times of low usage. | These requirements in 5G system requires to be extended to support HAPS. |

8.19.6	Potential New Requirements needed to support the use case

[PR 8.19.6-1] Subject to regulation, operator(s) policy, the 6G system shall support load balancing between a base station on the ground and a base station on board a HAPS platform connected to the same core network for service continuity taking into account load status, energy consumption, etc.

NOTE 1: The load balancing can be assisted by the core network e.g. mobility suggestion

NOTE 2: The energy efficiency within HAPS mounted RAN base station can be considered through O&M or the core network.

* * * End of Change * * * *