3GPP TSG SA WG 1 Meeting #112 Ad Hoc - e	S1-260071r2

(revision of S1-260071r1)

12-16 January 2026, electronic meeting	(revision of S1-260071)

Source:	CEWiT

pCR Title:	Pseudo-CR on update of 8.19 for EN resolution

Draft Spec:	3GPP TR 22.870 v1.0.1

Agenda item:	1.3

Document for:	Approval

Contact:	ankita.lachhwani@cewit.org.in

Abstract: This contribution is to provide new requirements to resolve EN on potential requirements for use case for ensuring essential services using real time traffic offloading with HAPS in a time specific requirement on a geographically challenging area

* * * First Change * * * *

## 8.19	Use case for ensuring essential services using real time traffic offloading with HAPS in a time specific requirement on a geographically challenging area

### 8.19.1	Description

During mass gatherings or temporary events in a geographically challenging or remote areas, existing terrestrial network infrastructure often becomes insufficient to meet the surge in service demand. The deployment of additional or backup infrastructure in such ad-hoc scenarios is typically not cost-efficient due to high logistical and operational complexities. Furthermore, the increased load significantly raises energy consumption, making traditional approaches less sustainable. Following challenges are faced during these situations

Limited Scalability for Sudden Traffic Surges:

•	Terrestrial networks are typically designed based on average or predictable usage patterns.

•	During large gatherings or events, the sudden influx of users can overwhelm the available network capacity, leading to congestion, degraded performance, and service outages.

High Deployment and Operational Costs:

•	Expanding coverage or capacity temporarily (e.g. using portable base stations) involves significant logistical and financial resources.

•	These solutions are often not cost-effective for short-duration or one-time events.

Challenging Terrain and Inaccessible Locations:

•	In mountainous, forested, or remote areas, installing or upgrading terrestrial infrastructure is complex, slow, and expensive.

•	It may require building roads, laying fiber cables, or using helicopters, all of which increase costs and delays.

Energy Constraints:

•	Increased load on the network leads to higher energy consumption, especially in areas without access to reliable power grids.

•	Temporary power solutions (e.g. diesel generators) are not sustainable and contribute to environmental concerns.

Limited Coverage and Service Availability:

•	Even with portable units, the coverage radius is limited, and service quality may be inconsistent, especially with obstructions or interference.

•	Ensuring seamless connectivity across large or irregular terrains becomes technically difficult.

Long Setup Times and Low Flexibility:

•	Traditional deployments require extensive planning, permitting, and physical setup time.

•	They lack the flexibility needed for rapid deployment or dynamic scaling during unpredictable situations.

To address these challenges, this use case proposes the deployment of High Altitude Platform Stations (HAPS) to rapidly establish or restore communication services. HAPS mounted base station can provide supplemental or standalone coverage with minimal ground infrastructure, especially in areas where:

•	Network congestion leads to service degradation

•	Temporary events result in short-term spikes in demand

. HAPS can be deployed on-demand, providing temporary or supplemental coverage as needed. HAPS platforms can operate using solar energy, aligning with sustainability requirements. HAPS mounted base could be utilized to provide at least essential voice and data connectivity to the users in such scenarios

To enable this, it is required to establish a wireless link for the backhaul with core network. It may also be required to control the enablement of access of base station on board HAPS for energy optimization, which necessitate to update the existing or introduce addition procedures. In addition to this, the network should enhance the mobility procedures based on the location, congestion and link quality. Due to radio condition differences (e.g. signal strength) as seen by UE(s) between Terrestrial Network and HAPS mounted base station there is support required from the network to enable mobility of UE(s) from an overloaded terrestrial network to HAPS mounted base station and to further avoid ping-pong. This support should be provided by the core network as direct base station interface (e.g. Xn) and OAM connectivity may not always be assured in a HAPS setup. Furthermore, HAPS mounted base station may not be capable of providing all services that terrestrial network could support. This requires updation of possible services to UE(s) connected through HAPS mounted based station to ensure service continuity.

### 8.19.2	Pre-conditions

•	A mass gathering event has occurred, overloading terrestrial communications.

•	HAPS platform(s) are available for deployment or are already airborne near the designated area.

•	The base station on the ground and the base station on board a HAPS platform are connected to the same 6G Core Network.

•	Authorized spectrum is available for the usage.

•	A mechanism exists to establish backhaul connectivity for the HAPS (e.g. via satellite link (HTS), a connection to an intact ground gateway outside the zone, or through another nearby HAPS (HTH)).

•	Coordination with relevant authorities (e.g. aviation, emergency management) is established.

### 8.19.3	Service Flows

1.	Upon notification/authorization, HAPS platform(s) are deployed/re-tasked to the target area.

2.	Backhaul connectivity (HTS, HTG and/or HTH) is established.

3.	HAPS activates communication to the Core Network

4.	Based on the Core Network’s suggestion, UE(s) are offloaded from the overloaded Terrestrial Network to HAPS mounted base station.

5.	Further movement of the UE(s) are co-ordinated by the Core Network depending on the traffic load in terrestrial network.

6.	Voice, data, and potentially location services are provided. HAPS should prioritize traffic for emergency services.

### 8.19.4	Post-conditions

•	Essential communication services are established/restored in the desired area via HAPS.

•	Network status is monitored, and operational logs are maintained.

•	The essential services (voice, data) are provided with acceptable QoS.

•	The emergency services are provided to the user.

•	Thanks to solar power usage and dynamic access activation of HAPS mounted base station by the Core Network, the sustainability of the system is maintained.

### 8.19.5	Existing features partly or fully covering the use case functionality

Table 8.19.5-1: Gap analysis for this use case

| Specifications and clauses | Examples of Existing Requirements | Gap Analysis |
| --- | --- | --- |
| TS 22.261 [14] clause 6.3.2 clause 6.5.2 clause 6.12.2 | Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic (e.g. from 3GPP RAT to non-3GPP access technology), taking into account traffic load and traffic type. The 5G system shall be able to support mobility between the supported access networks (e.g. NG-RAN, WLAN, fixed broadband access network, 5G satellite access network). The 5G network can also support multiple wireless backhaul connections (e.g. satellites and/or terrestrial), and efficiently route and/or bundle traffic among them The 5G network shall support autonomous configuration of access and wireless self-backhaul functions | These requirements in 5G system requires to be extended to support HAPS. |

### 8.19.6	Potential New Requirements needed to support the use case

[PR 8.X.6-1]: The 6G network shall support dynamic load sharing among terrestrial base station(s) and base station on board HAPS based on the events (e.g. overloaded situation, resource constraints) for service continuity

[PR 8.X.6-2]: The 6G network shall support energy saving mechanism for HAPS mounted base station during load sharing.

NOTE 1: The dynamic load sharing can be assisted by 6G Core Network e.g. mobility suggestion

NOTE 2: This requirement applies to scenarios where there is a transition between two 3GPP RAN, operated by the same MNO, involving at least one HAPS Mounted RAN