3GPP TSG SA WG 1 Meeting #112 Ad Hoc - e	S1-260080

12-16 January 2026, electronic meeting	(revision of S1-xxxxxx)

Source:	Huawei, HiSilicon

pCR Title:	Pseudo-CR on adding pointer to reference Figure 6.59.3-1 in clause 6.59.3

Draft Spec:	3GPP TR 22.870 v1.0.1

Agenda item:	1.4

Document for:	Approval

Contact:	Shuang ZHANG, zhang.shuang2 AT Huawei DOT com

Abstract: adding pointer to reference Figure 6.59.3-1 in clause 6.59.3.

1. Introduction

Figure 6.59.3-1 in clause 6.59.3 illusrates service flow related to cloud-side AI inference. Rapporteurs point out Figure 6.59.3 is not referenced.

2. Reason for Change

For readability, it is better to add information in clause 6.59.3, to reference the figure.

3. Conclusions

Adding pointer to reference Figure 6.59.3-1 in clause 6.59.3.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v1.0.1.

* * * First Change * * * *

### 6.59.3	Service Flows

As illustrated in Figure 6.59.3-1, the cloud-side AI inference includes the following steps:

Figure 6.59.3-1 illustration of Service flows of cloud-side AI inference

- Bob sends voice command to the robotic dog: "Go to the delivery station and pick up my packets".

- The robotic dog tokenizes the task instruction (including parcel ID, pickup location) and real-time sensor data (e.g., cameras data, LiDAR, GPS) into tokens and sends them to cloud server though 6G network, e.g., via RTC protocol. To ensure the cloud server inference quality, the robotic dog provides additional information to the 6G network in order for it to determine suitable mechanism to transfer the tokens. For example, the robotic dog exposes the token traffic characteristics to the 6G network, e.g. token communication type (agent-to-agent communication), token importance or priority information and other traffic parameters (token generation speed, expected latency, et al).

- The cloud processes the input tokens after having received them from the 6G network, performs task planning, and decomposes the mission into subtasks such as:

  - Navigate to the delivery station

  - Locate the correct packets locker or pickup counter

  - Authenticate identity and retrieve the packets

  - Return to the Bob’s home

- The cloud sends the list of subtasks back to the robotic dog via the 6G network for execution.

- During execution, at key decision points (e.g., arriving at an intersection or reaching the station), the robotic dog captures images or other sensor data and uploads the corresponding tokens to the cloud for verification after exposing the new token characteristics to the 6G network.

- The cloud analyses the uploaded data and confirms whether the robotic dog is at the correct location or needs correction.

- If needed, the cloud sends updated instructions or re-plan the path accordingly.

If the robotic dog enters an area with poor network coverage/connectivity. Then 6G network can utilize the received traffic characteristics of tokens (e.g. token importance or priority information of all the transmitted tokens needed for a particular AI inference process on the cloud side) to conduct differentiated scheduling to guarantee the user experience (e.g. cloud-side AI inference task success rate). In this case, the 6G network allows the robotic dog to continue executing the current subtask successfully as long as the token error rate within a “data unit” for the AI inference process is acceptable.

NOTE: 	“a data unit” here simply describes the ensemble of tokens that are expected to be received by the cloud for performing a particular AI inference task. As described earlier, some of these tokens in the “data unit” are more important than others to the AI inference task success rate.

- The robotic dog executes each subtask sequentially, while continuously uploading the tokens for status updates and sensor data to the cloud for monitoring and potential re-planning.

* * * End of Change * * * *