3GPP TSG-SA WG1 Meeting #112 	S1-254061

17-21 November 2025, Dallas, USA	(revision of S1-244046)

Title:	Use case on digital twin for AI operation

Agenda Item:	8.1.2.2

Source:	ZTE

Contact:	Chen Lin   chen.lin23@zte.com.cn

Abstract: <provide a short description of the content>

This contribution proposes a use case about network digital twin for AI operation, which is created by operator and deployed at network. The digital twin can monitor the physical network entity and help to retrain the AI model used by physical network entity.

************************ New Use Case ***********************************

5.x	 Use case on network digital twin for AI operation

5.x.1	 Description

- 6G targets to provide AI-native wireless network, where AI will be incorporated into the network architecture, radio resource management, PHY layer design, network security and application enhancement from the very beginning of the network design. Various AI techniques (e.g., machine learning, deep learning, reinforcement learning, federated learning) can be leveraged for the design, deployment, management and operation of the 6G devices and network entities. Generally speaking, the AI intelligence in 6G network can be divided into following two categories:

- 1)Intelligent and predictive data analytics: the ability of the 6G network to intelligently process large volumes of data, in order to analyze and predict the context of the users and the wireless network’s environmental states thus enabling data-driven network operational decisions.

- 2) Intelligent/self-organizing network control and optimization: the ability of the 6G system to dynamically learn the wireless environment and intelligently control the network/UE and optimize its resources according to information smartly learned about the wireless environment and users’ states.

- AI enabled intelligent functions include the resource allocation, mobility management, interference management, user experience enhancement, energy efficiency fault detection and self-healing, etc. These AI enabled functions can reside in various network entities, suvh as RAN, NF, OAM, etc. When applying the AI/ML model to 6G network, a pre-trained model based on learning in the simulated environment can be developed. However, due to the discrepancy between simulation and real environment, it is challenging to adapt the AI/ML model to real world network entities.

- Instead of using the pre-trained AI/ML model directly, a digital twin can be used to retrain, validate and monitor the AI/ML model from the virtual environment as shown in Figure 5.x.1-1.

- To be specific, digital twin of network entities can mirror the behavior and environment of its real-world counterpart, allowing real-time monitoring. The simulated network entities in digital twin can be used to verify the AI/ML models before rolling them out to the live network entities, ensuring safety and reliability. For example, the digital twin can simulate rare or extreme events and observe how the AI/ML model responds. If the AI model makes a poor decision, the digital twin helps trace the logic and data leading to the outcome and then fix the issue. If there is unknown situation the AI/ML model could not handle, the digital twin can enter into a retraining mode using the real time environment state of the real world mirrored.

Figure 5.x.1-1 Network digital twin for AI operation

As we can see, the digital twin provides a safe, real-time virtual environment to retrain, validate and monitor the AI/ML model used in the network entities of 6G system. The AI/ML model can quickly adapt to the new environment and the network entities can realize the automated AI operation, maintenance and optimization.

5.x.2	 Pre-conditions

- Operator A provides the 6G communication service for millions of commuters daily. During rush hours, handover failures and dropped calls spike, especially on high-speed trains and in dense urban canyons. To fix this, Operator A develop an AI-driven mobility management model that predicts user movement and proactively optimizes handover timing and target cells. But Operator A does not want to risk deploying a new AI model directly into the live network. Instead, they use a 6G network digital twin, a hyper-realistic, real-time virtual replica of their physical network. The digital twin of network entities

- is created and deployed at operator managed edge server or cloud data center. The digital twin periodically collect the data from the network entities, replicate the physical environment.

5.x.3	 Service Flows

- The new mobility AI/ML model is uploaded into the digital twin, which mirrors thousands of real cell sites, user devices, vehicles, and even environmental factors like weather and signal reflections. For a week, the digital twin mirrors live network traffic, including morning commutes, sporting events, and late-night rides. The AI/ML model’s predictions, such as anticipated user trajectories and handover targets, are logged.

- Network entities log the mobility related data (e.g. handover targets, handover success rates, dropped calls, and user experience scores) and transmit these log data to digital twin. Digital twin compare these data with mobility AI/ML model’s predictions.

- After several days, the digital twin’s analytics detects that the mobility AI/ML model’s performance dips where handover failures increase in city-center cells crowded with runners and spectators during a weekend marathon event.

- Based on the event logs, digital twin retrains the mobility AI/ML model, making it more robust to sudden mass mobility and crowd dynamics. Moreover, the retrained model is tested and validated in the digital twin, with simulated marathons and even rare events like sudden network outages or weather disruptions.

- Once the mobility AI/ML model consistently outperforms the legacy system in the network entities with higher handover success and lower dropped calls, it is ready for live deployment. This model is transferred to the network entities. Now the network entities can perform the mobility decision based on the mobility AI/ML model while the digital twin only performs the real-time monitoring.

- After a while, if sub-optimal performance of the mobility management is detected by the digital twin, the mobility AI/ML model can be retrained and verified at the digital twin again to further improve the performance.

5.x.4	 Post-conditions

Based on the AI/ML model retraining, validation and monitoring by digital twin, the network entities can adapt to varying and unforeseen conditions efficiently. Digital twin enables safe, visible, and rapid evolution of AI-powered mobility management, leading to smoother commutes, happier users, and a smarter city.

5.x.5	Existing features partly or fully covering the use case functionality

NA

5.x.6 Potential New Requirements needed to support the use case

[PR.5.x.6-1]	Subject to operator’s policy, the 6G network shall be able to support the network digital twin to collect the network entity related information to replicate 6G network.

[PR.5.x.6-2]	Subject to operator’s policy, the 6G network shall be able to support mechanism for the network digital twin to retrain and validate the AI/ML model to be used in network entity.

[PR.5.x.6-3]	Subject to operator’s policy, the 6G network shall be able to securely transfer the retrained/validated AI/ML model from network digital twin to network entity.

[PR.5.x.6-4]	Subject to operator’s policy, the 6G network shall be able to support mechanism for the network digital twin to monitor the performance of AI/ML model used in network entity.