3GPP TSG-SA WG1 Meeting #112 	S1-254092

17-21 November 2025, Dallas, USA	(revision of S1-254xxx)

Title:	Proposal on Clause Y.1.3 Resilience

Agenda Item:	8.1.10

Source:	China Telecom

Contact:	Yinglin Chen (chenyl37@chinatelecom.cn)

Abstract: Propose to keep the consolidation of PR 5.6.2.6-1 separate.

Reason for change: PR 5.6.2.6-1 is about fast network provisioning when the PLMN is no available due to urgent event. PR 5.9.5.6-1 and PR 5.9.5.6-2 are about rollout of new services when the PLMN is operational. These requirements do not apply to both scenarios and therefore cannot be merged.

Y.1.3	Resilience

Editor’s Note: This clause contains consolidated potential requirements for 6G that address resiliency. This includes (but not limited to) PRs from clauses 5.6 and 5.9.

Table Y.1.3-1 – Resilience

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.3-1-1 | Subject to regulatory requirements and operator’s policy, 6G network should support suitable means to ensure the stability of services provided by the edge network in case of abnormal transmission occurring between the public network and the edge network. | PR 5.6.1.2-2 | Edge resilience |
| Y.1.3-1-2 | Subject to operator policies and service level agreements, the 6G system shall support on-demand rollout (e.g. within hours) of new or updated services/capabilities - with minimal disruption to existing services, including the ability to efficiently rollback those services/capabilities, as needed (e.g. in case of failures or demand from other services), while minimizing the impact to the user experience | PR 5.9.5.6-1 PR 5.9.5.6-2 | On-demand/quick rollout for new/updated services |
| Y.1.3-1-x | Subject to operator policies and service level agreements, the 6G system shall enable operators to provision network services as part of the operator’s PLMN network on-demand, e.g. in response to an urgent event (e.g. disaster, emergency and DDoS events), with certain level of local control and specific functionalities in a given area during a specific time period. NOTE 1: The level of local control can be based on operator policies and agreements with 3rd party. For example, the authorization and policy control of users to access the provisioned services are not affected by the failure of the operator’s PLMN network. NOTE 2: The enabled functionalities can be based on operator policies and agreements with 3rd party. For example, data connectivity service and voice service are prioritized when an urgent event happens in a residential community; small data transfer service is prioritized when an urgent event happens in an IoT based farmland. NOTE 3: Some situations can target the required network services to be provisioned within hours to serve certain users whose QoE is impacted by an urgent event. NOTE 4: Local control refers to the capability of part of the operator’s PLMN network to operate autonomously and independently, e.g. management of local subscription, local traffic, without interaction with the operator’s PLMN. | PR 5.6.2.6-1 | Fast network provisioning to improve resilience |
| Y.1.3-1-3 | Subject to operator policies and service level agreements, the 6G system shall enable a network operator to authorize a UE, that is subscribed to local network services, to access services from the PLMN of the same operator. NOTE 5: This applies to scenarios where a service is not available in the local network services that have been provisioned on-demand but is available from the PLMN of the same operator. | PR 5.6.2.6-2 | Same operator/cross PLMN service |
| Y.1.3-1-4 | Subject to operator policy and regulatory requirements, the 6G system shall be able to minimize service interruption or degradation during disasters taking into account the affected/not affected parts of a single or multiple network(s) serving an area impacted by a specific type of disaster. NOTE: In case of multiple networks serving the disaster area, it is assumed that under disaster conditions, the remaining available parts of the networks in that area can serve the UEs of other networks. | PR 5.6.4.6-1 |  |
| Y.1.3-1-5 | The 6G network shall provide a mechanism to minimize the signalling arising from recovery of network failure. NOTE: Massive signalling can happen e.g. when a network element fails and registrations or data sessions are re-routed to a backup network element. | PR 5.6.5.6-1 | Recovery |