3GPP TSG SA WG 1 Meeting #112	S1-254097

17-21 November 2025, Dallas, USA	(revision of S1-25xxxx)

Source:	Qualcomm, SKT

pCR Title:	Update of UC 5.7.6 on Unified Access Control (UAC)

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.2.2

Document for:	Approval

Contact:	Francesco Pica

fpica@qti.qualcomm.com

Abstract: This contribution proposes to update the use case on UAC, capturing new descriptive text and one PR on per-UE configurability of access policies.

1. Introduction

As discussed in last SA1 meeting (S1-253096), 5G UAC supports some limited form of per-UE access control policies via the Operator-defined Access Categories, which include few criteria that can be set and provisioned by the network to a UE (stored in the ME), i.e. based on DNN, OS Id + OS App Id and S-NSSAI. 6G should be able to address more flexible scenarios, allowing new configuration and differentiation of per-UE/user access control policies and criteria.

2. Reason for Change

This pCR proposes to update the use case on UAC, to cover aspects related to per-UE configurability of access policies.

3. Proposal

It is proposed to agree the following changes to 3GPP 3GPP TR 22.870 v0.4.1

* * * First Change * * * *

### 5.7.6	Unified Access Control (UAC)

#### 5.7.6.1	Description

Unified Access Control (UAC) mechanism is specified in [14], enables the operators to control the access to 3GPP network in congestion scenarios by determining which access attempt should be allowed or blocked based on operator's policies, deployment scenarios, subscriber profiles, and available services.

6G will introduce new services and capabilities, such as artificial intelligence, sensing, and computing, which require enhancement of the existing mechanism in [14] to efficiently manage evolving technical demands and service requirements. Therefore, the UAC shall be enhanced to address these new services, enabling network operators to control and manage network overload or congestion scenarios.

Furthermore, 6G should be able to address new operator-controlled access control scenarios, e.g. compared to 5G Operator-defined Access Categories, for instance allowing more flexible configuration and differentiation of per-UE/user access control policies and criteria. Some examples of flexible configurations are:

- the network indicates to UE-1 that a certain access attempt should map to category-A, whilst UE-2 should apply category-B, for the same traffic.

- the network can assign different access category mapping (to UE-1 and UE-2) for traffic originated over terrestrial vs satellite access. For example, the network may differentiate SMS access attempts from a specific IOT device or subscriber, when originated over NTN, compared to TN.

- certain UEs or users can be provisioned with dedicated Access Categories to be used, or not, in specific locations (e.g. home, office, stadium, mall), or times (e.g., low/high peak hours).

#### 5.7.6.2	Potential New Requirements

[PR 5.7.6.2-1] The 6G system shall support Unified Access Control requirements as specified in [14].

[PR 5.7.6.2-2] The 6G system shall support suitable access categories to manage access attempts for new services (such as sensing, AI applications, computing), e.g., in congestion scenarios.

[PR 5.7.6.2-3] The 6G system shall enable means for the network to configure access control policies per UE,  including e.g., which Access Identity to use, mapping of traffic type to Access Category, per-RAT access control policies (e.g. TN vs NTN), applicable geographical area or time period.

NOTE 1: Access control policies can be stored in the ME / USIM and can be updated by the network as needed, e.g. based on subscription, UE information, user preference or specific network conditions.

* * * End of Change * * * *