3GPP TSG SA WG 1 Meeting #112 	S1-254104

17-21 November 2025, Dallas, USA	(revision of S1-25xxxx)

Source:	KDDI CORPORATION (Moderator)

pCR Title:	Consolidation of KPI requirements on Further Use Cases on Industry and Verticals

Draft Spec:	3GPP TR 22.870 V0.4.1

Agenda item:	8.1.10

Document for:	Approval

Contact:	you-nakano_atmark_kddi_dot_com

Abstract: This document proposes the consolidated KPI table for Further Use Cases on Industry and Verticals.

1. Introduction

The Moderator proposes the consolidation of the KPI tables which have been agreed in the clause 11 of TR 22.870 V0.4.1.

2. Summary of Changes

- “Characteristic parameter” and “Influence quantity” are removed based on the comment by Ericsson.

- Alignment with functional requirements consolidation is applied.

- KPI tables still under FFS status are removed.

- UAV, UAM and Airplane related KPI tables (i.e., KPIs in 11.2, 11.5 and 11.6) are put under the umbrella of Y.2.6.1 “UAV, UAM and Airplane”.

- Positioning related KPI table and high reliability and low latency KPI table are put into “Other”.

- It is described as “No performance requirement related to this topic.” for Mission critical Consolidated Performance requirements.

- For the reference, the index of sub-clause for each use case is captured in the table respectively as [11.x].

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 V0.4.1.

* * * First Change * * * *

All new texts

Y.2.6	Consolidated Performance requirements for Further Use Cases on Industry and Verticals

Y.2.6.1 UAV, UAM and Airplane Consolidated Performance Requirements

Y.2.6.1.1	Consolidated Performance requirements for Sensing services for UAM

Table Y.2.6.1-1: Consolidated Communication Performance requirements to report Sensing results

| Scenario | Typical transmission interval | Altitude AGL | Typical message Size | End to end Latency | Reliability |
| --- | --- | --- | --- | --- | --- |
| [11.2] Sensing result to a UAM aircraft (note) | [20] ms | up to 1000 m | [25] KB | [20] ms | [99.9] % |
| NOTE: Typically, the message size for one object detected and/or tracked via sensing is 1 kbyte [37]. It is assumed that around 25 objects (25 kbyte) per frame (20 ms) are sensed surrounding an aircraft. The reliability requirement for UAM aircrafts is about 99.9 %. |  |  |  |  |  |

Y.2.6.1.2	Consolidated Performance requirements for Streaming services for UAM

Table Y.2.6.1-2: Consolidated Performance requirements for Streaming services for UAM

| Scenario | Data rate | End to end Latency | Altitude AGL | Reliability (note 4) | Service area |
| --- | --- | --- | --- | --- | --- |
| [11.2] 8K video live broadcast | 100 Mbps Traffic from the UAM aircraft (note 1) | 200 ms (note 1) | up to 1000 m | 95 % | Urban, scenic area |
|  | 600 kbps Traffic towards the UAM aircraft (note 1) | 20 ms (note 1) | up to 1000 m | 95 % |  |
| [11.2] Video streaming | 4 Mbps for 720p video 9 Mbps for 1080p video Traffic from the UAM aircraft (note 1) | 100 ms (note 1) | up to 1000 m | 95 % | Urban, rural area |
|  | 100 Mbps for 8K video Traffic from the UAM aircraft (note 1) | 100 ms (note 1) | up to 1000 m | 95 % |  |
| [11.2] Remote controller through HD video | >=25 Mbps Traffic from the UAM aircraft (note 1) | 100ms (note 1) | up to 1000 m | 99 % | Urban, rural area |
|  | 300 kbps Traffic towards the UAM aircraft (note 1) | 20 ms (note 1) | up to 1000 m | 99 % |  |
| [11.2] Video conferencing or video chat | 25 Mbps Traffic from passengers onboard UAM aircraft (note 1) (note 2) | 100 ms (note 1) | up to 1000 m | 99 % | Urban, rural area |
|  | 25 Mbps Traffic towards passengers onboard UAM aircraft (note 1) (note 2) | 100 ms (note 1) | up to 1000 m | 99 % |  |
| [11.2] Immersive multimedia service (e.g., cloud gaming) | 500 kbps Traffic from passengers onboard UAM aircraft (note 2) | 50 ms (note 3) | up to 1000 m | 99 % | Urban, rural area |
|  | 100~500 Mbps Traffic towards passengers onboard UAM aircraft (note 2) (note 5) | 50 ms (note 3) | up to 1000 m | 99 % |  |
| NOTE 1: These values are aligned with the KPIs for services provided to the UAV applications in TS 22.125 [35], Table 7.1-1. NOTE 2: The value is per passenger; and it is assumed that up to 4 passengers per UAM aircraft use communication services simultaneously. NOTE 3: According to TR 26.928 [50], typically a 50ms latency is a required for cloud gaming use case. NOTE 4: According to [306], the reliability of real-time services is set to 99%, and that of video streaming is set to 95%. NOTE 5: Data rate is calculated assuming typical parameters (e.g. resolution, refresh rate and compression rate). Some codecs may further drop the bitrate requirement. |  |  |  |  |  |

Y.2.6.1.3	Consolidated Performance requirements for satellite access for UAM and Airplane

Table Y.2.6.1-3: Consolidated performance requirements for satellite access for UAM and Airplane

| Scenario | Service interruption time | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) | Area traffic capacity (UL) | Overall user density | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [11.5] Immersive media service via AAM/UAM (direction: from network to user, uncompressed) | [10] ms for 60 fps, [20] ms for 30 fps (note 1) | [1.3] Gbits/s (note 2) | – | TBD | – | [4]/km2 | Up to [300] km/h | UAM mounted |
| [11.5] Immersive media service via AAM/UAM (direction: from network to user, compressed) | [10] ms for 60 fps, [20] ms for 30 fps (note 1) | [25] Mbits/s (note 3) | – | TBD | – | [4]/km2 | Up to [300] km/h | UAM mounted |
| [11.6] Airplanes connectivity (note 5) | – | 6 Gbps/plane | 3 Gbps/plane | – | – | – | Up to 1500 km/h | Airplane mounted (note 6) |
| NOTE 1: It is assumed that the interruption time is less than a single frame (16ms for 60 fps, 32 ms for 30 fps) plus a margin of an order of millisecond (e.g. for other processing time). NOTE 2: Uncompressed (0.8 M points per frame, total 300 frames are considered: i.e. 43.2 Mbits/s, 30 fps) NOTE 3: Compressed (Video-based Point Cloud Compression (V-PCC), bpip (bits per input points) total 1.14 (Color 0.9, Geometry 0,11, Occupancy 0.13) are considered: 25 Mbits/s) NOTE 4: It is assumed that the number of passengers using immersive media service per UAM is up to 2. NOTE 5: - Required experienced peak data rate corresponding to the aggregated passenger traffic at aircraft level - Based on an assumption of 450 seats, average take rate of 75% (free model) and load factor of 85% - Assumption of 2:1 Downlink / Uplink ratio, anticipating future usages - The Downlink & Uplink throughput can be achieved using one or multiple satellite links NOTE 6: This airplane mounted UE is operaed as a mobile base station relay, providing connectivity to normal UEs used by passengers. These KPIs enable the case of XR UEs for applications in highly dynamic environments, where UEs offload processing to an edge-cloud. |  |  |  |  |  |  |  |  |

Y.2.6.1.4	Consolidated Performance requirements for huge data transfer for Airplane

Table Y.2.6.1-4: Consolidated performance requirements for huge data transfer for Airplane

| Scenario | Experienced data rate (DL) (note) | Experienced data rate (UL) (note) | Area traffic capacity (DL) | Area traffic capacity (UL) | Overall user density | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [11.6] Airplanes connectivity in airport vicinity (within 20 km airport radius, up to 1500 m) | 11 Gbps/ plane | 22 Gbps/ plane | TBD | TBD | TBD | Up to 350 km/h | Airplane mounted |
| NOTE: These KPIs are related to massive data transfer including operational data when the aircraft is on the ground or near the ground. The need is to send 10 TByte from aircraft to the ground (aircraft 1 hour in 20 km radius → 22 Gbit/s) [125]. |  |  |  |  |  |  |  |

Y.2.6.2	Digital twin Consolidated Performance requirements

Editor’s Note: The KPI table for Digital twin is FFS.

Y.2.6.3	Network managed Localized Communication Consolidated Performance requirements

Table Y.2.6.3-1: Consolidated performance requirements for Network managed localized communication

| Scenario | Service bit rate: user-experienced data rate | End-to-end latency |
| --- | --- | --- |
| [11.20] Direct device communication in a local network - Factory | [≤ 1.5] Gbps (note 1) | [5] ms (note 1) |
| [11.20] Direct device communication in a local network - Vehicle | [0,1 to [1]] Gbps (note 2) | [5] ms (note 2) |
| NOTE 1: Refer to the KPI requirement in TS22.261 [14] Table 7.10.2-1. NOTE 2: Refer to the performance of Gaming or Interactive Data Exchanging in TS22.261 [14] Table 7.6.1-1 |  |  |

Y.2.6.4	Mission critical Consolidated Performance requirements

No performance requirement related to this topic.

Y.2.6.5	Robotics Consolidated Performance requirements

Table Y.2.6.5-1: Consolidated performance requirements for Robotics services

| Scenario | Max allowed end-to-end latency | Application data rate | Area traffic capacity | Reliability | Message size | Service area | Transfer interval | Connection density | UE speed | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [11.3] Cooperating Mobile Robots – 6G Network Services | [1 to 10] ms | [<10] Mbps [<250] Mbps (note 2) | – | [99.999 to 99.99999] % (note 4) | (note 3) | [20 up to 1,000,000] m² (note 5) | [≤ 10] ms | [≤ 0.5] devices/ m² (note 1) | [< 20] km/h | Robot to Network for services such as digital twin applications, AI/ML compute servers. |
| [11.9] AMRs performing tasks with low acceleration-sensitivity (note 11) | [100] ms (note 6) (note 8) | – | DL: TBD UL: - | [99.9999] % (note 8) | – | >0.4 km (note 9) (note 10) | – | – | Up to [10] m/s (note 7) | UE Type: AMR |
| [11.9] AMRs performing tasks with relatively high acceleration-sensitivity (note 11) | [10] ms (note 6) (note 8) | – | DL: TBD UL: - | [99.9999] % (note 8) | – | >0.6 km (note 9) (note 10) | – | – | Up to [10] m/s (note 7) | UE Type: AMR |
| NOTE 1: Local concentration of mobile robots, for instance, cooperative carrying assuming 8 robots on a space of 4 m x 4 m carrying an object. NOTE 2: The higher data rate applies when e.g. immersive XR, AI/ML traffic, digital twin data, is exchanged. Otherwise, the lower data rate applies. It is expected uplink video will be most demanding. NOTE 3: Any common message size to be expected. Message size depends on type of exchanged data, for instance, sensor data, video streams, immersive XR, AI/ML traffic, digital twin data, etc. NOTE 4: The control of real-time industrial processes requires very high service reliability. Can be lower for non-real-time digital twin or in other contexts (e.g. training, engineering, planning). NOTE 5: Service area depends on scenario and size of location/collaboration area (collaborative carrying mobile robots (11.x.3.1), factory, construction site (11.x.3.5), field (11.x.3.4)) NOTE 6: The latency considered for up to two hops only. The latency is measured in the time interval between the time that collaborative awareness data is ready to be sent and the time that the data is received so that the information can be delivered to a nearby UE to prepare for necessary action to take. NOTE 7: Relative speed between a pair of UEs (AMRs) is considered. NOTE 8: The latency and reliability may vary depending on the type of application and the role of AMR performing different sub-tasks. NOTE 9: It is assumed that AMRs with heavy payloads are operating passively (e.g. at a very low acceleration level) such that the time it takes for them to make a complete stop is long (e.g. more than 20 seconds). NOTE 10: This means the minimum required distance when the UEs need to start communication in order to provide minimum ample time for UEs (AMRs) to prepare. NOTE 11: This includes some scenarios where there are different AMRs using different NPNs. |  |  |  |  |  |  |  |  |  |  |

Y.2.6.6	Other industry and vertical Consolidated Performance requirements

Table Y.2.6.6-1: Consolidated performance requirements for services with high reliability and low latency

| Scenario | Communication service availability: target value | Communication service reliability: mean time between failures | End-to-end latency | Experienced Data rate | Message size | Transfer interval: target value | UE speed | Number of UE | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [11.8] AR guided task (note 1) |  | Up to 99.999 % | 1 – 5 ms (note 2) | DL: 240 – 500 Mbps UL: 50 Mbps |  |  | Pedestrian | [20000] devices/km2 (note 3) | Factory |
| NOTE 1: All the values in this table are targeted values and not strict requirements. The DL data rate is assumed 4K video flow + control information + audio data with data compression ratio of 1:25 to 1:50; and UL data rate is assumed video flow 25 Mbps [24] + high resolution depth sensor data 25 Mbps + audio data. NOTE 2: It is for downlink or uplink one way delay. NOTE 3: Not all UEs are consuming the same 3GPP services at the same time. |  |  |  |  |  |  |  |  |  |

Table Y.2.6.6-2: Consolidated performance requirements for Positioning for Further Use Cases on Industry and Verticals

| Scenario | Positioning Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Number of UE | UE speed | UE type | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |  |
| [11.8] AR remote guided task – outdoor | 1-3 m (note 3) | 3-5 m (note 3) | 99.9 % (note 4) | 15 ms (note 5) | TBD (note 6) | – | – | TBD (note 7) |
| [11.8] AR remote guided task – indoor factory | 0,2-1 m (note 3) | 1-3 m (note 3) | 99.9 % (note 4) | 15 ms (note 5) | ≥200 (note 6) | – | – | 200,000m2 (note 7) |
| [11.3] Human presence detection for mobile Robots | [40] cm (note 2) | – | – | – | – | Pedestrian | – | – |
| [11.3] Cooperating Mobile Robots | [<10] cm [<1] m (note 1) | – | – | – | – | [< 20] km/h | – | – |
| [11.4] Real-Time Digital Twin | [≤ 10] cm | – | – | [1-10] ms (note 12) | [1-10] devices/m2 | Up to vehicular speed | – | [up to 1] km2 (note 11) |
| [11.16] Critical Infrastructure Monitoring – Outdoor | [10] cm (note 8) | [1] m | [99] % | [100] ms (note 9) | – | stationary | Infrastructure mounted | – |
| [11.17] Machine Guidance, Machine Control – Outdoor | [2-10] cm (note 10) | [10] cm | [99] % | [30] s | – | [5] km/h | Machinery mounted | – |
| NOTE 1: Collaborative tasks such as handing over material and objects or robot navigation in close distance to other objects require more accurate positioning. Tasks like robot localization can work with less accurate positioning. NOTE 2: [40 cm] resolution within an area defined by [1 m] range around the robot NOTE 3: The positioning accuracies are based on the UEs on boarded on the AR devices and factory equipment which are used to operate AR remote guided tasks. They are almost same as the values in level 5 in TS 22.261 Table 7.3.2.2-1. NOTE 4: The availability reuses the values in level 4 in TS 22.261 Table 7.3.2.2-1. NOTE 5: It reuses the values in level 4 in TS 22.261 Table 7.3.2.2-1. NOTE 6: The number of positioning UE is the number of UEs which the 6G system is asked to position simultaneously in the positioning service area with the required positioning performance NOTE 7: The 6G positioning service area is based on the typical single industry factory size. NOTE 8: Assuming the UEs are installed at each story of a building with story height of 4m. NOTE 9: Assuming the frequency of an earthquake is 10 Hz. NOTE 10: Assuming the similar performance as RTK-GNSS [231]. NOTE 11: Service coverage both outdoor & indoor (e.g. industrial plant, city area) [31] NOTE 12: Very low latency for the Real-time aspect, both in UL (to report up-to-date data) and DL (to send timely controls) |  |  |  |  |  |  |  |  |

* * * End of changes * * * *