3GPP TSG-SA WG1 Meeting #112 	S1-254190

17-21 November 2025, Dallas, Texas, USA

Title:	Proposed consolidated requirements on IC (Clause Y.1.12)

Agenda Item:	8.1.10

Source:	ZTE Corporation

Contact:	Lijuan Chen (chen.lijuan@zte.com.cn)

Abstract: This contribution is a proposal on consolidation of immersive communication related requirement.  Rapporteurs draft(S1-254020) is uses as baseline.

Summary:

Comment on Y.1.12-1-2; proposal text on Y.1.12-1-15; CPR number correction in table Y.1.12-2; comment on PR 9.16.6-1; adding PR 9.10.6-2 as Y.1.12-2-5.

Y	Consolidated Potential Requirements

Y.1.12	Immersive Communication

Editor’s Note: This includes (but not limited to) PRs from clause 9.

Table Y.1.12-1 – Immersive Communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.12-1-1 | Subject to operator policy, the 6G system shall provide efficient means to inform an authorised third party (residing in the network or in a terminal) the guaranteed throughput (current and/or predicted, within the boundary of the 6G system) of the associated ongoing session. | PR 9.3.6-3 | Third-party Support QoS (throughput) availability |
| Y.1.12-1-2 | Subject to operator policy, the 6G network (e.g. core network) shall support hosting of e.g. an AI/ML model in the Service Hosting Environment based on latency, transport load or data privacy requirements. | PR 9.5.6-3 | Hosting AI/ML model in Service Hosting Environment Comment: Suggestion is going to Y.1.9 computing |
| Y.1.12-1-3 | The 6G system shall support service continuity for mixed reality gaming between indoor and outdoor, and between Dense Urban and Rural deployments. | PR 9.9.6-1 | Service Continuity |
| Y.1.12-1-4 | Subject to operator policy and user consent, the 6G system shall provide means to allow a trusted third-party to provide assistance information in order to identify and deliver the error tolerant data traffic. NOTE 1: The assistance information can be IP-tuple or something in existing header field that is used to mark the data for error tolerant data traffic | PR 9.11.6-1 | Third party Support Assistance info to ID and deliver error tolerant data |
| Y.1.12-1-5 | Subject to operator policy, the 6G system shall be able to transmit the error tolerant data traffic with adaptive QoS which is used for efficient delivery of the error tolerant data traffic, i.e. the reliability and latency can be adjusted automatically without influence from 3rd party application NOTE 2: As an example, the range of reliability can be [99.99 to 50%], and the range of latency can be [10-100 ms] for error tolerant data transmission | PR 9.11.6-2 | QoS Error tolerant data transmission KPIs in Note 2? |
| Y.1.12-1-6 | Subject to operator policy and user consent, the 6G system shall provide a suitable means for communication with error tolerance to evaluate the high data transmission error rate and accept the data packet that fulfil the condition of high data transmission error rate (e.g. allow a percentage of error bits for a successfully delivered packet). | PR 9.11.6-4 | QoS Error tolerant data evaluation |
| Y.1.12-1-7 | Subject to operator policy and privacy considerations, the 6G system shall support a mechanism, including enabling one or more authorized third party(ies) to adapt XR session QoS dynamically at different levels of granularity (e.g. per type of media stream, per data flow, per burst) based on the fluctuations of the 6G connection conditions and QoE policies. | PR 9.12.6-3 | Exposure QoS adaptation QoE |
| Y.1.12-1-8 | Subject to operator policy and user consent, the 6G system shall provide a means to expose to a 3rd party application the status of the communication link specific to a user upon request from 3rd party. NOTE 3: The communication link status can be used by application to better recover the lossy content. | PR 9.11.6-3 | Exposure Communication link status |
| Y.1.12-1-9 | Subject to operator policy and privacy considerations, the 6G system shall provide means (e.g. association of XR content with location and relevant positioning information) to ensure an optimal placement in viewer relative to the real environment (e.g. to ensure safety or optimal interactions between participants, etc.). | PR 9.12.6-1 | Placement of content in real environment |
| Y.1.12-1-10 | Subject to operator policy and privacy considerations, the 6G system shall provide a means to synchronize media/flow and data for rendering in time and in XR space (e.g. coherent gestures, facial expressions the personalized avatars of the group of participants. NOTE: For “XR space” clarifications, see TS 26.119 [304]. | PR 9.12.6-2 | Synchronization of media flow and data for rendering |
| Y.1.12-1-11 | Subject to operator policy and privacy considerations, the 6G system shall provide a means to synchronize heterogeneous data flows from/to a set of UEs (e.g. phone, glasses, tethered ring) associated with a single user and between UEs associated with multiple users in a multimodal communication (e.g. each user having glasses and/or phones). | PR 9.12.6-4 | Synchronization of data flows across UEs |
| Y.1.12-1-12 | Subject to operator policy and privacy considerations, the 6G system shall provide a means to keep heterogeneous media flows synchronized within an acceptable threshold whether rendered on a single or multiple UEs, for each user in a multimodal communication. | PR 9.12.6-5 | Synchronization of media flows on one or more UEs |
| Y.1.12-1-13 | Subject to operator’s policy, the 6G network shall support QoS adjustment based on real time network status to ensure user experience. NOTE: The QoS is adjusted to realize high-priority transmission for key feature information and low-priority transmission for redundant information | PR 9.13.6-1 | Adjustment of QoS based on network status |
| Y.1.12-1-14 | Subject to operator’s policy and user consent, the 6G network shall support prediction of network status based on context information (e.g. user location, mobility path, and environmental knowledge), and may expose such network status to authorized applications | PR 9.13.6-2 | Prediction of network status |
| Y.1.12-1-15 | Based on operator policy and user consent, the 6G system shall provide - means to receive information about the target user service experience and information related to the application from authorized UE or 3rd party applications (such as real time multimedia services) and, - support mechanisms to leverage information related to the application to efficiently deliver communication service and meeting the application’s performance requirements and meet target user service experience. NOTE: Example of information related to the application can include previous, current and/or expected user application characteristics. When provided by the UE, the network can verify the information. | PR 9.14.6-1 PR 9.14.6-2 PR 9.17.6-1 PR 9.17.6-2 | Third Party support User experience |
| Y.1.12-1-16 | Table 9.2.6-1: Key Performance Indicator (KPI) for Immersive Gaming | Clause 9.2.6.1 (no PR #) | KPI |
| Y.1.12-1-17 | Subject to user consent, the 6G system shall support burst type QoS meeting the KPI requirements from Table 9.3.6-1: KPIs for multi-media services with deterministic experience via collaborative processing among UE-network-cloud | PR 9.3.6-1 | KPI |
| Y.1.12-1-18 | The 6G system shall be able to provide deterministic user experience for multi-party call meeting the KPI requirements from Table 9.3.6-2: KPIs for deterministic user experience for multi-party call | PR 9.3.6-4 | KPI |
| Y.1.12-1-19 | 6G system shall be able to support immersive media content production meeting the KPIs from Table 9.6.6-1: Performance requirements for immersive media content production via wireless link | PR 9.6.6-3 | KPI |
| Y.1.12-1-20 | Table 9.7.2-1: KPIs for media synchronization for multiple applications | Clause 9.7.2 (no PR #) | KPI |
| Y.1.12-1-21 | Dense Urban, Urban and Rural indoor/outdoor scenario should meet the KPIs in Table 9.9.6-1: Performance of Mixed Reality Gaming | PR 9.9.6-2 | KPI |
| Y.1.12-1-22 | Use case 9.12.6 (interactive immersive guided tour) needs KPIs from Table 9.12.6-1: KPIs for personalized interactive immersive use-cases | Clause 9.12.6 (no PR #) | KPI |

Table Y.1.12-2 – Immersive Communication related to IMS

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.12-2-1 | Subject to operator policy, the 6G system including IMS shall support the synchronization of independent traffic flows of one or more applications, to be delivered to more than one device (i.e. UE or tethered devices). NOTE: The applications whose traffic flows are synchronized are associated. It is assumed that the association is known to the 6G system. | PR 9.7.2-1 | IMS Synchronization of associated independent media flows Delivery to multiple devices |
| Y.1.12-2-2 | Subject to operator policy and user’s consent, the 6G system (including IMS) shall support intelligent immersive calling service for users via various UEs (e.g. smart wearables, mobile phones, intelligent devices in the home, low-power devices). NOTE 1: Intelligent immersive calling service: An immersive calling service that is empowered by AI capabilities, e.g. generative AI, multi-modal model etc. The service takes various input from different kinds of smart devices and sensors, e.g. camera, smart watches, AR/VR glasses, to enable users to communicate with each other. It can be provided natively by the operators. | PR 9.10.6-1 | IMS Intelligent Immersive Calling Service |
| Y.1.12-2-3 | Subject to operator policy, the 6G system (including IMS) shall provide mechanisms for the intelligent immersive calling service to render media based on the received intent from a user (e.g. voice, gesture) during the calling. NOTE 2: Media rendering could include e.g., switching video input, altering facial expression. | PR 9.10.6-4 | IMS Intelligent Immersive Calling Service Media rendering Intent based |
| Y.1.12-2-4 | The 6G network and IMS shall support communication between UEs with different capabilities, e.g., that requiring multi-channels video and/or multi-channels voice. | PR 9.16.6-1 | Reception of application info from UE/3rd party App Could also go into Y.1.12-1-15 Comment: It cannot go into Y.1.12-1-15. Let’s keep it as a CPR. |
| Y.1.12-2-5 | Subject to operator policy and regulatory requirement, the intelligent immersive calling service shall comply to the existing regulatory requirements (e.g. lawful interception). | PR 9.10.6-2 | intelligent immersive calling service |