3GPP TSG-SA WG1 Meeting #112	S1-254204

# Dallas, USA, 17 – 21 November 2025

Source:	SK Telecom, LG Uplus, Verizon, Huawei, Hisilicon, China Unicom, China Telecom, vivo, NTT DOCOMO

Title:	Discussion on access category for Unattended Data Traffic

Agenda item:	New Work Items

Document for:	Discussion

# 1	Summary from the last meeting

During the TSG-SA WG1 meeting #111, we discussed the CR [1] for TS 22.261 Table 6.22.2.3-1, which proposed a new access category for Unattended Data Traffic. The motivation of introducing an access category, to address the congestion scenarios where the large portion of access attempts is from user-unaware data traffic, was generally agreed. However, the following topics are necessary to discuss further, which this present discussion paper handles.

- Review on whether the definition of ‘Unattended Data Traffic’ can be used

- Gap analysis

- Impact analysis

S1-253047[1] Table 6.22.2.3-1: Access Categories Access Category number; Conditions related to UE; Type of access attempt. 0; All; MO signalling resulting from paging. 1 (NOTE 1); UE is configured for delay tolerant service and subject to access control for Access Category 1, which is judged based on relation of UE’s HPLMN and the selected PLMN.; All except for Emergency, or MO exception data. 2; All; Emergency. 3; All except for the conditions in Access Category 1.; MO signalling on NAS level resulting from other than paging. 4; All except for the conditions in Access Category 1.; MMTEL voice (NOTE 3). 5; All except for the conditions in Access Category 1.; MMTEL video. 6; All except for the conditions in Access Category 1.; SMS. 7; All except for the conditions in Access Category 1.; MO data that do not belong to any other Access Categories (NOTE 4). 8; All except for the conditions in Access Category 1; MO signalling on RRC level resulting from other than paging. 9; All except for the conditions in Access Category 1; MO IMS registration related signalling (NOTE 5). 10 (NOTE 6); All; MO exception data. 11; All except for the conditions in Access Category 1.; MO data consisting only of Unattended Data Traffic. 12-31; Reserved standardized Access Categories. 32-63 (NOTE 2); All; Based on operator classification. NOTE 1: The barring parameter for Access Category 1 is accompanied with information that define whether Access Category applies to UEs within one of the following categories: a) UEs that are configured for delay tolerant service; b) UEs that are configured for delay tolerant service and are neither in their HPLMN nor in a PLMN that is equivalent to it; c) UEs that are configured for delay tolerant service and are neither in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list on the SIM/USIM, nor in their HPLMN nor in a PLMN that is equivalent to their HPLMN. When a UE is configured for EAB, the UE is also configured for delay tolerant service. In case a UE is configured both for EAB and for EAB override, when upper layer indicates to override Access Category 1, then Access Category 1 is not applicable. NOTE 2: When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is neither 0 nor 2, the UE applies the Access Category based on operator classification. When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is 0 or 2, the UE applies the standardized Access Category. NOTE 3: Includes Real-Time Text (RTT). NOTE 4: Includes IMS Messaging. NOTE 5: Includes IMS registration related signalling, e.g. IMS initial registration, re-registration, and subscription refresh. NOTE 6: Applies to access of a NB-IoT-capable UE to a NB-IOT cell connected to 5GC when the UE is authorized to send exception data..

# 2	Discussion

## 2.1	Review on whether the definition of ‘Unattended Data Traffic’ can be used

The existing definition of Unattended Data Traffic in TS 22.101 [2] already provides a clear and comprehensive description of the intended traffic type, referring to data traffic initiated without user awareness, such as transmissions occurring when the screen is locked or after a period of user inactivity. This definition effectively captures the characteristics that its delayed transmission does not degrade user experience.

Therefore, redefining the term appears unnecessary, as the current definition sufficiently serves the purpose of distinguishing user-unaware traffic from user-initiated traffic. Although the GSMA NG.141 [3] defines a similar concept ‘Background traffic category’, the Unattended Data Traffic definition in 3GPP is already well-aligned with that concept and better reflects user-awareness aspect relevant to access control. Therefore, it is reasonable to retain the existing definition rather than introducing a new one.

TS 22.101[2] Unattended Data Traffic: Data traffic of which the user is unaware he/she initiated, e.g. based on the screen/keypad lock being activated, length of time since the UE last received any input from the user, known type of app (e.g. an application monitoring a user's health – "mHealth" – may need its data never treated as Unattended Data Traffic.) Attended Data Traffic: Data traffic of which the user is aware he/she initiated, e.g. based on the screen/keypad lock being deactivated, length of time since the UE last received any input from the user, known type of application (e.g. an application monitoring a user's health – "mHealth" – which may need its data always treated as Attended Data Traffic.)

GSMA NG.141[3] Traffic Category; Description. Background; Any traffic that is not time-sensitive, e.g., firmware/ software updates over the air. This traffic has no critical requirements from latency or data rates perspective. This traffic should/can be subject of scheduling (e.g., at specific time of day) by the applications/networks..

## 2.2	Gap analysis

The following approaches, based on the existing access categories in the UAC framework, can be considered to control the access attempts of Unattended Data Traffic. However, each approach has its inherent limitations, which implies that we have no other choice but introduce a new access category.

- Access Category 1 (=delay-tolerant):

Applicable only when the UE is configured for NAS signalling low priority. Typical UE, e.g. smartphones, do not meet this condition, since the smartphones are not to be configured for NAS signalling low priority. Also, the related configuration leaf does not even exist in NR. Moreover, this approach could interfere with its intended use for the existing IoT devices.

- Operator-defined Access Category via App ID:

Specific applications considered to generate Unattended Data Traffic are mapped to an operator-defined access category. This approach assumes that Unattended Data Traffic is identified on a per-app basis, but Unattended Data Traffic is often context-dependent, not app-specific. This approach should be avoided, as it may lead to an unfair policy for certain Apps.

- Operator-defined Access Category via URSP Routing:

With a specific URSP routing policy on Connection Capability, Background traffic (e.g., Connection Capability identifier "10101000" indicates background traffic) is routed to a dedicated slice and that slice is mapped to an operator-defined access category [5]. This approach requires dedicated slicing per traffic type, which imposes operational complexity of core networks. This approach is contrary to the original intent of UAC, which is to adaptively vary policies based on network conditions.

- Access Category 7 (=MO data):

It can be suggested that configuring the barring parameters for Access Control 7 can partially alleviate the excessive access attempts from UDT. However, this approach blocks the user-initiated data traffic, which is not desirable.

## 2.3	Impact analysis

The impacts derived from the introduction of a new access category for Unattended Data Traffic can be investigated in different aspects, including network-side impact, mobile equipment-side impact and backward compatibility.

- Network-side impact

In UAC, it is the UE who performs a test whether the actual access attempt can be made or not [6]. To be specific, when UE determines the access category applicable for its access attempt, the UE NAS shall check the rules in table 4.5.2.2 [7], and use the access category for which there is a match for barring check. Therefore, there is a minimal impact on network, including the updates of RRC messages to incorporate a new access category.

- Mobile equipment-side impact

From the mobile equipment perspective, the impact can be considered in two layers: the operating system (OS) layer and the UE modem layer. The OS needs to identify whether the data traffic is Attended or Unattended, based on factors such as user interaction status or application characteristics, and provide the corresponding indication to the modem. Since many modern mobile OSs already have similar traffic classification feature based on user interaction status for power saving purpose, it is technically possible. Once this differentiation is made, the modem simply declares the appropriate access category according to the received indication and performs the standard UAC procedure. Therefore, while some additional handling may be required on the OS side for traffic classification, the impact on the modem implementation is expected to be minimal.

- Backward compatibility

Backward compatibility should be ensured by avoiding any conflict with the existing access categories. Considering that the access attempts corresponding to the new access category are a subset of the MO data access attempts under Access Category 7, it is intended that only MO data consisting only of Unattended Data Traffic be categorized into the new access category, as illustrated in Figure 1. When MO data includes both Unattended and Attended Data Traffic, the access attempts shall continue to be categorized as Access Category 7. Therefore, the introduction of the new access category does not affect existing access control behaviour and maintains full backward compatibility.

[Figure 1. The relationship between access category 11 and access category 7]

# 3 Conclusion

- The motivation of introducing an access category, to address the congestion scenarios where the large portion of access attempts is from user-unaware data traffic, was generally agreed.

- Redefining the term appears unnecessary, as the current definition sufficiently serves the purpose of distinguishing user-unaware traffic from user-initiated traffic.

- The candidate approaches for the same purpose, based on the existing access categories, have its inherent limitations, which implies that we have no other choice but introduce a new access category.

- The impacts derived from the introduction of a new access category for Unattended Data Traffic can be investigated in different aspects, including network-side impact, mobile equipment-side impact and backward compatibility.

- Since the existing list of access categories do not cover the access attempts from Unattended Data Traffic, we propose the introduction of a new access category for Unattended Data Traffic.

# References

[1] S1-253047, “Access category for Unattended Data Traffic” SA1#111

[2] 3GPP TS 22.101 V20.0.0: “Service aspects; Service principles”

[3] GSMA PRD NG.141, version 2.0: “Guidelines for URSP”

[4] 3GPP TS 24.526 V19.3.0: “User Equipment (UE) policies for 5G System (5GS) Stage 3”

[5] 3GPP TS 22.261 V20.3.0: “Service requirements for the 5G system”

[6] 3GPP TS 24.501 V19.3.0: “Non-Access-Stratum (NAS) protocol for 5G System (5GS) Stage 3”