3GPP TSG SA WG 1 Meeting #112 	S1-254249

17-21 November 2025, Dallas, Texas, USA	(revision of S1-25xxxx)

Source:	Siemens

pCR Title:	Use Case on Cooperating Mobile Robots – Update

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.8.1

Document for:	Approval

Contact:	Michael Bahr ( bahr et siemens dod com )

Abstract: Update of Use Case on Cooperating Mobile Robots for 3GPP SA1 TR 22.870 Clause 11 Further Use Cases on Industry and Verticals. KPIs with square brackets / FFS have been resolved.

1. Introduction

There have been several KPIs in Tables 11.3.6-1 and 11.3.6-2 with square brackets. These have been resolved.

2. Reason for Change

- Resolution of square brackets around several KPI values in Tables 11.3.6-1 and 11.3.6-2.

- Three broken references (internal to Clause 11.3) have been corrected.

- Several improvements for readability.

3. Conclusions

Square brackets of several KPIs in Tables 11.3.6-1 and 11.3.6-2 have been resolved.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1.

* * * First Change * * * *

### 11.3.6	Potential New Requirements needed to support the use case

[PR 11.3.6-1] The 6G System shall be able to support the following performance requirements to support Cooperating Mobile Robots.

Table 11.3.6-1: Performance Requirements for Cooperating Mobile Robots use case [112]

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |  |  | Remarks |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency (ms) | Application data rate (Mb/s) | Reliability (%) | Message size (byte) | Service area (m²) | Transfer interval (ms) | Connection density [devices/m2] | UE speed (km/h) |  |
| (2) Cooperating Mobile Robots – 6G Network Services | 1 to 10 | <10 <250 (note 2) | 99.999 to 99.99999 (note 4) | (note 3) | from 20 m2 (indoor) up to 1 km2 (outdoor) (note 5) | ≤ 10 | ≤ 0.5 (note 1) | < 20 | Robot to Network for services such as digital twin applications, AI/ML compute servers. |
| NOTE 1: Local concentration of mobile robots, for instance, cooperative carrying assuming 8 robots on a space of 4 m x 4 m carrying an object. NOTE 2: The higher data rate applies when e.g. immersive XR, AI/ML traffic, digital twin data, is exchanged. Otherwise, the lower data rate applies. It is expected uplink video will be most demanding. NOTE 3: Any common message size to be expected. Message size depends on type of exchanged data, for instance, sensor data, control data, video streams, immersive XR, AI/ML traffic, digital twin data, etc. NOTE 4: The control of real time industrial processes requires very high service reliability. Can be lower for non-real time digital twin or in other contexts (e.g. training, engineering, planning). NOTE 5: Service area depends on scenario and size of location/collaboration area (collaborative carrying mobile robots (11.3.3.1), factory, construction site (11.3.3.2), field) |  |  |  |  |  |  |  |  |  |  |

[PR 11.3.6-2] The 6G System shall be able to support the following positioning performance requirements to support Cooperating Mobile Robots.

Table 11.3.6-2: Positioning Performance Requirements for Cooperating Mobile Robots use case [112]

| Use Cases | Positioning accuracy (m) | Velocity (km/h) | Remarks |
| --- | --- | --- | --- |
| Cooperating Mobile Robots | <0.1 | slow (note) | Collaborative tasks such as handing over material and objects or robot navigation in close distance to other objects require more accurate positioning. |
| Cooperating Mobile Robots – Robot localization | <1 | < 20 | Tasks like robot localization can work with less accurate positioning. |
| Human presence detection | 0.4 | Pedestrian | 40 cm resolution within an area defined by 1 m range around the robot |
| NOTE: Velocity of robots is usually slow and adapted to the localization performance in such situations. |  |  |  |