3GPP TSG-SA WG1 Meeting #112 	S1-254394

17-21 November 2025, Dallas, USA	(revision of S1-244221,254042,4089,4391)

Title:	Use case on sensing result confidence monitoring

Agenda Item:	8.1.4.2

Source:	ZTE Corporation

Contact:	xu.ling@zte.com.cn

Abstract:  This contribution proposes to resubmit a use case about sensing result monitoring and validation.

***********************************************************************************************

First change begins

Followings are all new text

**********************************************************************************************

7.x	Use case on sensing result monitoring and validation

7.x.1	Description

In V2X scenarios, vehicles communicate with other vehicles, infrastructure, and pedestrians to facilitate intelligent traffic management and Advanced Driver Assistance Systems (ADAS). Ideally, the various types of sensing data are collected and fused for required sensing result with certain confidence level. The confidence level of the sensing result is very important to impact how to use the sensing result. In fact, the vehicle OEM always expects to achieve sensing results of minimal difference with actual sensing target characteristics. However, due to variations in hardware and sensing algorithms performance among sensing transmitter and sensing receiver (e.g. network node, devices or vehicles), the sensing result accuracy the 3GPP system provided can’t be alwats stable during sensing operation period. Therefore, monitoring and validate the sensing results are valuable for network operator to guarantee or improve its sensing service.

Validation the sensing result is operated via comparing sensing results with actual sensing target characteristics information, over a period of time. The actual sensing target characteristics information could be obtained from the authoritative party, e.g. traffic management department to support the validation. In some cases, while it can’t get the actual sensing target characteristics information, the network could collect the non-3GPP sensing data from other independent sensors e.g. ladar, camera.

7.x.2	Pre-conditions

The 6G network belongs to Operator O. It can provide sensing service. It could obtain related assistance information e.g. sensing target characteristics information from the traffic management department based on the Operator O and the traffic management department agreement.

Vehicle A has subscribed the sensing service provided by the 6G system of Operator O.

7.x.3	Service Flows

- Vehicle A is driving in the city.

- The 6G network performs sensing operation in the intersection area where the Vihecle A is driving, and achieve sensing result SR-N1 (e.g. the number of vehicle, direction, velocity of moving vehicles in this area) for this area.

- The 6G network accesses the traffic management platform to get the actual vehicles moving information, e.g. moving direction, moving velocity etc. in the sensing area.

- The 6G network continues performing sensing operation to collect sensing results and get the actual vehicles moving direction, velocity information in target sensing area.

- When the vehicle A stops, the 6G network compares the sensing results with the actual vehicles information,e.g. moving direction , velocity information etc. And reports the comparison results to network operator O.

7.x.4	Post-conditions

The operator O can monitor and validate the sensing results from sensing service offering.  .

7.x.5	Existing features partly or fully covering the use case functionality

In TS22.137[6] clause 5.2.3, the following requirements have been specified:

Subject to operator’s policy, the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met.

NOTE: These conditions could be e.g., the target object distance from the restricted area border or entering restricted area.

Subject to operator’s policy, the 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing service based on specific parameters (e.g., refresh rate, period of time, sensing KPIs, geographical location) and to receive the corresponding sensing results.

Subject to operator’s policy and regulation, the 5G system shall be able to provide secure means for a trusted third-party to receive sensing results with contextual information.

Subject to user’s consent, regulation and operator’s policy, the 5G network may provide secure means to expose to a trusted third-party the combined sensing result derived from the joint processing of the 3GPP sensing data and non-3GPP sensing data.

Subject to operator’s policy, the 5G network may provide secure means for the operator to expose information towards trusted third-party on whether a given sensing service is available and the estimated quality of the given service for a certain geographic area and time.

Subject to operator’s policy, the 5G network may enable secure means for a trusted third party to provide sensing assistance information.

But it could be observed that the sensing results monitoring and validation don’t be covered. Considering the sensing result monitoring and validation could help to guarantee sensing service quality, it is suggested to add new requirements to support monitoring and validating the sensing results provided by the 6G system.

7.x.6	Potential New Requirements needed to support the use case

[PR.7.x.6-1] Subject to operators’ policies, regulations, when offering sensing service, if the assistance information(e.g.the actual sensing target characteristics) from a trusted 3rd party is available, the 6G network shall provide means for a mobile network operator to monitor and validate the sensing result (e.g. by comparing the sensing results with the actual sensing target characteristics etc.).