3GPP TSG SA WG 1 Meeting #112 	S1-254125r1

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254125)

Source:	Verizon, AT&T, Vodafone, Sony, Interdigital, Ericsson, Mediatek, EDF, Itron

pCR Title:	Pseudo-CR on updating utility use case in clause 11.25

Draft Spec:	3GPP TR 22.870-V0.4.1

Agenda item:	8.1.x

Document for:	Approval

Contact:	samita.chakrabarti@verizon.com

Abstract: Updating the KPI table for TR 22.870 clause 11.25 on monitoring Utility Transmission Grid assets

1. Introduction

The utility usecase described in clause 11.25 has a FFS requirement. This proposal is updating the table with the current findings.

2. Reason for Change

Updating data in table 11.25.6-1

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870-V0.4.1

* * * First Change * * * *

### 11.25.5	Existing features partly or fully covering the use case functionality

3GPP Massive IoT communication via sensor UEs with NB-IoT or LTE-M RAT on 4G mostly.

For 5G system:

In TS 22.261 [14] clause 6.4 Resource efficiency states some general requirements for IoT devices.

In TS 22.261 [14] clause 8 Security, security requirement for IoT devices is listed.

In TS 22.261 [14] clause 6.28.2.2 describes Smart Grid requirements

TS 22.104 [64] Appendix A.4 also contains a number of Utility grid use cases.

However, the transmission line monitoring is a new area of measurement which can be possible with a variety of new generation of 6G IoT devices.

### 11.25.6	Potential New Requirements needed to support the use case

[PR 11.25.6-1] The 6G System shall ensure that IoT communication services shall minimize impact on other services (e.g. regular voice, video and data).

[PR 11.25.6-2] Subject to the Operator’s policies and control, the 6G system shall provide means to support extended coverage.

[PR 11.25.6-3] The 6G System shall support diverse device types with long lifetime (e.g. 10 to 20 years).

NOTE: 	3GPP should consider support of device lifetimes longer than the lifetime of the supporting 6G core network.

[PR 11.25.6-4] Subject to the Utility Operator’s and MNO policies, the 6G system shall support the 6G IoT communication services with the following KPI values:

Table 11.25.6-1: KPI for monitoring Utility transmission grid assets

| Profile | Characteristic parameters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Data packet size (Kbytes) | Monitoring frequency | End-to-end latency: maximum (ms) | Devices per km of transmission line | Reliability of the connection: target value | Communication service availability: target value | Battery lifetime (years) |
| Monitoring Utility transmission grid assets | [ 2 – 2000] NOTE1 | [500ms to 15min or more] NOTE2 | [<5 to 100+ ] NOTE3 | [5-15] | [99.999] | [99.999] | 10-20 |
|  | NOTE1: Approximately 200–500 bytes for normal telemetry; 2–10 kB for vibration summaries; about 300 kB for arcing/PD bursts; 150–300 kB for photos and 0.3– 2 MB for short videos. NOTE2: Monitoring frequency varies depending on what is monitored and during which period. Environmental monitoring is less frequent than voltage monitoring NOTE3: Environmental monitoring latency could be delay tolerant (>100ms). But the transmission line monitoring is very latency sensitive and may require response in the order of 5ms or less |  |  |  |  |  |  |